Program for calculating properties of built sections on HP 28S calculator

>> PROF <<

Ulrik Dan Christensen, 1997

Purpose of the program:

The program is able of calculating the following properties for built-up sections: Moments of inertia, modulus (for base and face), area, position of neutral axis, total height. Furthermore it's possible to calculate properties for a known profile together with a given plating.

To whom could the program be useful?

Engineers especially naval architects who need to determine properties of steel members in large constructions. Typical minimum required modulus and area are known, and the engineer need to choose a suitable profile or built-up section which fulfil these requirements.

How to use the program:

1, Start the program named "BEGIN". Intro screen is shown.

2, Hit any key (except "ON") to proceed.

3, The "choose element type screen" is now shown, those between the 3 types of elements by pressing "1","2" or "3". Press "H" to get help concerning element types.

Element type no. 1, Input: width and height of the element (vertical centre of gravity is automatically set to half the element height).

Element type no. 2, Input: width, height and vertical centre of gravity

Element type no. 3, Input: Profile height, profile area, moment of inertia for the profile, vertical centre of gravity for the profile.

4, Depending of which element type you choose, you will be prompted to enter more or less data for the element in question.

5,  After you have completed the data input for the first element, the following results are shown:

*	Area

*	Moment of inertia

*	Vertical centre of gravity (VCG)

*	Total area

*	Position of neutral axis (distance measured from the base line)

*	Total height

*	Moment of inertia about the base line

*	Modulus for base

*	Modulus for face

*	Total moment of inertia

(for element type no. 2 VCG is not calculated, for element type no. 3 area, moment of inertia and VCG are not calculated)

6, When the last calculated results have been shown, the user will be asked whether an additional element is needed. If the answer is affirmative, the program returns to the "choose element type screen", this way the user is able to enter multiple elements until the build-up section in question have been entered. The program stops when the user states that no more elements are needed.

All results are stored in variables which can be found in the TPRO catalog. In order to purge all variables created by the PROF program, start the "CLEA" program.

PATH necessary for the programs below:  { HOME PRG TPRO}

BEGIN

<< 1 'COUNT' STO TPRO

PROF CHO

>>

PROF

<< 0 'TY' STO

DO

"Choose element type

press 1,2 or 3"

1 DISP HOME UTIL

"(press H for help)"

5 LDISP HOME PRG

TPRO

DO

UNTIL KEY

END

IF DUP "1" SAME

THEN 1 'TY' STO

DROP

ELSE

IF DUP "2"

SAME

THEN 2 'TY'

STO DROP

ELSE

IF DUP "3"

SAME

THEN 3 'TY'

STO DROP

ELSE

IF "H"

SAME

THEN PHELP

ELSE

"WRONG INPUT" 1 DISP

1 WAIT

END

END

END

END

UNTIL 0 TY <>

END CLLCD PWID

PHEI

IF TY 3 <

THEN DUP2 DUP2

SWAP STR-> "WID"

SAVAR

END STR-> "HEI"

SAVAR PAREA IMOM

ANYK CLLCD CCG

"AREA" VADD 2 TDISP

NAX EVAL "N.A." 3

NDISP "HEI" VADD 4

TDISP ANYK CLLCD IBL

EVAL DROP "IBL" VADD

1 TDISP ZB "Zbase" 2

NDISP ZT "Zface" 3

NDISP IT "Imom" 4

NDISP ANYK

>>

PHEI

<<

IF TY 2 >

THEN

"Height Prof. "

'COUNT' RCL ->STR + 2

I

ELSE "Height "

'COUNT' RCL ->STR + 2

I

END

>>

PAREA

<<

IF  TY 3 <

THEN STR-> SWAP

STR-> * DUP "AREA" 3

VDISP

ELSE "AREA Prof. "

'COUNT' RCL ->STR + 3

I STR->

END "AREA" SAVAR

>>

PWID

<<

IF TY 3 <

THEN "Width "

'COUNT' RCL ->STR + 1

I

END

>>

PHELP

<< HOME UTIL CLLCD

"Type 1 : width and height known

Type 2 : width, height and

CG known

Type 3 : profile data known"

1 LDISP PRESS HOME

PRG TPRO CLLCD

>>

NAX

<< CL(SIGMA) 1 COUNT

FOR X "AREA" X

->STR + STR-> "CG" X

->STR + STR-> * (SIGMA)+ 1

STEP TOT '(SIGMA)DAT'

PURGE 'AREAT' RCL /

>>

NDISP

<< ROT ROT SWAP ->STR

SWAP ": " + SWAP +

SWAP DISP

>>

CLEA

<< 1 'G' STO { IBLT

AREAT HEIT } PURGE

WHILE G COUNT <=

REPEAT

IFERR "'" "IBL"

+ G ->STR + STR->

PURGE "'" "CG" + G

->STR + STR-> PURGE

"'" "IMO" + G ->STR +

STR-> PURGE "'"

"AREA" + G ->STR +

STR-> PURGE "'" "WID"

+ G ->STR + STR->

PURGE "'" "HEI" + G

->STR + STR-> PURGE

THEN KILL

END 1 'G' STO+

END 'G' PURGE

>>

CHO

<<

WHILE COUNT 0 <>

REPEAT CLLCD

"ANOTHER ELEMENT?" 1

DISP "(Y / N)" 2

DISP

IF

<<

DO

UNTIL KEY

END "Y" SAME

>> EVAL NOT

THEN KILL

ELSE CLLCD 1

'COUNT' STO+ PROF

END

END

>>

ZT

<< NAX HEIT SWAP - IT

SWAP /

>>

ZB

<< IT NAX /

>>

IT

<< NAX 2 ^ AREAT *

IBLT SWAP -

>>

IBL

<< "CG" COUNT ->STR +

STR-> 2 ^ "AREA"

COUNT ->STR + STR-> *

"IMO" COUNT ->STR +

STR-> + DUP "IBL"

SAVAR

>>

CCG

<<

IF TY 1 SAME

THEN "HEI" COUNT

->STR + STR-> 2 / 1

COUNT

FOR X "HEI" X

->STR + STR-> + 1

STEP "HEI" COUNT

->STR + STR-> - "CG"

SAVAR "CG" COUNT

->STR + STR-> "CG" 1

VDISP

ELSE

IF TY 2 SAME

THEN "CG "

'COUNT' RCL ->STR + 1

I STR-> "CG" SAVAR

ELSE "CG Prof. "

'COUNT' RCL ->STR + 1

I STR-> "CG" SAVAR

END

END

>>

TDISP

<< ROT ROT SWAP ->STR

SWAP " (SIGMA)" + ": " +

SWAP + SWAP DISP

>>

VADD

<< DUP CL(SIGMA) -> obj

<< 1 COUNT

FOR X obj "'"

SWAP + X ->STR + STR->

RCL (SIGMA)+ 1

STEP

>> TOT '(SIGMA)DAT' PURGE

SWAP DUP2 "'" SWAP +

"T" + STR-> STO

>>

ANYK

<< HOME UTIL PRESS

HOME PRG TPRO

>>

SAVAR

<< "'" SWAP + COUNT

->STR + STR-> 'NA' STO

'VA' STO VA 'NA' RCL

STO 'NA' PURGE 'VA'

PURGE

>>

VDISP

<< ROT ROT SWAP ->STR

SWAP COUNT ->STR +

": " + SWAP + SWAP

DISP

>>

IMOM

<<

IF TY 2 >

THEN "IMOM Prof. "

'COUNT' RCL ->STR + 4

I STR->

ELSE STR-> SWAP

STR-> SWAP 3 ^ * 12

INV * DUP "INERTIA" 4

VDISP

END "IMO" SAVAR

>>

I

<< HOME CODE IN HOME

PRG TPRO

>>

TPRO

<< HOME UTIL CLLCD

"PROPERTIES OF BUILT SECTIONS

BEGIN WITH ELEMENT

NEAR THE BASE LINE"

1 LDISP PRESS HOME

PRG TPRO CLLCD

>>

Main variables used in these programs

TY: Element type [value 1,2 or 3]

COUNT: Number of elements [value from 1 to n]

WID1..WIDN: WIDN contains width of element no. "N"

HEI1..HEIN: HEIN contains height of element no. "N"

AREA1..AREAN: AREAN contains area of element no. "N"

IMO1..IMON: IMON contains moment of inertia for element no. "N"

CG1..CGN: CGN contains centre of gravity for element no. "N"

IBL1..IBLN: IBLN contains moment of inertia about the base line for element no. "N"

AREAT: Total area of all elements

HEIT: Total height of all elements

IBLT: Total moment of inertia about the base line (used to calculate moment of inertia about the neutral axis)

PATH necessary for the programs below:  { HOME CODE }

IN

<< -> s p

<< "" s "" <> s ": "

+ s IFTE 's' STO

WHILE

IFERR

DO s OVER +

"_" + p ->IN s OVER +

p ->IN

UNTIL 0

END

THEN

END DUP

"ENTER" <>

REPEAT DUP "INS"

==

<< DROP LCD->

'lcd' STO HALT ""

lcd ->LCD 'lcd' PURGE

>> IFT DUP

"BACK" ==

<< DROP 1 OVER

SIZE 1 - SUB ""

>> IFT +

END DROP DUP s

SWAP + p DISP

>>

>>

->IN

<< DISP 1 40

START KEY '->IN'

IFT

NEXT

>>

PATH necessary for the programs below:  { HOME UTIL }

PRESS

<< # 1F8A7h SYSEVAL

DO

UNTIL KEY

END DROP # 1F8B4h SYSEVAL

>>

LDISP SYSTEM OBJECT (coded by: Mathieu Clabaut)

76C20 58A81 4C3C0 B35C0 69C20 F2200 8F72F 40340 48FF1 0A8F1 8050D 23018

B640D A3058 BA40D A1011 47137 14334 E4A20 8A201 8F8B0 508DB 05C08 08F85

21B00 FFF30 F15C0 11A06 17414 3818F 8481C CC560 6A711 00174 D0841 14B07

13706 31A09 66606 56131 03966 411BB 36D1D 2302D 76370 31169 E2333 102B6

A31A5 9EA32 31369 E2313 1F69E 6A0CC CC6A0 08082 1E631 029EE 90808 21E6B

6AC4C 4D6C4 CA348 75D1C A130D 215E0 CED71 60AF2 14EAF 5BED1 11CC8 72708

08818 1E81D CC432 81281 281E8 1E811 81181 D81DC C4606 2EF28 1531B 9E0E1

20E1D 15512 01773 482EF F1331 318B2 60665 03406 AFF8B 21134 00CFF 8B650

13587 1C116 1CF46 0606F 851AF 231EF 685F1 10CC4 31100 07137 06171 649E8

08007 8D1E5 21119 E6D51 09305 9813E 1F048 FF842 6DBF6 01200 9F20

Size  594

CRC  = #DDDCh

Type the hex-string without any blanks or new-line ("76C2058A814C...") verify the size and the CRC, and change it into a System-object with the program RASS.

RASS-generator

<<

# C02C9604F3D02C67h

# 3134105081F80005h

# 8183414719611310h

# 71B414718D30458Fh

# 8AE90A13AE30131h

# CDC0610851686818h

# 3CEAA125E1D8D95Dh

# 2F9004F3Dh 1 7

START # 3B82h

SYSEVAL

NEXT # 20238h

SYSEVAL # 4F3Dh

SYSEVAL

>>