-----------------------------------------------------------

# User’s Notes for the PLDL 100 Aplet

Version 1.1, 12/10/96

----------------------------------------------------------------------------

Notice

This material is transferred free of charge, as is. Hewlett-Packard makes no warranty, expressed or implied as to its performance. Hewlett-Packard specifically disclaims the implied warranties of merchantability and fitness for any particular purpose. Hewlett-Packard shall not be liable for any direct, indirect, special, incidental or consequential damages, whether based on contract, tort, or any other legal theory. Hewlett-Packard specifically will not offer any support concerning this material and grants no exclusivity for its use. Hewlett-Packard specifically does not offer this material as a product and does not commit to make this material available in the future as such.

Copyright Hewlett-Packard Company 1996. All rights reserved.

----------------------------------------------------------------------------

Getting Started

The PLDL 100 aplet enables the HP 38G to control the PLDL 100 Data Logger.  This aplet is an extension of the built-in Statistics aplet, with views added to watch real time measurements and to specify unattended data logging.

Like the Statistics aplet or the Function aplet, the PLDL 100 aplet is a base for constructing more specific aplets. If you set up an experiment with the PLDL 100 aplet, adding notes and sketches, and then save the aplet under a descriptive name, you can create an experiment aplet for others to use.

On the other hand, if you receive experiment aplets made by others, you don’t need the general PLDL 100 aplet. Just follow the directions for those experiment aplets.

These notes describe basic use of the PLDL 100 aplet. They assume you have some experience with the HP 38G, and that you have read the `HP 38G User’s Guide’ and the `PLDL 100 User’s Manual’.

# PLDL 100 Views

The PLDL 100 aplet has all the views of the built-in Statistics aplet, along with the following views:

- Numeric Setup (Press [SHIFT] [NUM])

Sets up parameters for running an experiment and specifies the probes in use.

- Monitor Data (Press [SHIFT] [VIEWS] `Monitor Data’)

Displays readings from the PLDL 100 in either numeric or graphical form.

- Log Data (Press [SHIFT] [VIEWS] `Log Data’)

Provides menu keys to start, check, and stop experiments, and to retrieve data from the PLDL 100.

- Enhanced Numeric View (Press [NUM])

An enhanced version of the Statistics Numeric view that provides additional data management and analysis features.

The other PLDL 100 aplet views are identical to views in the Statistics aplet.

# Downloading a PLDL 100 Aplet

To load the PLDL 100 aplet from a disk into the HP 38G:

1.	Put the disk in a Drive95 portable disk drive (or in a computer equipped with an HP Connectivity Kit).

2.	On the HP 38G, press [LIB] {RECV} and select `Disk drive...’ `PLDL 100’ to download the aplet.

3.	When downloading is completed, select `PLDL 100’ from the LIB catalog to start the aplet.

To update the base PLDL 100 aplet on your HP 38G:

1.	Send to disk any experiment aplets you want to keep.

2.	Delete the old PLDL 100 aplet and all experiment aplets.

3.	Load from disk the new PLDL 100 aplet.

4.	Load from disk the experiment aplets. (They will use the updated base aplet.)

# Setting Up a Probe

In many cases you can set up a probe using one of the standard calibrations listed below. To set up a probe:

1.	Press [SHIFT] [NUM] to start the Numeric Setup View.

2.	Press {PAGE \/} to go to Probe Setup.

3.	Highlight the desired port.

4.	Press {CHOOSE} and highlight the appropriate standard calibration.

5.	Press [ENTER] to select the calibration. The port is automatically checked.

Standard calibrations are available for the following probes:

Vernier Probe Name           Description      Slope    Intercept Part No.

--------------------------------------------- -------- -------------------

Current & Voltage Probe      Voltage          1        0         CV-DIN

Standard Temperature Probe   Temperature ©  58.341   -53.073   TPA-DIN

Temperature (F)  105.01   -63.531

Direct-Connect Temperature   Temperature ©  55.55    -17.78    DCT-DIN

Probe

Temperature (F)  100.01   0

Pressure Sensor              Pressure (atm)   2.203    0         PS-DIN

Pressure (psi)   32.37    0

Student Force Sensor         Force (N)        9.8      -9.8      SFS-DIN

pH System                    pH               -3.807   13.662    PH-DIN

Relative Humidity Sensor     Humidity         -36.562  35.74     RH-DIN

Light Sensor                 Light (0-600)    200      0         LS-DIN

Light (0-6000)   2220     0

Light (0-150K)   50000    0

Motion Detector              Distance (cm)    .03556   0         U-MD

Distance (in)    .014     0

# Specifying a Custom Calibration

To specify a custom calibration, enter an expression in the variable X, where X is the uncalibrated reading coming from the PLDL 100, and the value of the expression is the desired calibrated reading.

1.	Highlight the desired port.

2.	Type in the expression.

3.	Press [ENTER] to enter the calibration. The port is automatically checked.

In the special case of a linear expression such as 1.5*X+.65, you may enter a list containing the slope and intercept, such as {1.5,.65}.

# Setting Up a Motion Detector

To use a motion detector, plug it into port 4, specify a calibration, and check the SONIC checkbox.

Note that the standard calibration is only an approximation. Since the motion detector’s measurement is based on the speed of sound, local conditions such as temperature, humidity, and barometric pressure will affect the results. You can create your own calibration by taking data with a calibration of `Uncalibrated’, fitting the data, and using the fit results in a new calibration.

----------------------------------------------------------------------------

Monitoring Data

The Monitor Data view displays the current readings from the PLDL 100.

1.	Make sure the probes are set up correctly.

2.	Press [SHIFT][VIEWS] to show the VIEWS menu.

3.	Highlight `Monitor Data’ and press [ENTER]. The menu labels show:

o DIG

Takes readings and displays them in digital form.

o GRP

Plots 131 readings using the reading number for the X coordinate and the reading value as the Y coordinate, then pauses.

o START/PAUSE

Starts or pauses monitoring.

o SAVE

Takes a reading and appends it to the appropriate variables C1...C5.

4.	Press {START} to begin monitoring data. The display shows digital readings for the selected ports.

5.	If you are collecting occasional data, press {SAVE} as needed to save the current readings as Statistics data.

6.	For a graphical view of readings, press {GRP}. The HP 38G plots 131 readings, then pauses.

To adjust the vertical scale, press [SHIFT] [PLOT] to enter the Plot Setup view; adjust YRNG (Y-range); and return to the Monitor Data view.

To clear the display of previous plots, press {ERASE}. (This menu label appears only in the Graphical Monitor Data view.)

7.	Press {PAUSE} (or leave the view) to stop monitoring.

----------------------------------------------------------------------------

# Running an Experiment

To run an experiment you first specify the parameters for logging the data; then you tell the PLDL 100 to start logging; and finally you transfer the data back to the HP 38G.

Rather than immediately starting logging, you can specify a minimum or maximum valid reading that will trigger logging. The probe that makes the triggering measurements must be in port 3. You can save a fixed number of readings that immediately preceded the triggering reading.

While the PLDL is logging data, you can disconnect it from the HP 38G and use the calculator for other purposes.

To set up and run an experiment:

1.	Make sure the probes are set up correctly.

2.	Press [SHIFT] [NUM] to show the `Timing Setup’ form.

3.	Fill in the fields in the form:

o SAMPLE INTERVAL

The interval in seconds between readings.

o NO. OF READINGS

The number of samples to take.

o TRIGGER

The type of trigger condition:

- None (No triggering; start immediately).

- Minimum (Trigger when the reading on port 3 satisfies the specified minimum level).

- Maximum (Trigger when the reading on port 3 satisfies the specified maximum level).

If a trigger condition is specified, two additional fields appear:

o LEVEL

The minimum or maximum valid reading for the trigger.

o NO. PRETRIGGER READINGS

The number of pre-trigger readings to save.

4.	Press [SHIFT] [VIEWS] to show the VIEWS menu.

5.	Highlight `Log Data’ and press [ENTER]. The Log Data view appears, with the following menu labels:

o ABOUT

Displays version and copyright information about the PLDL 100 aplet.

o ->38G

Clears C1...C5 and transfers records from PLDL 100 memory to C1...C5, applying calibrations as specified in the numeric setup view. When complete, displays the message “nnn records received”.

o HALT

Instructs the PLDL 100 to halt data collection. Displays the message “Instrument halted”.

o INFO

Queries the PLDL 100 and displays “nnn records in memory” and either “Experiment running” or a blank line, depending on the instrument status.

o START

Instructs the PLDL 100 to begin the experiment as described in the numeric setup. Displays the message “Experiment started”.

6.	Press {START} to begin the experiment.

While the experiment is running, you can disconnect the HP 38G and use it for other purposes, including running other aplets. To return to the experiment, start the PLDL 100 aplet again, return to the `Log Data’ view, and reconnect the PLDL 100.

7.	To check on the progress of the experiment, press {INFO}.

8.	When the experiment is completed, press {->38G} to transfer records to the HP 38G.

----------------------------------------------------------------------------

# Analyzing Data

All the tools in the Statistics aplet are available for analyzing the data collected by the PLDL 100. The data from ports 1 to 4 are stored in Statistics variables C1 to C4. The timestamps for the readings are stored in C5.

The PLDL 100 aplet has the following additional tools to help in analyzing data:

- DeltaList

Calculates first differences of the data in the highlighted column and copies them to a selected column.

- Smooth 5-pt.

Performs 5-point smoothing on the data in the highlighted column, leaving the endpoints unchanged.

- Sort List

Sorts the data in the hightlighted column. (The numeric view’s standard SORT functionality.)

- Copy to list

Copies the data in the highlighted column to a selected column.

----------------------------------------------------------------------------

Monitoring Data From a Program

You can make an aplet easy to use by writing programs that automate each step and then offering those steps as custom views in the VIEWS menu. The PLDL 100 aplet includes a function you can use in programs to get a reading from the PLDL 100 Data Logger.

The READING function takes no arguments and returns the same readings as would appear in `Monitor Data’ view, bundled according to the number of ports checked:

Number of ports       Value returned

-------------------------------------------

0        No Probes Selected

1	Real number

2	Complex number

3	List of three real numbers

4	List of four real numbers

----------------------------------------------------------------------------

Aplet Error Messages

The following error messages may be generated by the aplet:

- Experiment Running

The PLDL 100 is running an experiment, so a new experiment cannot be started.

- Instrument Error

Unexpected or missing information was received from the PLDL 100 in response to a command.

- Invalid Record Number

The requested record number is not in the range of valid record numbers for the current experiment.

- Timeout

The PLDL 100 is not responding. Check that the PLDL 100 is connected properly to the serial port.

- Invalid Trigger Specification

The current trigger level or the calibration for port 3 is invalid.

- Non-Real Result

The value of the calibration expression was not a real number.