This document was created using MS-Word 97

Program : PHOT1-1.LIB (for the HP48S/G series)

Checksum: # 10718d

BYTES:  5464.5

Lib.#1501

by K. Rainer Birk

rbirk1@wanadoo.es

krbirk1@lycosmail.com

This program calculates DEPTH OF FIELD and HYPERFOCAL DISTANCE (and a few things more)

General information

The main areas of practical camera handling in photography concern sharpness control, exposure, and lighting.

The image on the film is sharpest when the lens is focused to the exact object distance. Usually, however, a scene includes objects at varying distances from the camera. Various factors affect the sharpness distribution in a picture of such a scene.

DEPTH OF FIELD

The sharpness in the image of objects in front of and behind the focused distance falls off gradually. Within a certain range of object distances this sharpness loss is still comparatively unnoticeable.

This range is the depth of field and depends on:

(1) the amount of sharpness loss regarded as acceptable: miniature negatives requiring big enlargement must be sharper than larger format negatives, which are enlarged less;

(2) the lens aperture used: stopping down the lens (higher f-numbers) increases the depth of field;

(3) the object distance: the depth of field is smaller for near objects than for more distant ones; and

(4) the focal length of the lens: depth of field is reduced with longer focus lenses (and with larger picture formats requiring lenses of longer focal length), and the depth increases with shorter focus lenses. A depth of field indicator, often included on the focusing mounts of lenses, shows on the distance scale how far in front of and behind the focused distance objects will be in focus at different diaphragm openings.

Unfortunately most Zoom lenses and also some modern lenses with fixed focal length do not have

depth of field markings. That's where this program wants to help.

Short Program Description

Inputs:

COC		CIRCLE OF CONFUSION

APERT		APERTURE (F-STOP)

DIST		DISTANCE of the object from the lens

FOC.L		FOCAL LENGTH  of the lens

MAGN		MAGNIFICATION

Outputs:

DEPTH OF FIELD ( defined by the distance of the nearest and the farthest object whose images can be

considered in focus or sharp, taking the COC as criterion.)

HYPERFOCAL DISTANCE

Definition: if you set your camera to the HYPERFOCAL DISTANCE  (H) then everything

from H/2 to infinity  will be sharp.

MAGN 		Linear MAGNIFICATION (Ratio:  Image / Object  or 1 : Object / Image)

D		(approximate) Distance between the object and the film

EXT		for close-ups: length of the EXTENSION TUBES or BELLOWS needed

Detailed Instructions

1.  Transfer the self - attaching library PHOT1.LIB to your HP 48.

2.  Recall the library to the stack and type : 0  [STO]

You can purge the original variable now.

3.  Turn the HP48 off, then on.

4. To start the program  press [Library] [PHOT1] [START] ... wait a while...[DOF=?]

You will be presented with the following screen

___________________________________

| Focal Length: 50 mm			|

| Aperture (f-stop) 8			|

| HYPERFOCAL DISTANCE:		|

| ---- > 10.47 m				|

| Distance 3.000  m			|

| DEPTH OF FIELD [m]:			|

| From  2.338   to   4.185			|            _________________________________

|  DOF=?  | FOC.L | APERT  |  DIST  |  COC  |  EXIT  	|  [NXT]  | MAGN | EXT=? |  D = ?  | METR | FEET | INCH |

Explanation of the Menu Buttons

Button		Action

[DOF=?]	Calculate  1)DEPTH OF FIELD  and   2)HYPERFOCAL DISTANCE

[FOC.L ]	Display / input* the  FOCAL LENGTH  ( F )

[APERT]	Display / input* the APERTURE (f-stop)

[  DIST ]	Display / input* the DISTANCE of the object from the lens (D)

[  COC ]		Display / input*  the  CIRCLE OF CONFUSION

Default Circle of Confusion:   .03 mm. This is a more or less reasonable  standard for the 35mm format.

Formats  	 Circle of Confusion:

35 mm			0.03 mm

6x6 cm      	 	0.06 mm
4x5 in   		 	0.15 mm

*If the stack is empty, pressing  [FOC.L] , [APERT], [DIST], or [ COC ]  causes the FOCAL LENGTH, the APERTURE,  etc.  to be displayed for two seconds. If there is a number in  level 1 of the stack this number

is used as the new value.

Example: Enter the number  16,  press [APERT]  to change the aperture to 16.

[ EXIT  ]	Leave the program (Surprise! )

[MAGN]	4)Calculate / input the MAGNIFICATION ( = 1: Object Size / Picture Size)

If nothing is on the stack then the current magnification is displayed, else the number in level 1 of the stack is  used as the new MAGNIFICATION .The corresponding values of DIST,  EXT, and D are calculated, (but not displayed, you have to press the appropriate menu buttons to see them)

[EXT=?]	5)Calculate the "EXTENSION" i.e. distance the lens has to be moved forward from the infinity position ( at F mm from the film plane) to focus on an object at a finite distance.

EXT  = F^2 / ( DIST – F)

As DIST decreases EXT must be increased. Down  to a certain minimum DIST this adjustment is done  by means of the focussing  ring of the lens. For close-ups extension rings may be needed.

[ D = ?  ]	3)Calculate the (approx.) Distance between the object and the film plane.

This calculation neglects the distance between the Principal Planes of the lens, which depend on the lens design.

[METR ]	Set the mode to METERS**

[ FEET ]	Set the mode  to Feet**

[ INCH ]	Set the mode to INCHES**

**Applies to the  Distances only. FOCAL LENGTH of the lens and CIRCLE OF CONFUSION use mm (1/1000 METER ) as units.

The default UNIT for the distances is METER (m)

You can switch between modes pressing  [FEET]  ,[INCH] ), or  [METR]

None – Menu Buttons

[CST] can be used to change the number of decimals

(For display of DIST, the Hyperfocal Distance , and Depth of Field limits)

Technical Notes:

1)The focus calculation assumes a simple, single element lens with the focus distance measured

from the center of the lens to the object. In reality, most lenses have multiple elements and it is  assumed that

the focus distance is from middle of the lens assembly (lens barrel) to the object. But at close focus this may

be inaccurate because there are often two Principal Planes. Precise close-up focusing requires

determining these planes. Here is how it's done:

When a camera lens is focussed at infinity, the rear principal plane is exactly one focal length in front of the film.

To find the front Principal Plane, take the lens off the camera and let light from a distant object pass through it "backwards". Find the point where the image is formed, and measure toward the lens one focal length. (If  the object is closer than infinity add  EXT to the focal length to locate the principal planes.)

With some lenses, particularly ultra wide-angle ones, you can't use this method, since the image is not formed in front of the front element. (This all assumes that you know the focal length. I suppose you can trust the manufacturer's numbers enough for educational purposes.)

For my 135 mm  Minolta Tele-lens I got the following results:

The rear principal plane coincides with the front of the lens barrel. The front principal plane lies about 9.5 mm in front (i.e. outside the lens assembly).

2)The focusing ring on the lens normally refers to the distance (D) of the object to the  film plane, while DIST  is the distance of the object to the lens. This difference is under normal picture taking conditions negligible.

But the closer to the lens the object gets, the larger becomes the difference between  D and DIST.

For close-up photography  ( DIST <  10  F or so ) it is not practical to use D because of its weird behavior

described by the equation D = F (1 / MAGN +  MAGN + 2)

When the object approaches the lens  D decreases,  reaches a minimum  of  4 F, and on further approach to the lens D actually increases ferociously. Thus for all values of  D, (except 4F) there actually exist two values of  DIST, a large one for a scale of e.g. 1:10 and a small one for  10:1.

You can check this out entering  0.1 and 10 for  [MAGN]) and read  D and DIST for either case.

Note: The word "scale" is used here instead of "magnification". It seems silly to talk about a "magnification of 1:10" when the image is smaller than the object.

3) The program does not take into account  the additional 'confusion' caused by diffraction effects

which become  significant  at very small apertures (i.e. large f-stops).

Have fun!

K.R.B

Further Reading:

Encyclopaedia Britannica

http://www.photo.net/photo/optics/lensTutorial