BODE-ROUTH

v6.3

Plus!

(c)APO

for the HP48-G,GX

FIRST OF ALL

============

English is not my native  language  so  I'd like to apologize for

the errors in this doc.

I'd also thank all people who takes a moment of its life to send

me a coment about this or another one library.

THANKS !!!

NOTE:

Along this document you'll find some notes like this one. Please

read them; they are important.

THE FILES

=========

BodeR6x3.lib    Bode-Routh library with spanish texts v6.3.

BodeReng.lib    English version 6.3.

BodeR6x3.esp    Spanish manual.

BodeR6x3.eng    English manual.

INSTALATION

==========

WARNING !!!

THIS LIBRARY  NEEDS THE  POLYNOMIAL LIB "NEOPOLYS", VERSION 5.3

OR LATER (the last version at this moment is v5.6)WITH THE LAPLACE

AND Z TRANSFORMS COMMANDS.

Just download the .lib you  want ( either the spanish version or

the english one ) with the kermit server. Now put the library on the

stack and do "0 STO" (or "n STO" if you want to  install the library

on port n rather than in port 0) and reboot your HP by pressing ON-C

or by turning it off and on.

THINGS YOU HAVE TO KNOW

=======================

Polynomials are represented as in NEOPOLYS lib  either  by their

coefficients or by their roots. For instance:

x^3-2x^2-5x+6

can be represented as

{ 1 -2 -5 6 }

or

{ { 1 3 -2 } }

NOTE:

I recommend you to read the Neopolys manual to learn more a-

bout that library.

COMMAND REFERENCE

=================

I think it's very obvious  what each command does . Moreover they

are very easy to understand and have a nice use because almost all of

them were made with input forms whose  fields you  have to fill . The

default values of the fields are as often as  not correct  enough  to

obtain a nice result so you'll have to change only a few of them.

Bode

----

With this command you can draw either the  magnitude or the angle

Bode plot for the transfer function whose numerator  is  given at the

field "NUM" and whose  denominator is the one at field "DEN" . In the

magnitude plot vertical  axis is in  decibels and in the angle one in

degrees. In both the horizontal axis is Log(w).

* The fields:

NUM,DEN : Transfer function's numerator and denominator.

D   : Transfer function's delay (if any).

T   : Choose the plot type to draw.

RES : Graphic resolution. A number between  0 and 1  representing

the distance between two consecutive points of the plot.

WARNING !!!

Due to a correction used to get  past the 180  range when

drawing the angle  a small value in this  field can lead in

drawing an incorrect plot. If you don't know  what would be

better either to change this value or not just let it as is

to ensure that the plot will be correct.

AUTO: Check this field to automatic scale vertical axis.

HR

VR  : Horizontal and vertical ranges of the plot.

* The menu:

EDIT,CHK

CHOOS: The normal input form menu for editing fields.

PICT : Press here to go to PICTURE.

ERASE: Erase PICT.

DRAW : Press here to start drawing.

After pressing NXT...

LABL : Check this key to  label the  plot before  drawing it with

the minimum and maximum axes values.

AXES : Check it to draw the axes.

DATA : This option allows you to calculate the gain and phase mar-

gins and associated frequencies as well as the bandwidth.

NOTES:

Before entering Bode you can  push on levels 1 and 2 the transfer

function and the fields NUM and DEN will take those values . Likewise

when exiting Bode you'll get the transfer function on the stack.

At any moment you can press the DEL key to reset values of one or

the whole input form fields . The default  values for the NUM and DEN

fields are those one they have the first time you entered Bode.

When drawing the angle diagram you'll see a dotted line at 180.

Nyquist

-------

* The fields:

All fields have the same meanning that in the  previous command .

There is just one different which is "w" . This value is the range of

values that the variable w takes when drawing the plot . Thus a value

of  6.5  means  that the range is from -6.5 to 6.5 . You can increase

this value to obtain more resolution near the (0,0).

* The menu:

Is exactly the same that in the previous command.

NOTES:

If you see that drawing spend a lot of time it  must  be due to a

great value of the "w" field . Just  press the ON key to abort at any

moment.

The autoscale function is  not very efficient for great values of

"w".

Nichols

-------

With this  command you can plot the ABS(G(jw)) vs ARG(G(jw)) dia-

gram also known as Nichols' diagram, for the transfer function G(s).

* The fields:

They are exactly the same that those one for the Nyquist.

* The menu:

The same thing of the same one.

NOTES:

I could have applied the correction we talked  about  in the Bode

command about the 180 limit but I  prefered to let it as is because

thus you can easily  calculate  approximately the gain and phase mar-

gins. For instance:

Push on the stack the next transfer function:

2:               { 4 }

1:         { 1 2 4 0 }

and now press "Nichols". Choose a resolution of .1 and press "DRAW" .

After a while you'll see how the plot jumps from left to right of the

LCD. Well, press the ON key and place the cursor where the plot cross

the horizontal  axis and  if you  press the (X,Y) menu key you'll see

something like this:

X: -1.30E2          Y: -3.17E-1

Now add 180 degrees to the x value and you'll obtain the phase margin

(or substract -180):

PHASE MARGIN = -130+180 = 50.

To obtain the gain margin just place the pointer to the right , where

the plot arrived  after  jumpping one side to  another and you'll see

the new coords:

X: 180          Y: -6.33E0

and now you can obtain the gain margin just doing (-1)Y, that is :

GAIN MARGIN = (-1)(-6.33) = 6.33.

If you want to obtain the exact  values of  those  margins enter Bode

and use the "GFM" option in the menu. You'll obtain:

gain margin  =  6.32059991328 dB

phase margin = 50.290385343

Easy!.

See also the Nyquist note about the "w" field.

RLoc

----

This command calculates and plots the locus of the roots of

NUM

1 + K ------- = 0

DEN

for a set of gains K.

* The fields:

RES : Resolution . This field represents the amount added to K at

each step.

AU  : Check it to autoscale  horizontal and vertical axesa utoma-

ticly.

LIN : By default the root  locus  is  drawing with points . Check

this field to connect the points.

* The menu:

There is just one different option:

DATA : By pressing this  option you'll see  another  window where

you'll can choose between calculate one of:

* open loop poles

* open loop zeros

* closed loop poles ( unitarian negative

feedback is supposed )

* stability

The last option let  you see the  coordinates at which the

root locus crosses the vertical axis and  values of gain K

for those points.

NOTES:

Sometimes it seems that nothing is being drawing. Posible causes and

their solution are:

1) The resolution is very high and the first points in the plot

are out of range.

SOLUTION:

decrease the value at field 'RES' (dividing it by 5 is

almost always satisfactory).

2) The resolution is very low and it spends a lot of time to plot

each point.

SOLUTION:

increase the value at field 'RES' (multiplying it by 5 is

almost always satisfactory).

When drawing the root locus you'll see  some "x" at the open loop

poles and some "o" at the open loop zeros.

When drawing, press any key to stop or the ON key to abort.

Resp

----

With this  command you can obtain and plot the response of a sys-

tem to some different inputs such as pulse, step, ramp and sine for a

CONTINUOUS or DISCRETE system.

Notice that the fields NUM and DEN can be either for a CONTINUOUS

or for a DISCRETE system, and that they must be the numerator and

denominator of a CLOSED LOOP TRANSFER FUNCTION.

* The fields:

T   : Input signal type.

A   : When working with CONTINUOUS systems introduce in this

field the input signal amplitude.

When working with DISCRETE systems introduce here the

sampling period.

Particularizing for each input:

* step: in this case this value is the step magnitude

* ramp: represents the ramp inclination

* pulse: in this case its value is not considered

* sine: wave amplitude.

w   : This field has only meanning when working with the sine in-

put and represents the wave frequency.

AUTO: Autoscale vertical axis.

* The menu:

INPU: Activate this option to draw the input with points.

It has no meanning for a "Pulse" input.

DATA: With this option you can obtain the characteristic values

for the system response given by the CLOSED LOOP transfer

function at fields NUM and DEN, supposing an UNITARIAN STEP

INPUT.

LINS: Deactivate this option to plot with points rather than with

lines.

DISC: Selects if you're going to draw the response for a DISCRETE

system (when activated) or for a CONTINUOUS one (when deac-

tivated). By default is deactivated.

NOTES:

In the EQ variable you'll  obtain the  response  function that is

the transfer function's reverse Laplace transform  along with the in-

put signal.

DON'T FORGET TO ACTIVATE "DISC" IF YOU ARE WORKING WITH A DISCRETE

TIME SYSTEM.

Routh

-----

Applies the Routh-Hurwitz stability test to the  polynomial given

at level 1. The result is the  first  column  of the  Routh matrix at

level 1 and the whole matrix at level 2.

Special cases  such as null rows or a null first coef in the pre-

vious row are also considered. In the last case this coef will be re-

placed with 1.E-50.

You can also use constants and symbolic expressions.

Example:

2:                        Routh     2:  { { ......... } }

1:      { 1 2 3 4 5 } ->->->->->->  1:     { 1 2 1 -6 5 }

meanning:

+-----------------------------------+

s^4 + 2s^3 + 3s^2 + 4s + 5 = 0

+-----------------------------------+

+---+

s^4 :  1  3   5

s^3 :  2  4   0

s^2 :  1  5   0

s^1 : -6  0   0

s^0 :  5  0   0

+---+

NOTE:

The result will depend on the state of user flags 2 and 3 . These

flags have the same meaning that in Neopolys, that is, flag 2 simpli-

fies final result and flag 3 search for numeric rational results.

C2DM

----

With this command you can transform the continuous transfer func-

tion given at levels 3 (numerator) and 2 (denominator) to its discre-

te equivalent, with the sample period given at level 1. You must be-

fore choose the method you want use to make the conversion:

Zoh: supposing a zero holder. In this case, given the

continuous transfer function G(s), the conversion

is:

G(z) = (1-Z^-1)Z[G(s)/s]

bilineal: bilineal approximation (or 'tustin' method),

consisting of making the substitution

2   (z-1)

s = - -----

T   (z+1)

->CL (Close the loop)

----

Given the direct transfer  function and the  feedback one returns

the closed loop transfer function. Negative or  positive  feedback is

considered depending on the sign of the real number given at level 1.

+

U(s)---->0---> G(s) -------> Y(s)

/\             |

|               |

---- H(s) <----

G(s)

=> F(s) = ---------------

1  G(s)H(s)

sign:

+ <=> negative feedback

- <=> positive feedback

For example:

5:               { 1 }   \

4:           { 1 2 4 }   / G(s)

3:             { 1 2 }   \

2:               { 1 }   / H(s)

1:                   5   (positive feedb.)

->CL    =>  4:               { 1 }   \

3:           { 1 1 2 }   / F(s)

or:

5:               { 1 }   \

4:           { 1 2 4 }   / G(s)

3:             { 1 2 }   \

2:               { 1 }   / H(s)

1:                  -5   (negative feedb.)

->CL    =>  4:               { 1 }   \

3:           { 1 3 6 }   / F(s)

For H(s)=1 just push the polynomial { 1 } at levels 2 and 3.

THE AUTHOR

==========

I'd like to know if you found this lib usefull.

If you found any bug or you have any suggestion please e-mail me

to

apolo@sorolla.eis.uva.es

LIST OF CHANGES

===============

CHANGES FROM VERSION 6.1

Added the command 'C2DM'.

The 'Resp''s menu option 'CNCT' has been substituted by 'DATS',

allowing you to calculate some data for a step response.

It has been added the posibility to calculate the bandwidth inside

the 'Bode' command.

There were corrected some minor bugs and optimized the code.

The default values for the fields 'AUTO' and 'RES' in the 'RLoc'

command have been changed.

The command 'RLoc' has been completely rewritten to gain for some

speed.

CHANGES FROM VERSION 6.0

The commands were updated to take advantage of the last version

(5.3) of Neopolys.

There were corrected some mistakes in the manuals.

CHANGES FROM VERSION 5.9

I made a mistake in version 5.8 for which the graphs were always

plotted without connecting the points.

The graphics display was not correctly reset by the menu option

'ERASE', which made that plot dimensions were the previous graphic

dimensions.

There was corrected the error checking of the horizontal and verti-

cal plot ranges and now you can introduce the lower and greater

values in any of the two fields representing the horizontal and

vertical ranges, and they will be placed in the correct position

after drawing the plot and returning to the input form.

CHANGES FROM VERSION 5.8

There was corrected a bug which made the command 'GFM' return

incorrect values for some transfer functions.

An option was added to the command 'Resp' in order to be able to

draw the response plots either with lines or with points ( es-

pecially for discrete responses ).

CHANGES FROM VERSION 5.6

In previous versions the transfer function delay was not consi-

dered when calculating the gain and phase margins. Now it does.

Due to the incorporation of the Z transform calculation commands

in Neopolys, the command "Step" can now graph the response of a

continuous system or a discrete one.

The whole commands were updated in order to make use of the new

advantages of Neopolys version 4.4.

CHANGES FROM VERSION 5.5

The notation used in version 5.5 for Wcg and Wcf (in the GFM

command in "BODE" one ) were not correct. Really, Wcg was Wcf and Wcf

was Wcg. That was corrected.

In the "->CL" command, the feedback sign considered to close the

loop was not the correct one. That was corrected.

Another few changes were made in order to work properly with Neo-

polys (v4.1).

CHANGES FROM VERSION 5.1

Almost the whole set of commands have been revised to make use of

the new Neopolys v4.0 advantages , and an extensive code optimization

have been made leading in a great speed increase.

There were  corrected some  terms of the  manuals  which were not

correct as for example when talked about closed loop poles in v5.1

really I was talking about the open loop poles, etc...

The Routh command considers the state of user  flags 2 and 3 when

calculating and showing the results.

CHANGES FROM VERSION 5.0

There was corrected an error in the "Bode" command for  which the

results in the  calculations of the  gain and  phase margins were not

correct when working in "RAD" mode.

When drawing the response to an  input of  type "Pulse" the graph

spent some time trying to draw the input function. That was corrected

and now the "Pulse" input function is not  drawn even when the option

"INPU" is activated.

CHANGES FROM VERSION 2.3

There are so many many many many .....