����������������������������������Ŀ

� EZCHAR V 1.2, by Daniel Lidstrom �

� 980413                           �

������������������������������������

Introduction

============

EZCHAR is meant to be a replacement of the (slow) internal

character browser (G/GX). It is written in assembly as my first major

project. It has been tested to work fine under low-memory conditions

in port 0/1 on a GX-R. I don't have my SX any longer so I don't know

about compatibility.

Installing

==========

Send EZCHAR.LIB to your HP48 in binary mode using your favorite

transfer program. Recall the variable to the stack, purge the original from

the var menu, store the library in any port:

'EZCHAR.LIB' DUP RCL SWAP PURGE 0 STO

substitute the 0 with a port of your choice.

Now turn off and then on your HP48. The EZCHAR should now be installed.

UFL

===

This application utilizes a utility called Universal Font Library. This is a

standardised set of two fonts used to get quicker access to fonts. You

need to download these fonts and install them in the same manner as

with EZCHAR.

Size and checksum

=================

EZCHAR.LIB      lib no. 640

size:           2044

checksum:       # 7042h

Commands

========

Switch to the EZCHAR submenu by pressing RS LIBRARY (S/SX: LS LIBRARY)

followed by the menu key for EZCHAR.

EZCHAR: this is the browser

EZINIT: this program assigns a small program to key 22.3. This is the

key used to invoke the inbuilt browser on the G/GX. This will

allow you to start the browser even in the middle of an editing

session! The pushed characters will be pushed right into the text

you are editing. I strongly suggest you do this. Don't forget to

turn on USER mode.

Browsing

========

ENTER:          Push a character

RtArrow:        Move cursor right

LtArrow:        Move cursor left

UpArrow:        Move cursor up

DnArrow:        Move cursor down

ON:             Exit

Daniel Lidstrom

danli97@ite.mh.se

http://www.ite.mh.se/~danli97/

980413 Sundsvall, Sweden