Introduction to HP 48SX Internals (version 0)     (C) 1990, Alonzo Gariepy

========================================================================

Following is some information that may be useful if your interests

run to machine language.  Misapplication of the information contained

in this document may result in corruption of memory and even damage

to the hardware.  Backup the contents of your HP 48SX if necessary.

Objects

=======

Address   Type Name            Example Display          Syntax (if different)

-----------------------------------------------------------------------------

02933     0    Real            3.14159265359

02977     1    Complex         (1,2)

02a2c     2    String          "hello" or C$ 5 hello

029e8     3    Real Array      [1 2 3]

029e8     4    Complex Array   [(1,0) (2,2)]

02a74     5    List            { DUP 3 '3/4' }

02e48     6    Global Name     'PEEK'

02e6d     7    Local Name      't'

02d9d     8    Program         << 440 .5 BEEP >>

02ab8     9    Algebraic obj   'SIN(X)'

02a4e     10   Binary Integer  # 454432h

02b1e     11   Graphics obj    Graphic 4 x 4            GROB 4 4 70607050

02afc     12   Tagged obj      root1: 2.6               :root1:2.6

02ada     13   Unit obj        1_lyr                    1_lyr

02e92     14   XLIB name       XLIB 2 261               none

02a96     15   Directory       DIR Avog 6.02E23 END

02b40     16   Library         Library                  none

02b62     17   Backup obj      Backup Object            none

02e92     18   Function        SIN

02e92     19   Command         SWAP

02911     20   Address         <28FCh>                  none

02955     21   Long Real       Long Real                none

0299d     22   Long Complex    Long Complex             none

02a0a     23   Linked Array    Linked Array             none

029bf     24   Character       Character                none

02dcc     25   Code            Code                     none

02b88     26   Library Data    Library Data             none

other     27   External        External                 none

*    *    *    *    *    *

Memory Layout

=============

00000-0FFFF    ROM (and registers for the display controller, I/O, and timers)

10000-6FFFF     ROM

70000-7FFFF    User/Display RAM (overlaid on Font/Strings ROM)

80000-BFFFF     128k for plug-in

C0000-FFFFF     128k for plug-in

F0000-FFFFF    User/Display RAM (when 70000-7FFFF is used for Font/Strings ROM)

Memory Scanner

==============

There is a built-in memory scan mode that lets you examine and modify

memory.  This is a convenient way to enter machine code programs into

the calculator, to examine memory, and to download memory to a PC via

the serial cable.

In the key descriptions below, I have referred to several of the keys

by their orange labels because the title on the key is a non-ascii symbol.

None of these keys should be orange shifted, however.

Back up your calculator if necessary.  To get into the memory scanner,

press [ON][D] together, then [DROP].  You are now in memory scan mode.

Following is a list of keys and the functions they perform in scan mode.

Most keys will repeat if held down for a moment.  All numbers are in hex.

[DROP]         refresh

[LIBRARY] increment by 1000

[REVIEW]  decrement by 1000

[/]       decrement by  100

[*]       increment by  100

[-]       decrement by    1

[+]       increment by    1

[ENTER]        goto address 00100 (?display controller?)

[+/-]          goto address F000A

[1/X]          goto address F0A8C or F1210 (display, see below)

[EEX]          goto address 80000 (?port 1?)

[DEL]          goto address C0000 (?port 2?)

[0]-[F]        enter corresponding nibble at address, increment by 1

[.]       transmit 16 nibbles to IR and serial, increment by 10

[SPC]          dump 1000h memory locations to the serial port (9600 baud)

[EVAL]         execute at address

When you first press [DROP] to enter scan mode, the address is

705D9.  Pressing [EVAL] at this address displays:

Version HP-48A Copyright HP 1989.

The most interesting thing about scan mode is that the memory layout

is different than in normal operation.  The 32k of user/display RAM

is moved from 70000 to F0000 so that you can see the ROM that is normally

hidden underneath.  In normal operation, the address space at 70000 is

shared between this ROM and display RAM.  The hidden ROM is used for code

and data related to I/O, such as strings and font bitmaps, as well as for

diagnostic functions, such as the self tests and scan mode.

In normal operation, whenever you need access to the hidden ROM, you have

to perform some magic to switch the address space.  This must be done in

the few cycles between display refresh.

F0A8C and F1210 are the addresses of the second line of the bitmap,

depending on whether you were looking at the stack or plot displays

when you entered scan mode.  If you press one of the keys [0]-[F] after

pressing [1/X], you can see the effect of writing each nibble into the

display bitmap.  You can draw on the display this way.

*    *    *    *    *    *

The first variable in the HOME directory is stored ending at #7FFFAh,

unlike the HP28, where this variable ended at #CFFFF.  Address #7FFFAh

appears at FFFFA in the memory scanner.  Once you have use libraries

or ports, you can no longer easily find the first variable's location!!

I believe that coma mode is entered with [ON][SPC].  When you press on,

much of the calculator will have been reinitialized.  One undocumented

command is WSLOG, which gives you a log of warm starts.  I cannot find

any command strings with my FIND program.  They are most probably stored

in the hidden ROM.

*    *    *    *    *    *

Easy Machine Code: PEEK, POKE, and FIND

=======================================

The obvious way to enter machine code is with scan mode.  You don't

need to waste time with hex string converters and other silliness.

Store the following PEEK program template in the first variable of the

HOME directory.

<< RCWS SWAP 64 STWS #0h OR SWAP STWS

"ABCDEFGHIJKLMNOPQRSTUVWXYZ" >>

HOME 'PEEK' DUP PURGE STO

Now go into scan mode and find the start of the string object:

[ON][D], [DROP], [ENTER], [/], hold down [-] until you get to FFFB3.

Now change the object type (C2A20) to

CCD20

Note that the length is 93000 -> 39 hex = 57 decimal = 26*2 + 5.

Move forward to address FFFBD using the [+] key, and enter the machine

code into the string.  Remember, don't type past FFFF0.

13210314313016914613615671301691547113132142164808C0

Press [ON][C] together to get back to the calculator.

You can now checksum PEEK with the BYTES command.  The result should

be #8568h.  This is a binary checksum, so it includes the contents of

the Code object, and is independent of display modes.

Store the following POKE program template in the first variable of the

HOME directory.

<< SWAP OVER #0h AND OR SWAP

"123456789 123456789 123456789 123456789 123" >>

HOME 'POKE' DUP PURGE STO

Go into scan mode, change type of the string to a Code object as above,

and enter the machine code into the string.  This string is longer, so

you have to go back further to get to the beginning.  The machine code

still ends at the same address.  Be sure you understand the format of

string and code objects so that you don't make a fatal mistake.

13210314317414717413416414613618513680D0164

1561E71301691421301541113132142E720164808C0

Press [ON][C] together to get back to the calculator.

You can now checksum POKE with the BYTES command.  The result should

be #725D.

*    *    *    *    *    *

Mini RAM Map

============

The address of the end of the stack, normally stored in register D1, is

saved at #70579.  To examine the stack, first go to F0579 in scan mode:

[1/X], [/] 5 times, [-] 19 times

The address of the end of the stack is displayed in reverse.  If your

display says

F0579:9F8E7309E7...

then the stack starts at 7E903 and ends at 7E8F9.  There are obviously

two objects on the stack.  By moving to FE8F9, you can see the stack

itself, and the two addresses it contains.  Here is a list of interesting

locations (the initial 7 is mapped to an F in scan mode):

Real Address   Description

#704EAh        key buffer

#70579h        top (end) of stack

#7057Eh        bottom (start) of stack

#70583h        ?? local variables ??

#70588h        ?? internal loop ??

#7058Dh        ?? menu keys ??

#70592h        HOME directory

#70597h        end of HOME directory (7FFFB)

(points to five 0 nibbles, probably used for ATTACH)

#7059Ch        current directory

#70713h        graphic obj used for composing lines of the stack.  There

is just enough memory reserved for a 19 character bitmap.

#70844h        graphic obj used for menu display: 8 by 131

#70968h        graphic obj used for the rest of the screen: 56 by 131

#710ECh        graphic obj used for plot display: variable size

The questionable values are inferred from the HP-28 RAM map by Dave Kaffine.

There are obviously many similarities.  You can use the real addresses at

#7xxxxh employing PEEK.

*    *    *    *    *    *

Keyboard Buffer and Scan Codes

==============================

The keyboard buffer works just like in the 28.  The scan codes run in

sequence: #01h for A, #02h for B, ..., #30h for SPC, #31h for +.  There

are 49 keys and the scan codes run from 1 to 49 (#01h to #31h), with

four exceptions.  The key codes #1Eh, #23h, #28h, and #2Dh are not used.

The first three are replaced with #80h for AlphaShift, #40 for LeftShift,

and #C0 for RightShift, and the ON/ATTN/CONT/OFF key has no scan code.

When a key is pressed, the interrupt handler puts the scan code into

the next position of the keyboard buffer, except for the ON key which

gets special treatment.  I assume that the shift values can be OR'd

with the scan value to get more compact key codes.  Obviously one can

not apply mutliple shifts to the same character this way.

*    *    *    *    *    *

Command List and SYSEVAL Map

============================

Here are all the commands in the HP48 with their command number and

address.  This list serves as a map for navigating through the ROM.

Each address is the beginning of an RPL routine.  Note how short these

routines are.  The 48 has been set up so that command addresses never

change.  Disassembling the commands will lead the way to the underlying

RPL routines stored elsewhere in ROM and eventually a complete SYSEVAL

map.

On the 48, for the first time, programming with these faster SYSEVAL

addresses is easy (it can be done on the PC and downloaded).  Of course,

that defeats the whole purpose of having the standardized ROM addresses

below.  A utility for speeding up user programs could automatically

expand commands into the underlying RPL.  Once a program is tested, some

of the built in error checking code can be circumvented.  This is kind

of like a compiler since your source would be user code.

Addr   XLIB # Name         Addr   XLIB # Name          Addr   XLIB # Name

----------------------     ----------------------      ----------------------

1957B  000002 ASR          1C3CF  089002 GRAD          1FC7F  116002 DUPN

1959B  001002 RL           1C3EA  08A002 FIX           1FC9A  117002 PICK

195BB  002002 RLB          1C41E  08B002 SCI           1FCB5  118002 ROLL

195DB  003002 RR           1C452  08C002 ENG           1FCD0  119002 ROLLD

195FB  004002 RRB          1C486  08D002 STD           1FCEB  11A002 CLEAR

1961B  005002 SL           1C4A1  08E002 FS?C          1FD0B  11B002 STOsigma

1963B  006002 SLB          1C520  08F002 FC?C          1F"2B  11C002 CLsigma

1965B  007002 SR           1C559  090002 BIN           1FD46  11D002 RCLsigma

1967B  008002 SRB          1C574  091002 DEC           1FD61  11E002 sigma+

1969B  009002 R->B         1C58F  092002 HEX           1FD8B  11F002 sigma-

196BB  00A002 B->R         1C5AA  093002 OCT           1FDA6  120002 Nsigma

196DB  00B002 CONVERT      1C5C5  094002 STWS          1FDC1  121002 CORR

1971B  00C002 UVAL         1C5FE  095002 RCWS          1FDDC  122002 COV

1974F  00D002 UNIT         1C619  096002 RCLF          1FDF7  123002 sigmaX

19771  00E002 UBASE        1C67F  097002 STOF          1FE12  124002 sigmaY

197A5  00F002 UFACT        1C78C  098002 ->LIST        1FE2D  125002 sigmaX^2

197F7  010002 TIME         1C79E  099002 R->C          1FE48  126002 sigmaY^2

19812  011002 DATE         1C7CA  09A002 RE            1FE63  127002 sigmaX*Y

1982D  012002 TICKS        1C819  09B002 IM            1FE7E  128002 MAXsigma

19848  013002 WSLOG        1C85C  09C002 SUB           1FE99  129002 MEAN

19863  014002 ACKALL       1C8EA  09D002 REPL          1FEB4  12A002 MINsigma

1987E  015002 ACK          1C95A  09E002 LIST->        1FECF  12B002 SDEV

1989E  016002 ->DATE       1C98E  09F002 C->R          1FEEA  12C002 TOT

198BE  017002 ->TIME       1C9B8  0A0002 SIZE          1FF05  12D002 VAR

198DE  018002 CLKADJ       1CAB4  0A1002 POS           1FF20  12E002 LR

198FE  019002 STOALRM      1CB0B  0A2002 ->STR         1FF7A  12F002 PREDV

19928  01A002 RCLALRM      1CB26  0A3002 STR->         1FF9A  130002 PREDY

19948  01B002 FNDALRM      1CB46  0A4002 NUM           1FFBA  131002 PREDX

19972  01C002 DELALRM      1CB66  0A5002 CHR           1FFDA  132002 XCOL

19992  01D002 TSTR         1CB86  0A6002 TYPE          1FFFA  133002 YCOL

199B2  01E002 DDAYS        1CE28  0A7002 VTYPE         2001A  134002 UTPC

199D2  01F002 DATE+        1CEE3  0A8002 EQ->          2003A  135002 UTPN

1A105  020002 CRDIR        1CF7B  0A9002 OBJ->         2005A  136002 UTPF

1A125  021002 PATH         1D009  0AA002 ->ARRAY       2007A  137002 UTPT

1A140  022002 HOME         1D092  0AB002 ARRY->        2009A  138002 COLsigma

1A15B  023002 UPDIR        1D0DE  0AC002 RDM           200C4  139002 SCLsigma

1A194  024002 VARS         1D186  0AD002 CON           200F3  13A002 sigmaLINE

1A1AF  025002 TVARS        1D2DC  0AE002 IDN           2010E  13B003 BINS

1A1D9  026002 BYTES        1D392  0AF002 TRN           20133  13C002 BARPLOT

1A2BC  027002 NEWOB        1D407  0B0002 PUT           20167  13D002 HISTPLT

1A303  028002 KILL         1D5DF  0B1002 PUTI          2018C  13E002 SCTRPLT

1A31E  029002 OFF          1D7C6  0B2002 GET           201B1  13F002 LINFIT

1A339  02A002 DOERR        1D8C7  0B3002 GETI          201D6  140002 LOGFIT

1A36D  02B002 ERR0         1DD06  0B4002 V->           201FB  141002 EXPFIT

1A388  02C002 ERRN         1DE66  0B5002 ->V2          20220  142002 PWRFIT

1A3A3  02D002 ERRM         1DEC2  0B6002 ->V3          2025E  143002 BESTFIT

1A3BE  02E002 EVAL         1E04A  0B7002 INDEP         202CE  144002 SINV

1A3FE  02F002 IFTE         1E07E  0B8002 PMIN          2034D  145002 SNEG

1A4C0  030002 IFT          1E09E  0B9002 PMAX          203CC  146002 SCONJ

1A52E  031002 SYSEVAL      1E0BE  0BA002 AXES          2044B  147002 STO+

1A584  032002 DISP         1E0E8  0BB002 CENTR         20538  148002 STO-

1A5A4  033002 FREEZE       1E126  0BC002 RES           2060C  149002 STO/

1A5C4  034002 BEEP         1E150  0BD002 *H            20753  14A002 STO*

1A5E4  035002 ->NUM        1E170  0BE002 *W            208F4  14B002 INCR

1A604  036002 LASTARG      1E190  0BF002 DRAW          209AA  14C002 DECR

1A71F  037002 WAIT         1E1AB  0C0002 AUTO          20A15  14D002 COLCT

1A858  038002 CLLCD        1E1C6  0C1002 DRAX          20A49  14E002 EXPAN

1A873  039002 KEY          1E1E1  0C2002 SCALE         20A7D  14F002 RULES

1A8BB  03A002 CONT         1E201  0C3002 PDIM          20A93  150002 ISOL

1A8D8  03B002 =            1E22B  0C4002 DEPND         20AB3  151002 QUAD

1A995  03C002 NEG          1E25F  0C5002 ERASE         20AD3  152002 SHOW

1AA1F  03D002 ABS          1E27A  0C6002 PX->C         20B20  153002 TAYLR

1AA6E  03E002 CONJ         1E29A  0C7002 C->PX         20B40  154002 RCL

1AABD  03F002 pi           1E2BA  0C8002 GRAPH         20CCD  155002 STO

1AADF  040002 MAXR         1E2D5  0C9002 LABEL         20D65  156002 DEFINE

1AB01  041002 MINR         1E2F0  0CA002 PVIEW         20EFE  157002 PURGE

1AB23  042002 e            1E31A  0CB002 PIXON         20FAA  158002 MEM

1AB45  043002 i            1E344  0CC002 PIXOFF        20FD9  159002 ORDER

1AB67  044002 +            1E36E  0CD002 PIX?          210FC  15A002 CLVAR

1ACD7  045002 +            1E398  0CE002 LINE          2115D  15B002 TMENU

1AD09  046002 -            1E3C2  0CF002 TLINE         21196  15C002 MENU

1ADEE  047002 *            1E3EC  0D0002 BOX           211E1  15D002 RCLMENU

1AF05  048002 /            1E416  0D1002 BLANK         211FC  15E002 PVARS

1B02D  049002 ^            1E436  0D2002 PICT          2123A  15F002 PGDIR

1B185  04A002 XROOT        1E456  0D3002 GOR           2125A  160002 ARCHIVE

1B1C4  04B002 XROOT        1E4E4  0D4002 GXOR          2133C  161002 RESTORE

1B278  04C002 INV          1E572  0D5002 LCD->         2137F  162003 MERGE

1B2DB  04D002 ARG          1E58D  0D6002 ->LCD         213D1  163002 FREE

1B32A  04E002 sigmaN       1E5AD  0D7002 ->GROB        2142D  164002 LIBS

1B374  04F002 sqrt         1E5D2  0D8002 ARC           21448  165002 ATTACH

1B426  050002 SQ           1E606  0D9002 TEXT          2147C  166002 DETACH

1B4AC  051002 SIN          1E621  0DA002 XRNG          21E75  167002 XMIT

1B505  052002 COS          1E641  0DB002 YRNG          21E95  168002 SRECV

1B55E  053002 TAN          1E661  0DC002 FUNCTN        21EB5  169002 OPENIO

1B5B7  054002 SINH         1E681  0DD002 CONIC         21ED5  16A002 CLOSEIO

1B606  055002 COSH         1E6A1  0DE002 POLAR         21EF0  16B002 SEND

1B655  056002 TANH         1E6C1  0DF002 PARMTRC       21F24  16C002 KGET

1B6A4  057002 ASIN         1E6E1  0E0002 TRUTH         21F62  16D002 RECN

1B72F  058002 ACOS         1E701  0E1002 SCATTER       21F96  16E002 RECV

1B79C  059002 ATAN         1E721  0E2002 HISTGRM       21FB6  16F002 FINISH

1B7EB  05A002 ASINH        1E741  0E3002 BAR           21FD1  170002 SERVER

1B830  05B002 ACOSH        1E761  0E4002 SAME          21FEC  171002 CKSM

1B8A2  05C002 ATANH        1E783  0E5002 AND           2200C  172002 BAUD

1B905  05D002 EXP          1E809  0E6002 OR            2202C  173002 PARITY

1B94F  05E002 LN           1E88F  0E7002 NOT           2204C  174002 TRANSIO

1B9C6  05F002 LOG          1E8F6  0E8002 XOR           2206C  175002 KERRM

1BA3D  060002 ALOG         1E972  0E9002 ==            22087  176002 BUFLEN

1BA8C  061002 LNP1         1EA9D  0EA002 !=            220A2  177002 STIME

1BAC2  062002 EXPM         1EBBE  0EB002 <             220C2  178002 SBRK

1BB02  063002 !            1EC5D  0EC002 >             220DD  179002 PKT

1BB41  064002 FACT         1ECFC  0ED002 <=            224CA  17A002 INPUT

1BB6D  065002 IP           1ED9B  0EE002 >=            224F4  17B002 ASN

1BBA3  066002 FP           1EE38  0EF002 OLDPRT        22514  17C002 STOKEYS

1BBD9  067002 FLOOR        1EE53  0F0002 PR1           22548  17D002 DELKEYS

1BC0F  068002 CEIL         1EE6E  0F1002 PRSTC         22586  17E002 RCLKEYS

1BC45  069002 XPON         1EE89  0F2002 PRST          225BE  17F002 ->TAG

1BC71  06A002 MAX          1EEA4  0F3002 CR            22633  180002 DTAG

1BCE3  06B002 MIN          1EEBF  0F4002 PRVAR         22EC3  000700 IF

1BD55  06C002 RND          1EF43  0F5002 DELAY         22EFA  001700 THEN

1BDD1  06D002 TRNC         1EF63  0F6002 PRLCD         22FB5  002700 ELSE

1BE4D  06E002 MOD          1EF7E  0F7002 delta         22FD5  003700 END

1BE9C  06F002 MANT         1EFCC  0F8002 delta         22FEB  004700 ->

1BEC8  070002 D->R         1F133  0F9002 RCEQ          23033  005700 WHILE

1BEF4  071002 R->D         1F14E  0FA002 STEQ          2305D  006700 REPEAT

1BF1E  072002 ->HMS        1F16E  0FB002 ROOT          230C3  007700 DO

1BF3E  073002 HMS->        1F1D4  0FC002 integral      230ED  008700 UNTIL

1BF5E  074002 HMS+         1F21D  0FD002 integral      23103  009700 START

1BF7E  075002 HMS-         1F2C9  0FE002 sigma         231A0  00A700 FOR

1BF9E  076002 RNRM         1F354  0FF002 |             2324C  00B700 NEXT

1BFBE  077002 CNRM         1F3ED  100002 |             23380  00C700 STEP

1BFDE  078002 DET          1F500  101002 QUOTE         233DF  00D700 IFERR

1BFFE  079002 DOT          1F55D  102002 APPLY         23472  00E700 HALT

1C007  07D002 %T           1F9C4  107002 ->Q           2349C  00F700

1C01E  07A002 CROSS        1F9E9  108002 ->Qpi         234C1  010700 ->

1C03E  07B002 RSD          1FA59  109002 ^MATCH        235FE  011700 >>

1C060  07C002 %            1FA8D  10A002 vMATCH        2361E  012700 <<

1C149  07E002 %CH          1FAEB  10B002 _             23639  013700 >>

1C1B9  07F002 RAND         1FB5D  10C002 RATIO         23654  014700 '

1C1D4  080002 RDZ          1FB87  10D002 DUP           23679  015700 '

1C1F6  081002 COMB         1FBA2  10E002 DUP2          23694  016700 END

1C236  082002 PERM         1FBBD  10F002 SWAP          236B9  017700 END

1C274  083002 SF           1FBD8  110002 DROP          2371F  018700 THEN

1C2D5  084002 CF           1FBF3  111002 DROP2         2378D  019700 CASE

1C313  085002 FS?          1FC0E  112002 ROT           237A8  01A700 THEN

1C360  086002 FC?          1FC29  113002 OVER          23813  01B700 DIR

1C399  087002 DEG          1FC44  114002 DEPTH         23824  01C700 PROMPT

1C3B4  088002 RAD          1FC64  115002 DROPN

Alonzo Gariepy

alonzo@microsoft