Saturn Instruction Set				Derek S. Nickel

================================================================

0...	* misc operations (RTNs, modes, stack, status, P, logic)

1...	* data movement/loading (D0,D1,R0-R4)

2n	P=     n

3xn..n	LC(m)  n..n	(x = m - 1)

3mc..c	LCASC  \A..A\	(m = 2*characters - 1)

3nh..h	LCHEX  h..h	(n = digits - 1)

400	RTNC

4aa	GOC    label

420	NOP3

500	RTNNC

5aa	GONC   label

6aaa	GOTO   label

6300	NOP4

64000	NOP5

7aaa	GOSUB  label

8...	* misc, tests and branching

9...	* tests w/fs

Aax	* math and assignment w/fs

Abx	* math and assignment w/fs

Bax	* math and assignment w/fs

Bbx	* math and assignment w/fs

Cx	* math and assignment w/A

Dx	* math and assignment w/A

Ex	* math and assignment w/A

Fx	* math and assignment w/A

0

================================================================

00	RTNSXM

01	RTN

02	RTNSC

03	RTNCC

04	SETHEX

05	SETDEC

06	RSTK=C

07	C=RSTK

08	CLRST

09	C=ST

0A	ST=C

0B	CSTEX

0C	P=P+1

0D	P=P-1

0Exx	* logic operations

0F	RTI

-- field ---

fs	 A

------------

0Ea0	0EF0	A=A&B  field

0Ea1	0EF1	B=B&C  field

0Ea2	0EF2	C=C&A  field

0Ea3	0EF3	D=D&C  field

0Ea4	0EF4	B=B&A  field

0Ea5	0EF5	C=C&B  field

0Ea6	0EF6	A=A&C  field

0Ea7	0EF7	C=C&D  field

0Ea8	0EF8	A=A!B  field

0Ea9	0EF9	B=B!C  field

0EaA	0EFA	C=C!A  field

0EaB	0EFB	D=D!C  field

0EaC	0EFC	B=B!A  field

0EaD	0EFD	C=C!B  field

0EaE	0EFE	A=A!C  field

0EaF	0EFF	C=C!D  field

fs: P  WP XS X  S  M  B  W

a:  0  1  2  3  4  5  6  7

10, 11, 12

================================================================

100	R0=A

101	R1=A

102	R2=A

103	R3=A

104	R4=A

105

106

107

108	R0=C

109	R1=C

10A	R2=C

10B	R3=C

10C	R4=C

10D

10E

10F

110	A=R0

111	A=R1

112	A=R2

113	A=R3

114	A=R4

115

116

117

118	C=R0

119	C=R1

11A	C=R2

11B	C=R3

11C	C=R4

11D

11E

11F

120	AR0EX

121	AR1EX

122	AR2EX

123	AR3EX

124	AR4EX

125

126

127

128	CR0EX

129	CR1EX

12A	CR2EX

12B	CR3EX

12C	CR4EX

12D

12E

12F

13 through 1F

================================================================

130	D0=A

131	D1=A

132	AD0EX

133	AD1EX

134	D0=C

135	D1=C

136	CD0EX

137	CD1EX

138	D0=AS

139	D1=AS

13A	AD0XS

13B	AD1XS

13C	D0=CS

13D	D1=CS

13E	CD0XS

13F	CD1XS

---------- field -----------

A	 B	 fs	 d

----------------------------

140	148	150a	158x	DAT0=A field

141	149	151a	159x	DAT1=A field

142	14A	152a	15Ax	A=DAT0 field

143	14B	153a	15Bx	A=DAT1 field

144	14C	154a	15Cx	DAT0=C field

145	14D	155a	15Dx	DAT1=C field

146	14E	156a	15Ex	C=DAT0 field

147	14F	157a	15Fx	C=DAT1 field

16x	D0=D0+ n

17x	D1=D1+ n

18x	D0=D0- n

19nn	D0=(2) nn

19hh	D0=HEX hh

1Annnn	D0=(4) nnnn

1Ahhhh	D0=HEX hhhh

1Bnnnnn	D0=(5) nnnnn

1Bhhhhh	D0=HEX hhhhh

1Cx	D1=D1- n

1Dnn	D1=(2) nn

1Dhh	D1=HEX hh

1Ennnn	D1=(4) nnnn

1Ehhhh	D1=HEX hhhh

1Fnnnnn	D1=(5) nnnnn

1Fhhhhh	D1=HEX hhhhh

fs: P  WP XS X  S  M  B  W

a:  0  1  2  3  4  5  6  7

x = d - 1		x = n - 1

d = x + 1		n = x + 1

80, 81

================================================================

800	OUT=CS

801	OUT=C

802	A=IN

803	C=IN

804	UNCNFG

805	CONFIG

806	C=ID

807	SHUTDN

8080	INTON

808C	PC=(A)

808F	INTOFF

809	C+P+1

80A	RESET

80B	BUSCC

80Cn	C=P    n

80Dn	P=C    n

80E	SREQ?

80Fn	CPEX   n

810	ASLC

811	BSLC

812	CSLC

813	DSLC

814	ASRC

815	BSRC

816	CSRC

817	DSRC

818

819

81A

81B

81C	ASRB

81D	BSRB

81E	CSRB

81F	DSRB

82 through 8F, 9

================================================================

82n	no opcode, clear hardware status w/n as bit mask

821	XM=0

822	SB=0

824	SR=0

828	MP=0

82F	CLRHST

831yy	?XM=0

832yy	?SB=0

834yy	?SR=0

838yy	?MP=0

84n	ST=0   n

85n	ST=1   n

86nyy	?ST=0  n	?ST#1  n

87nyy	?ST=1  n	?ST#0  n

88nyy	?P#    n

89nyy	?P=    n

8Axyy	* tests w/A (equal and not equal)

8Bxyy	* tests w/A (inequality)

8Caaaa	GOLONG label

8Daaaaa	GOVLNG label

8Eaaaa	GOSUBL label

8Faaaaa	GOSBVL label

9zxyy	* tests w/fs

TEST00	RTNYES

TESTyy	GOYES  label

8A, 8B, 9

================================================================

--- field ----

A      fs

--------------

8A0yy	9a0yy	?A=B   field		?B=A   field

8A1yy	9a1yy	?B=C   field		?C=B   field

8A2yy	9a2yy	?A=C   field		?C=A   field

8A3yy	9a3yy	?C=D   field		?D=C   field

8A4yy	9a4yy	?A#B   field		?B#A   field

8A5yy	9a5yy	?B#C   field		?C#B   field

8A6yy	9a6yy	?A#C   field		?C#A   field

8A7yy	9a7yy	?C#D   field		?D#C   field

8A8yy	9a8yy	?A=0   field

8A9yy	9a9yy	?B=0   field

8AAyy	9aAyy	?C=0   field

8AByy	9aByy	?D=0   field

8ACyy	9aCyy	?A#0   field

8ADyy	9aDyy	?B#0   field

8AEyy	9aEyy	?C#0   field

8AFyy	9aFyy	?D#0   field

8B0yy	9b0yy	?A>B   field

8B1yy	9b1yy	?B>C   field

8B2yy	9b2yy	?C>A   field

8B3yy	9b3yy	?D>C   field

8B4yy	9b4yy	?A<B   field

8B5yy	9b5yy	?B<C   field

8B6yy	9b6yy	?C<A   field

8B7yy	9b7yy	?D<C   field

8B8yy	9b8yy	?A>=B  field

8B9yy	9b9yy	?B>=C  field

8BAyy	9bAyy	?C>=A  field

8BByy	9bByy	?D>=C  field

8BCyy	9bCyy	?A<=B  field

8BDyy	9bDyy	?B<=C  field

8BEyy	9bEyy	?C<=A  field

8BFyy	9bFyy	?D<=C  field

fs: P  WP XS X  S  M  B  W

a:  0  1  2  3  4  5  6  7

b:  8  9  A  B  C  D  E  F

A, C, D

================================================================

-- field --

A	fs

-----------

C0	Aa0	A=A+B  field

C1	Aa1	B=B+C  field

C2	Aa2	C=C+A  field

C3	Aa3	D=D+C  field

C4	Aa4	A=A+A  field

C5	Aa5	B=B+B  field

C6	Aa6	C=C+C  field

C7	Aa7	D=D+D  field

C8	Aa8	B=B+A  field

C9	Aa9	C=C+B  field

CA	AaA	A=A+C  field

CB	AaB	C=C+D  field

CC	AaC	A=A-1  field

CD	AaD	B=B-1  field

CE	AaE	C=C-1  field

CF	AaF	D=D-1  field

D0	Ab0	A=0    field

D1	Ab1	B=0    field

D2	Ab2	C=0    field

D3	Ab3	D=0    field

D4	Ab4	A=B    field

D5	Ab5	B=C    field

D6	Ab6	C=A    field

D7	Ab7	D=C    field

D8	Ab8	B=A    field

D9	Ab9	C=B    field

DA	AbA	A=C    field

DB	AbB	C=D    field

DC	AbC	ABEX   field

DD	AbD	BCEX   field

DE	AbE	ACEX   field

DF	AbF	CDEX   field

fs: P  WP XS X  S  M  B  W

a:  0  1  2  3  4  5  6  7

b:  8  9  A  B  C  D  E  F

B, E, F

================================================================

-- field ---

A	fs

------------

E0	Ba0	A=A+B  field

E1	Ba1	B=B+C  field

E2	Ba2	C=C+A  field

E3	Ba3	D=D+C  field

E4	Ba4	A=A+1  field

E5	Ba5	B=B+1  field

E6	Ba6	C=C+1  field

E7	Ba7	D=D+1  field

E8	Ba8	B=B-A  field

E9	Ba9	C=C-B  field

EA	BaA	A=A-C  field

EB	BaB	C=C-D  field

EC	BaC	A=B-A  field

ED	BaD	B=C-B  field

EE	BaE	C=A-C  field

EF	BaF	D=C-D  field

F0	Bb0	ASL    field

F1	Bb1	BSL    field

F2	Bb2	CSL    field

F3	Bb3	DSL    field

F4	Bb4	ASR    field

F5	Bb5	BSR    field

F6	Bb6	CSR    field

F7	Bb7	DSR    field

F8	Bb8	A=-A   field

F9	Bb9	B=-B   field

FA	BbA	C=-C   field

FB	BbB	D=-D   field

FC	BbC	A=-A-1 field

FD	BbD	B=-B-1 field

FE	BbE	C=-C-1 field

FF	BbF	D=-D-1 field

fs: P  WP XS X  S  M  B  W

a:  0  1  2  3  4  5  6  7

b:  8  9  A  B  C  D  E  F

Data Storage Allocation

================================================================

00..0	BSS    expr

nn..n	CON(m) expr

nn..n	REL(m) expr

zy..a	NIBASC 'chars'

zy..a	NIBASC \chars\

hh..h	NIBHEX h..hh

Saturn Registers

================================================================

+--------------------------------+

|                                |  A

+--------------------------------+

|                                |  B

+--------------------------------+

|                                |  C

+--------------------------------+

|                                |  D

+--------------------------------+

+----------+

|          |  D0

+----------+

|          |  D1

+----------+

+--+

|  |  P

+--+

+----------+

|          |  PC

+----------+

+--------------------------------+

|                                |  R0

+--------------------------------+

|                                |  R1

+--------------------------------+

|                                |  R2

+--------------------------------+

|                                |  R3

+--------------------------------+

|                                |  R4

+--------------------------------+

+--------+

|        |  IN (16 bits)

+--------+

|      |  OUT (12 bits)

+------+

+-+

| |  Carry (1 bit)

+-+

+--------+

| F .. 0 |  ST (16 bits)

+--------+

+-------------+

| MP SB SR XM |  HST (4 bits)

+-------------+