(Comp.sys.handhelds)

Item: 3767 by ftg0673 at tamsun.TAMU.EDU

Author: [Rick Grevelle]

Subj: HP48 Conference Preview

Date: Thu Jul 25 1991

This is one of the topics I plan to discuss in detail at the up and coming

Handhelds Conference in Corvallis.  Since there are those who will not wish

to wait until then to take advantage of this material, it is appropriately

complete to now provide a sneak preview.  However the finer details will be

reserved for the conference itself.

Rick Grevelle

ftg0673@tamsun.tamu.edu

(409) 774-1169

===========================================================================

HP48 Hard Key Assignment Interrupts

Normally each time one of the HP48's forty nine keys is pressed two system

binaries are pushed to the stack by the underlying operating system.  These

are never seen by the user who is probably unaware that his or her calculator

is using the very same stack for the SYSTEM RPL as is used by the USER RPL.

Once a key is pressed and the two system binaries are pushed to the stack, it

is then up to the SYSTEM RPL to make a decision base on the value of these

results as to what to do next (i.e. change menus, execute functions, initiate

the command line, etc.).

Interrupting the SYSTEM RPL responsible for interpreting and executing the

actual key stroke is easily done.  It is then possible to substitute a user

scheme to effectively handle the task of key press management, but this is no

small job, and the process can become quite lengthy.  All details regarding

the interrupt mechanism and the SYSTEM RPL management of key presses will not

be addresses here as I'm saving it for the August conference.  What will be

covered here is involved enough for now, and should be more than adequate for

the majority of users.  In other words, I'm providing all the support tools

necessary for interrupting the 48's hard key assignments.  Keeping it in mind

that this has nothing to do with the user key assignments as I have gone to

great lengths to maintain the integrity of this feature that actually coexists

with the regular key assignments.

The two directories at the end of this article are suitable for converting

to library format.  The first will be primarily of interest to those concerned

with hard addressing key assignments in software intended for ROM cards, and

the second for those who wish to perform interrupts from RAM.  Although I have

tried to stick to entry points supported by Hewlett-Packard, it wasn't always

possible to do so. I apologize for this, especially to those who haven't spent

the majority of their spare time for the past two years peeking at the hex in

the HP28/48.  So with all that said, let's get started.

o  Hard key assignments in the 48 are found in the hidden ROM

and packaged in an Array of System Binary data object that

contains forty nine System Binary elements.  Each of these

elements is in fact the address of another Array of System

Binary within the hidden ROM containing six elements which

are the addresses of the various key configurations that a

particular key is assigned.  Each element corresponds to a

specific key-shift pattern as follows:

1st element:    unshifted

2nd element:    left-shifted

3rd element:    right-shifted

4th element:    alpha-shifted

5th element:    alpha-left-shifted

6th element:    alpha-right-shifted

o  The key assignment array in an HP48 version E is located at

# 7B4E4h in the covered ROM.  While it is not necessary for

this to be done, the Array of System Binary can be recalled

by using \<< #3FB1Fh SYSEVAL #C612h SYSEVAL \>>.  Implement

OUT\-> to decompose the array into its individual elements.

The first element in this array of forty nine elements is a

System Binary that is the address of an array of six System

Binaries that are the addresses of each of the six specific

key-shift patterns for the upper most left-hand key.  So to

continue associating each of the forty nine elements with a

particular key, simply move from left to right, assigning a

new element for each one of the 48's keys.  This means that

the upper most right-hand key is the sixth element, and the

lower most right-hand is the forty ninth array element.

CORRESPONDING KEYBOARD ARRAY ELEMENTS

[ A ]    [ B ]    [ C ]    [ D ]    [ E ]    [ F ]

<7AA5Dh> <7AA94h> <7AACBh> <7AB02h> <7AB39h> <7AB70h>

[MTH]    [PRG]    [CST]    [VAR]    [ ^ ]    [NXT]

<7ABA7h> <7ABDEh> <7AC15h> <7AC4Ch> <7AC83h> <7ACBAh>

[ ' ]    [STO]   [EVAL]    [ < ]    [ v ]    [ > ]

<7ACF1h> <7AD28h> <7AD5Fh> <7AD96h> <7ADCDh> <7AE04h>

[SIN]    [COS]    [TAN]    [SQx]    [pwr]    [1/x]

<7AE3Bh> <7AE72h> <7AEA9h> <7AEE0h> <7AF17h> <7AF4Eh>

[ENTER]      [+/-]      [EEX]      [DEL]      [<==]

<7AF85h>    <7AFBCh>   <7AFF3h>   <7B02Ah>   <7B061h>

[alpha]      [ 7 ]      [ 8 ]      [ 9 ]      [ / ]

<7B098h>    <7B0CFh>   <7B106h>   <7B13Dh>   <7B174h>

[left]       [ 4 ]      [ 5 ]      [ 6 ]      [ * ]

<7B1ABh>    <7B1E2h>   <7B219h>   <7B250h>   <7B287h>

[right]      [ 1 ]      [ 2 ]      [ 3 ]      [ - ]

<7B2BEh>    <7B2F5h>   <7B32Ch>   <7B363h>   <7B39Ah>

[ ON ]       [ 0 ]      [ . ]      [SPC]      [ + ]

<7B3D1h>    <7B408h>   <7B43Fh>   <7B476h>   <7B4ADh>

o  Key reassignments are made by replacing array elements that

correspond to the appropriate keys.  Utilizing the provided

shareware, this is accomplished through the modification of

the Array of System Binary stored in the KEYS variable from

directory number one, or the Array of Global Name stored in

the KEYS variable, but in directory number two instead.  In

addition directory two contains forty nine Global variables

sequentially named beginning with K.1 through K.49.  Stored

in each of these is an Array of System Binary corresponding

to the key number from which individual variable names were

derived.  Therefore, the Global name K.1 contains the Array

of System Binary recovered from the hidden ROM at # 7AA5Dh.

o  The Array of System Binary stored in the KEYS variable from

directory one is the cache for the default key assignments.

This is because it's easiest to begin customizing hard keys

by altering the default assignments rather than building an

array from scratch.  The scheme used by the first directory

to override the default key assignments is quite similar to

that of the one found in the 48.  An Array of System Binary

is the only type of array that is allowed, and each element

must be an address in memory where a second Array of System

Binary is located whose elements are the addresses of entry

points for the various key configurations associated with a

specific key.  This requires knowing the location in memory

where the arrays are stored and that once these assignments

are made they cannot be moved.  Clearly this application is

better suited for software that is hard addressed.  However

there are applications for RAM oriented software exploiting

this scheme.  For example it might be desirable to reassign

the hard keys differently from the factory's.  For instance

it would be more convenient for the top row of menu keys to

be closer to the numerical keys for someone who was working

with hexadecimal integers.  A third directory included here

is identical to the first but with one exception; the Array

of System Binary has had its elements rearranged so the top

row of keys has been exchanged with forth row.  This brings

the alpha keys A through F closer to the numerical keys for

quicker access.  Implementing CUSTOM swaps row one and four

and DEFAULT restores them to their factory assignments.

o  The second directory provides the flexibility necessary for

software intended to be used in RAM.  Mainly this is due to

an embellished CUSTOM function which permits two additional

types of arrays to be used.  These additional types include

the previously mention Array of Global Name, as well as its

library counterpart Array of XLIB Name.  Where the Array of

Global Name stored in the KEYS variable contains all of the

forty nine sequentially named Global variables.  This means

that the routine can now find arrays stored in Global Names

or XLIBs as well as by their memory locations.  Furthermore

the secondary arrays that house the individual key patterns

may also be composed of either Global Names or XLIBs.  This

allows user defined functions to be assigned to keys either

by their Global Names or XLIBs.

o  Reassign a key to functions stored in Global Names or XLIBs

by constructing an array of Global Names or XLIBs using the

->ARR command from HACKIT and then storing it in any one or

more of the forty nine sequentially named global variables.

Once this is done implement the CUSTOM function in order to

interrupt the default key assignments.  Restore the default

key assignments using the DEFAULT function.

Example:  Assign the first menu page of the HACKIT

library to a single key of your choice.

_________________________               _________________________

|{HOME}                 |               |{HOME}                 |

|-----------------------|               |-----------------------|

|3:                     |               |4:                     |

|2:                     |               |3:                     |

|1: { ->ASCI ASCI-> SCRC|               |2:                     |

|   SEEK PEEK POKE   }  |               |1:   Array of XLIB Name|

|### ### ### ### ### ###|               |### ### ### ### ### ###|

-------------------------               -------------------------

1) Place the six commands from          2) Implement OUT-> to explode

the first page of HACKIT in             the list and then ->ARR to

a list then enter it on the             obtain the array.  Storing

on the stack in level one.              it in any K.n assigns that

(Display model is not exact.)            key the six library XLIBs.

o  These directories effectively permit user defined functions

to be assigned to keys without utilization of the USER MODE

which is still available for regular use.  Essentially this

routine doubles the amount of possible user key assignments

previously available.  Extensive measures were taken not to

restrict user applications, meaning the scheme has not been

one hundred percent idiot-proofed.  Two specific areas that

could be potentially dangerous are as follows:

+  Do not store any objects other than arrays

in the sequentially named Global variables

because a decision was made to eval rather

than rcl the elements from the KEYS array.

+  Avoid using subdirectories for this scheme

because moving up to a parent directory or

the HOME directory without first restoring

control to the default could perhaps cause

a memory loss.

One final note; if either of these two directories are to be

converted to libraries it would be advisable to place all of

variables other than CUSTOM and DEFAULT in a list in $HIDDEN

in order to hide them in the library.  This is because these

two variables are the only ones necessary for the control of

the scheme once the key assignments have actually been made.

Note that STDKEYS is an RPL subroutine called by CUSTOM.  Do

not implement it for any reason.  Both uuencode and ASC have

been provided.