*******************

DTERM V1.0 10.01.91 By Glenn G. Bruckno

*******************

THE POOP ON DTERM

-----------------

DTERM is a dumb terminal emulator for the HP48.  I posted it here

because I couldn't find any other terminal emulator which completely

fit my needs.  It was written for a very specific work-related

application; as such, it may need tailoring for your particular use.

Fortunately, it is written without machine code and is easily modified.

DTERM has some distinct advantages over the other terminal emulators

I've seen here on the BBS, including a true 10 line scrolling display.

Its big advantage is the ability to handle huge amounts of incoming

data, even at 9600 baud.  DTERM needs only the briefest of occasional

pauses from the host in order to buffer data.  Provided the HP returns

now and then from performing its own serial housekeeping, the amount

of data that can be buffered is limited only by available memory.

THE POOP ON THE PROGRAMS

------------------------

Download the following binary attachment and you will see the

following appear in your current directory:

TERM

A Directory consisting of the following programs:

Stack action

------------

DTERM     The main program,                      --  |  --

Saves your flags,

creates temporary globals

CRLF: A string "<cr> <lf>"

INLN: An empty string: ""

examines BUFLEN and assembles

serial data INLN

Restores your flags and

purges temporary globals

on exit

DOKEY     Routines for handling                  --  |  0/1

a keypress.  Returns 1

if the DEL key is pressed

which ends the program,

0 otherwise

SCROLL    Scrolls PICT up one line               --  |  --

(6 pixels)

PUTLN     Takes a string from Level         "string" |  --

1 and puts it on the bottom

of PICT

STRIP     Removes characters from                --  |  --

incoming string INLN

up to <cr> <lf>; calls

SCROLL, passes string

to PUTLN

Now for its limitations:  Because DTERM turns incoming ASCII into

graphic objects for its display, each line should not exceed the

width of PICT.  In other words, the line width measured in terms of

characters depends on the pixel width of the characters displayed.

You can fit a whole bunch of i's across the display; not so many w's.

If a line is received which exceeds this, the excess is lopped off

into pixel oblivion.

In addition, DTERM waits for a <cr> <lf> pair to be received before

it displays any preceding characters.  The <cr> <lf> is stripped off;

no other controls are stripped.  If your application is packet radio,

tell your fellow packeteers to send short lines!

You can remove both these limitations by modifying STRIP to simply wait

for an arbitrary number of characters (i.e., examine the SIZE of INLN)

before executing SCROLL and PUTLN.  You will have to decide on a

reasonable columnar display width.

If your host sends only <cr> at the end of a line, modify STRIP thusly:

change PUTLN 2 + to

PUTLN 1 +

Also, remove the 10 CHR + part from DTERM (the linefeed char).

CREDIT WHERE CREDIT IS DUE

--------------------------

Thanks to Mr. Kevin Jessup for the keytable idea from his excellent

program DTE48.  I've hacked it for my purposes, so the following

changes are noted:

1. Default is upper case.

2. No more toggle flag -- if you hit a gold

or blue key once, it can't be undone.

3. Alpha key eliminated -- I don't try to

throttle the host.

THE POOP ON USING DTERM

-----------------------

Set up your serial I/O with the standard HP I/O SETUP menu.  Press

DTERM to run.  Press letter keys to send characters.  Press <left shift>

to change the next character to lower case.  Press <right shift> to

change the next character to a Control character: A becomes ^A etc.

Press ENTER to send a <cr>; <left shift> ENTER sends a <lf>. Unassigned

keys shoot blanks. Press DEL to quit.

You can test DTERM in standalone mode by looping the serial connector

back to itself: jump the middle two pins on the serial connector.

Remember to set wire mode in the I/O SETUP menu.

Because the SCROLL and PUTLN routines juggle graphic objects, they

take a lot of time to execute.  Therefore DTERM assigns high priority

to the incoming data.  SRECV is performed any time BUFLEN is nonzero.

Only when BUFLEN is zero, meaning no characters are staged for reception,

do SCROLL and PUTLN perform.  Even then, only one line is manipulated

before BUFLEN is again examined.  Rest assured that your characters

are being buffered even though no data may appear on the display for

a while.  When the host takes a breather, the data will scroll onto

the display slowly enough to read.

I have reproduced here Mr. Kevin Jessup's translation table, modified:

KEY        UNSHIFTED RESULT   LEFT SHIFTED RESULT   RIGHT SHIFTED RESULT

--------   ----------------   -------------------   --------------------

A          A                  a                     ^A (ASCII 1)

B          B                  b                     ^B (ASCII 2)

C          C                  c                     ^C (ASCII 3)...

Z          Z                  z                     ^Z (ASCII 26)

1          1                  !

2          2                  @

3          3                  #

4          4                  $

5          5                  %

6          6                  ^

7          7                  &

8          8                  *

9          9                  (

0          0                  )

/          /                  ~

*          *                  ,

-          -                  _    (underscore)

+          +                  =

.          .                  <

SPC        space              >

<-         backspace          delete

ENTER      carriage return    linefeed

DEL        QUIT               QUIT

OTHER POOP

----------

I have posted this program here so as to provide a tool for other

HP48 bitheads.  Examine the code, change it, improve it, have a

blast.  In exchange, I ask you to likewise upload your interesting

stuff for the overall good of mankind.  However, please give credit

where credit is due, and if you find a way to make money with my code,

my ugly lawyers and I will demand some fat royalties.

Send comments, questions, etc to me:

here under the name:            ggbruckno

or via packet radio:            KB2HBW @ W2EMU-4.NJ.USA.NA

#############