Author: hpfcso.FC.HP.COM [Ray Depew]

Date: Thu Dec 19 1991

In comp.sys.hp48, robf@mcs213g.cs.umr.edu (Rob Fugina) writes:

> I just bought the serial interface kit, and I can't get anything to work!

> I set the right port, speed, and parity on the computer, and I set the

> 48 to wire, ascii, and the same baud and parity as the computer.  Nothing

> seems to happen!  The only thing that I can get to happen (sometimes) is

> when the computer is in terminal mode and I put the 48 in server mode,

> the computer will receive 4-6 characters (that I recognize as the 48

> telling the computer it's in server mode).  Is there any way to test the

> connection?  Does the computer's com port need to be set for DCE or

> something???  HELP!!!!!!!!!!

>

> Rob   robf@cs.umr.edu

1) To test the connection without Kermit getting in the way, type "connect"

at the 'Kermit >' prompt on your computer, or use a dumb terminal.

a) To test the computer->48 connection:

- press OPENIO on the 48.

- type HELLO on the computer and press RETURN/ENTER/whatever.

- press BUFLEN on the 48 (last pg. of the I/O menu).

- you should see two numbers on the stack.

Level 1 is a flag:  1 := data transferred o.k.

2 := framing error or UART overrun

Level 2 is the number of characters received from the computer.

- press DROP to remove the flag and put the # of char's into Level 1.

- press SRECV on the 48 (last pg. of the I/O menu).

- you should see two numbers on the stack.

Level 1 is a flag:  1 := data transferred o.k.

2 := unsuccessful receive.  (press ERRM for

more detail)

Level 2 should be the string received from the computer.

b) To test the 48->computer connection:

- (Assuming you already pressed OPENIO)

- put a string like "HI BACK" on Level 1 of the 48.

- press XMIT on the 48 (last pg. of the I/O menu).

- if data transfer is successful, you will see a 1 on Level 1.

- if part of the string failed to transmit, you will see a 0 on

Level 1 and the unsent portion of the string on Level 2.

2) Remember to press CLOSEIO when you've finished.

Regards

Ray Depew

Hewlett Packard Co, Fort Collins, Colorado

rrd@hpfitst1.fc.hp.com