From: bson@rice-chex.ai.mit.edu (Jan Brittenson)

Newsgroups: comp.sys.hp48

Subject: Shadowing Memory

Date: 9 Sep 91 08:24:38 GMT

Here are two small utilities, that let you map around your cards.

[Note: Both SHADOW and ZEROMAP are contained in the SHADOW directory

on this disk.  -jkh-]

SHADOW maps a card to shadow another device. The contents of the

device are first copied to the card so you can modify them. Of course,

if your card is ROM (or write-protected RAM), then the copy is ignored

and the contents of the card will effectively overlay those of the

second device. It takes two arguments: the address of your card and

the destination address.

SHADOW		Copy and map card to shadow another device

#card_address #destination_address  -->

ZEROMAP is similar to SHADOW, except the contents of the card are

cleared instead of copied.

Some examples:

#80000 #10000 SHADOW

Shadow parts of the system ROM (#10000-#1ffff) with a copy

in the card in port 1. This would allow full MLDB

breakpointing in the area 10000-1ffff. It could also be

used from a program that subsequently modifies the "ROM."

#10000 #80000 ZEROMAP

Restore from previous example. If we didn't clear the card

we would lose parts of it, and the next time we yanked it

and plugged it back in, we would get an "Invalid card data"

error. This has to do with the sort of trickery SHADOW

resorts to in order to avoid "Invalid card data" errors.

#C0000 #C0000 ZEROMAP

Clear card in port 2.

About ports and such:

A single plug-in card will always be in port 1, even if plugged

into connector 2. Port 1 is at #80000 and port 2 at #C0000. There is

also a third config table entry which I'm not sure what it's used for,

maybe a debug ROM. The default address for the third "port" is #D0000.

The system ROM is in the region #00000-#6FFFF.

Notice that I will not be responsible for any data or other forms

of lossage. If properly used, you should be able to work miracles.

Improperly used, you better have you backups readily available! Also,

using these routines assumes a certain knowledge of HP-48SX internals.

Do not attempt to store data on a card remapped by means of the

routines included here: that is almost guaranteed lossage.

The only argument checking performed is simple type checking.

Have fun,

-- Jan Brittenson

bson@ai.mit.edu

O  /

\/

/\  ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

O  \

; shadow.star - shadow device with port

;

; Jan Brittenson, September 1991

; This program is in the public domain.

; Assemble with (in order) hp48.star and stdsyms.star

radix	0x10

header

; SHADOW

;

; In:

;	2: old address (short)

;	1: new address (short)

conf_tbl = 70428			; System configuration table

rpl

Prg

Need_2_args

Switch_on_argtype

_Short 0xbb			; Two binary integers

Prg

Two_bins_to_shorts	; Convert addresses to shorts

endrpl

code

; Pop address, find fake pointer (all pages have one)

call	getshort		; A.A = shadow address

move	a, r2

call	getshort		; A.A = RAM address

swap	a, r2			; R2.A = RAM address

call	save_regs		; Save system RPL registers

move	3, p

clr.wp	a			; Clear low 4 nybbles of address

clr	p

move	a, r0			; R0.A = shadow location

move	a, d0

dec	d0

$1:

inc	d0

move.a	@d0, c

brnz.a	c, $1

swap	c, d0			; C.A = fake config address

move	c, r1			; R1.A = fake config address

; Copy ROM and remap RAM

clrb	0xf, st			; Disable interrupts

move	r2, c			; C.A = RAM adress

move	c, d1

move.p5	10000, c		; 32kB

move	a, d0			; D0 = shadow page address

call	blkcopy			; Copy 32kB from #10000->#80000

move	r2, c			; C.A = RAM adress

uncnfg				; Unconfig RAM card

move.p5	0xf0000, c		; 32kB Mask

config

move	r0, c			; New location

config

; Update system config table

move	r1, c			; Value to stick in config table

move.5	conf_tbl+^d10, d0	; D0 = & P1 config address

move.a	c, @d0			; Update config address

call	calc_conf_crc		; Calculate config table CRC

; D0 = & CRC, C.A = new CRC

move.4	c, @d0			; CRC = new CRC

; Finished

setb	0xf, st			; Enable interrupts

brbc	0xe, st, $100		; No interrupts occured

rsi				; Service pending interrupts

$100:

jump	rr_rplcont		; Restore RPL registers and continue

even

endcode

rpl

EndObj

EndObj

endrpl

O  /

\/

/\  ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

O  \

; zeromap.star - remap and clear port

;

; Jan Brittenson, Septembet 1991

; This program is in the public domain.

; Assemble with (in order): hp48.star and stdssyms.star

radix	0x10

header

; ZEROMAP

;

; In:

;	2: old address (short)

;	1: new address (short)

conf_tbl = 70428			; System configuration table

rpl

Prg

Need_2_args

Switch_on_argtype

_Short 0xbb			; Two binary integers

Prg

Two_bins_to_shorts	; Convert addresses to shorts

endrpl

code

; Pop address, find fake pointer (all pages have one)

call	getshort		; A.A = shadow address

move	a, r2

call	getshort		; A.A = RAM address

swap	a, r2			; R2.A = RAM address

call	save_regs		; Save system RPL registers

move	3, p

clr.wp	a			; Clear low 4 nybbles of address

clr	p

move	a, r0			; R0.A = shadow location

; Clear remap RAM

clrb	0xf, st			; Disable interrupts

move	r2, c			; C.A = RAM adress

move	c, d1			; D0 = RAM address

move.p5	10000, c		; 32kB

call	blkzero			; Clear RAM

move	r2, c			; C.A = RAM adress

uncnfg				; Unconfig RAM card

move.p5	0xf0000, c		; 32kB Mask

config

move	r0, c			; New location

config

; Update system config table

move.5	conf_tbl+^d10, d0	; D0 = & P1 config address

move.a	c, @d0			; Update config address

call	calc_conf_crc		; Calculate config table CRC

; D0 = & CRC, C.A = new CRC

move.4	c, @d0			; CRC = new CRC

; Finished

setb	0xf, st			; Enable interrupts

brbc	0xe, st, $100		; No interrupts occured

rsi				; Service pending interrupts

$100:

jump	rr_rplcont		; Restore RPL registers and continue

even

endcode

rpl

EndObj

EndObj

endrpl

O  /

\/

/\  ~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

O  \

;; stdsyms.star - some common symbols

nlistblock

radix	0x10

Not_bool = 3af2

Doerr_string = 15048

Text = 1314d

Toggle_text = 132c5

Cmd_DOERR = 1a339

Cmd_EVAL = 1a3be

Cmd_PATH = 1a125

Cmd_HOME = 1a140

Home = 8d92

Path = 1848c

Purge_global = 1854f

Sto_any_global = 18513

Rcl_global = 797b

Cmd_STO = 20ccd

GROB_null = 505b2

OpenIO = 2eb37

CloseIO = 315c6

Freeze = 142fb

Cllcd = 5046a

Eval = 18eba

Eval_prev_context = 6fd1

Push_next = 6e97

Char_000 = 6541e

Add_string_char = 52ee

Add5 = 6258a

Inc = 3def

Dup_Inc = 628eb

Short_200h = 64f0e

Short_100h = 64e32

Short_60h = 64c8e

Short_c0h = 64dd8

Short_bbh = 64dce

Short_31h = 64b44

String_to_grob_pitch8 = 11d00

Turn_menu_off = 4e2cf

Nullstring = 55df

Nullstring_copy = 1613f

Program_null = 40788

Realloc = 61c1c

Copy = 6657

Num = 1410f

Add = 3dbc

Add_string_string = 5193

LeftLess1_string_short = 63259

ArrayTo_array = 1d0ab

ArrayTo = 1d0ab

Dec = 3e0e

Dup = 3188

EndObj = 312b

Equalp = 3d19

Nonzerop = 3cc7

No_args = 18a15

Zerop = 3ca6

Dup_Zerop = 62266

Get_2Lastlocal = 613e7

Get_3lastlocal = 6140e

Get_4lastlocal = 61438

Get_lastlocal = 613b6

If_then_else = 61ad8

If_true_then = 619bc

Dup_If_zero_then_exec = 618a8

If_User_flag_then_else = 63ed9

Pview_short_short = 4f052

Listob = 2a74

ListAlgPrgTo = 54af

ListTo = 54af

ListTo_StoreLocals = 74d0

Local_M = 0xe4c1

Local_N = 0xe4ae

Local_S = 41bea

Mul = 3ec2

Mul2 = 3e6f

Mul_array_real = 362dc

Next = 7334

Prg = 2d9d

True = 3a81

False = 3ac0

Bin_to_short = 5a03

Two_Bins_to_shorts = 4f3d1

Real_to_short = 18cea

Real_to_short_Swap = 62e7b

Swap_Over = 61380

Roll = 3325

Drop = 3244

Swap_Drop = 60f9b

Drop2 = 3258

Drop4 = 60f7e

Xlib_to_Rpl = 7e99

Error_Bad_arg_type = 18cb2

Error_Undef_Name = 18c92

Short_0bh = 405d

Clear_lastargs = 4d33

Reset_lastargs = 53842

Need_1_arg = 18ab2

Need_2_args = 18a8d

Need_4_args = 18b9f

SL_Need_1_arg = 18aa5

Switch_on_argtype = 18fb2

SL_Switch_2_argtypes = 18edf

SL_Switch_on_argtype = 18ece

SL_Switch_4_argtypes = 18f01

SaveLast_Need_1_arg = 18aa5

Lastkey = 4714

Short_0 = 3fef

Short_1 = 3ff9

Short_2 = 4003

Short_3 = 400d

Short_4 = 4017

Short_5 = 4021

Short_6 = 402b

Short_8 = 403f

Short_0dh = 4071

Short_0eh = 407b

Short_0fh = 4085

Short_10h = 408f

Short_16h = 40cb

Short_1fh = 4125

Short_20h = 412f

Short_21h = 4139

Short_40h = 64bda

Short_7fh = 1cd16

Short_83h = 64d24

Pdim = 4b323

Equal2p = 6229a

Real_1 = 2a2c9

Real_2 = 2a2de

Real_3 = 2a2f3

Real_4 = 2a308

Real_5 = 2a31d

Real_7 = 2a347

Short_to_real = 18dbf

Start_1_to_N = 73db

Store_2lastlocal = 615f0

Store_3lastlocal = 61600

Store_4lastlocal = 61610

Store_lastlocal = 615e0

Store_local = 61625

Swap = 3223

Array_dims = 35a9

ToArray_real = 1d02c

ToArray_list = 1d040

Size_list = 567b

ToList = 5459

ToList2 = 631b9

TwoShorts_to_reals = 1950b

TwoReals_to_shorts = 194f7

Short_to_bin = 59cc

Swap_Over = 61380

Over = 32c2

Rot = 3295

Rolld3 = 60fac

Rolld4 = 6109e

Div2 = 3e8e

Div = 3ef7

Disp_any_real = 140ab

Disp_pitch8Str_line0 = 1245b

Disp_pitch8Str_line1 = 1246b

Disp_pitch8str_line2 = 1247b

Disp_pitch8str_line3 = 1248b

Disp_pitch8str_line4 = 1249b

Disp_pitch8str_line5 = 124ab

Disp_pitch8str_line6 = 124bb

Disp_pitch8str_line7 = 124cb

Attach_Home_short = 7709

Loop = 71a2

Exit_loop = 71e5

Until = 71c8

Next = 7334

If_ATTN_then_FlushKbd_Kill = 4243e

If_ATTN_then_End = 4245c

Pr1 = 318fe

Error_bad_arg_value = 18ca2

If_not_equal_then_short = 6336c

Get_StkGrob = 12655

Repl_Grob_short = 11679

;; ML routines

getshort = 6641

get2shorts = 3f5d

pushshort_r0r1 = 6529

pushshort_r0_rplret= 18d0a

pushobj_a = 54266

pushobj_a_rplret = 6ec9

strlen_a = 120cc

strlen_d1 = 120cf

stralloc_nibbles = 5b7d

blkcopy = 670c

blkzero = 675c

save_regs = 679b

restore_regs = 67d2

rplcont = 71be

rr_rplcont = 5143

kbflush = 0xd57

indicate_not_busy = 42359

int_mask = 10e

uart_xmit = 116

adjust_parity = 3113d

uart_xmit_rdy_p = 310ca

recv_getc = 31289

calc_conf_crc = 9b73

ATTN_p = 4988

clear_ATTN = 0xd8e

start_of_stack = 7057e

keybuf = 704ea

graph_grob_ptr = 70565

stack_grob_ptr = 70556

Copyright = ch^^d169

Beta = ch^^d223

CR = ch^015

LF = ch^012

-- Jan Brittenson

bson@ai.mit.edu