KERMIT.TXT File

(c)  Hewlett-Packard Company, 1990

OVERVIEW

--------

Kermit file-transfer protocol enables you to transfer files between your HP 48

calculator and a computer.  Kermit is built into the HP 48, and when you also

have Kermit running on your computer, your HP 48 and computer are ready

to "talk to each other."

Kermit protocol was developed at the Columbia University Center for

Computing Activities.  Kermit is available for many systems for a nominal

fee from Columbia and from various user group organizations.

PROCEDURE

---------

You may need to refer to your calculator and computer manuals to help you

follow this procedure.

1.  Connect the 4-pin end of the serial interface cable to your HP 48;

connect the 9-pin end of the cable to the COM1 or COM2 port on your

computer. (This may require the 25-pin adapter.)

2.  Run Kermit on the computer.  (From the prompt for the directory in

which Kermit was installed, type KERMIT and press [ENTER].)

3.  Configure the Kermit port setting to match the port to which the

serial cable was connected in step 1, and configure the Kermit baud

setting to match the computer's setting.  (You can use the Kermit SET

command to do both of these--see the "EXAMPLE" section below.)

-------------------------------------------------------------------------------

NOTE:  The default baud for the HP 48 is 9600.  So, unless you need

to use another setting for your system, you should set your computer

and Kermit to 9600 baud also.

-------------------------------------------------------------------------------

4.  Make sure the HP 48 baud matches the setting from step 3.  Also, make

sure the IR/wire transmission mode is set to "wire."  (Both of these

parameters can be viewed and set using the I/O SETUP menu on the HP 48.)

5.  Follow the instructions in the "HP 48 Owner's Manual" for transferring

data between an HP 48 and a computer.

6.  When you're finished, type Q and press [ENTER] to quit Kermit.

KERMIT HELP

-----------

To get help in Kermit on the computer, type a question mark (?) and press

[ENTER].  Kermit displays a list of commands.  To get help on a specific

command, type the name of the command followed by a space and then a question

mark, and then press [ENTER].  For example, type SET ? and press [ENTER] to

get help on the SET command.

EXAMPLE

-------

This example assumes that you've connected the serial cable both to your

calculator and to the COM1 port in the computer and that you have a

variable in your HP 48 VAR menu named DATA1.  It also assumes that the HP 48

baud and the computer baud are set to 9600 and that you've prepared for a file

transfer as described in the "HP 48 Owner's Manual."

Here are the keystrokes to transfer the file DATA1 from the HP 48 to the

computer:

Computer Keystrokes:    KERMIT [ENTER]          Runs Kermit protocol.

SET PORT 1              Sets Kermit port to COM1.

SET BAUD 9600           Sets Kermit baud to 9600 to

match the calculator and the

computer.

RECEIVE [ENTER]         Prepares the computer

to receive a file.

Calculator Keystrokes: [VAR] ['] {DATA1}        Recalls the variable to be

transferred to the stack.

[left-shift][I/O]        Selects the I/O menu and

{SEND}                   transfers the file to the

computer.  The file DATA1 is

copied into the directory in

which Kermit resides.

Now assume that you quit Kermit and edited the file DATA1 on your computer.

Here are the keystrokes to send it back to the HP 48:

Computer Keystrokes:    KERMIT [ENTER]          Runs Kermit protocol.

SET PORT 1

SET BAUD 9600

SEND DATA1 [ENTER]      Transfers the edited DATA1

to the HP 48, where it shows

up in the VAR menu.  (If the

old DATA1 still exists, the

new DATA1 is given an

extension so the old version

isn't overwritten.)

Calculator Keystrokes:  [left-shift][I/O]       Prepares the HP 48 to receive

{RECV}                  a file.

EXAMPLE 2: CONTROLLING THE HP 48 FROM THE COMPUTER

--------------------------------------------------

The HP 48 can be controlled from the computer by the REMOTE HOST command.

This example assumes that you've connected the serial cable both to your

calculator and to the COM1 port in the computer. Any command sent to the

HP 48 needs to be in upper case (CAPS). The calculator's stack will be

displayed on the computer's display after each REMOTE HOST command.

Here are the keystrokes to control the HP 48 from the computer.

Calculator Keystrokes:

[right shift][I/O]                   Puts the calculator in Server

mode.

Computer Keystrokes:

KERMIT [ENTER]                       Runs Kermit protocol.

SET PORT 1

SET BAUD 9600

REMOTE HOST 'X^3-4*X^2+X-10'         Sends an equation to the stack

of the calculator.

REMOTE HOST 'X'                      Sends the variable X.

REMOTE HOST 10                       Sends the number 10.

REMOTE HOST ROOT                     Sends the ROOT command.

The calculated root of the

equation is returned to the computer's

display (4.30691319972).

To quit, press [ATTN] on the HP 48 and type EXIT or Q (quit) [ENTER] on the

computer.

NOTE: The Kermit DEFINE function will allow you to assign the REMOTE HOST

command to fewer keystrokes.  For example, the computer keystrokes

DEFINE RH REMOTE HOST \%1

assigns the REMOTE HOST command to keystrokes  rh (or RH).

KERMIT HELPFUL HINTS

--------------------

o  The Kermit PUSH command gets you out of Kermit temporarily and into DOS

so that you can manipulate files.  You return to Kermit by typing EXIT

and pressing [ENTER].

o  The Kermit STATUS command displays a screen showing the current Kermit

configuration settings.  The baud and port settings are the first two

in the list.

o  You can execute the DOS directory command from within Kermit by typing

DIR and pressing [ENTER].

o  For additional information on Kermit protocol, a book by Frank da Cruz,

"KERMIT, A File Transfer Protocol" (1987. Bedford, MA: Digital Press) is

available in many bookstores or can be ordered.