X   X Y   Y                 d

X   X Y   Y                 d

X   X Y   Y                 d

X X   Y Y  mm m   ooo   dddd  eee  mm m

------------    X     Y   m m m o   o d   d e   e m m m  -------------

X X    Y   m m m o   o d   d eeeee m m m

X   X   Y   m m m o   o d   d e     m m m

X   X   Y   m   m o   o d   d e     m   m

X   X   Y   m   m  ooo   dddd  eeee m   m

Some words about Xmodem, Ymodem and Ymodem Batch protocols

==========================================================

Some persons had some problems with the latest version of X&YModem

concerning the transmission. For this reason, we include some indications

about the protocol. So, the configuration of your computer will be easier

and the error messages will be less mysterious. (We hope)

The file to transfer is decomposed in blocks. Each block has the

following structure :

-----------------------------//--------------

|   |   |   | <------- Data --------> |   |   |

-----------------------------//--------------

^   ^   ^                             ^   ^

|   |   |                             |   |

|   |    - Blk /Number                 ----- Control

|   |

|   |

|    - Blk Number

|

|

- Blk Header

- Blk Header : (1 byte long)

SOH or STX for respectively 128 and 1024 bytes of data.

^       ^

|       |

|        - Long Packet

|

- Short Packet

- Blk Number : (1 byte long)

The first block has a value of #1. This field is incremented for each

block transfered. When #FF is reached, the value wraps around to #0.

- Blk /Number : (1 byte long)

This field is the negate of the previous. It's usefull to detect the

lost of synchronization between sender and receiver. In such a case,

the transmission is aborted.

If Blk Number has a value of #1 then the Blk /Number has a value of #FE.

- Data : (128 or 1024 bytes long)

See Blk Header.

- Control : (1 or 2 bytes long)

The control is 1 byte long for Cheksum and 2 bytes long for CRC. The

control is set at the begining of the transfer. See below...

Xmodem protocol

---------------

At the begining, the sender is passive; it waits...

The receiver sends a NAK or a char "C" for respectively the Checksum

or the CRC error detection.

If the sender doesn't respond then the receiver waits 10 sec. and sends

again NAK or char "C". After ten retrys, the transmission is aborted.

If the sender has received the NAK or char "C", it sends the first

block of the file and waits. The receiver checks for the block number

and his complement (see BlK Number and Blk /Number) and verify the

Checksum or CRC of the data. ACK or NAK is send back if the control

is OK or not.

The sender sends the same packet if the control was bad or the next

if everything was good. After ten retrys for the same packet, the

transmission is aborted.

After the last block, the sender sends an EOT to indicate the end of

the transmission. The receiver acknowledges and the transmission is

closed.

Below is an example of the transmission in Xmodem with CRC detection

errors.

SENDER                             RECEIVER

"C"

STX 01 FE Data[1024] CRC CRC

ACK

SOH 02 FD Data[128] CRC CRC

ACK

SOH 03 FC Data[128] CRC CRC

ACK

EOT

ACK

To use Checksum detection errors, replace "C" by NAK in first line.

In this example, the first packet begin with STX so the data size

is 1024 bytes.

Remarks:

- the receiver selects the detection error (CRC/Cheksum)

- the sender selects the size of the data blocks (128 or 1024 bytes)

The last remark is important because the buffer on the HP48 is only

255 bytes long. So the option Long Packet can't be used !

Ymodem Batch protocol

---------------------

This protocol is an extension to Xmodem. Some usefull informations on

the file are send with the file (filename, size, date of creation...).

Unlike Xmodem, there is only the CRC error detection.

Like Xmodem, to start the communication, the receiver sends a char "C"

every ten seconds until the sender transmit the first block. With

Ymodem the first block is the block zero. This data block has the

following structure :

abc<null char>1234<space char><null char>...<null char>

In this example, 'abc' is the filename and 1234 is the size in byte.

They are separated by a null char. The size is a decimal string (like

doing ->STR with a real on the stack in STD mode). A space char follows

immediately the size. Null chars are used to complement the data block

to 128 or 1024 bytes.

If the block is good (no CRC error, no lost of sync...) the

receiver sends ACK. The rest of the file is sent like in Xmodem CRC.

Below is an example of the transmission in Ymodem Batch.

SENDER                             RECEIVER

"C"

SOH 00 FF Head[128] CRC CRC

ACK

"C"    \

SOH 01 FE Data[128] CRC CRC                |

ACK     |

SOH 02 FD Data[128] CRC CRC                |

ACK     | like in Xmodem CRC

SOH 03 FC Data[128] CRC CRC                |

ACK     |

EOT                                        |

ACK    /

"C"    \

SOH 00 FF Null[128] CRC CRC                | end the batch mode

ACK    /

To end a batch cession, a block zero with only null chars in data

field is sent. If the cession begin with a such block, no file is

sent !

Somes usefull ASCII codes (decimal):

SOH =  1          ACK =  6       "C" = 67

STX =  2          NAK = 18

EOT =  4          CAN = 24

The X & Ymodem protocol are explain in details in the following

document (very good) :

________________________________________

XMODEM/YMODEM PROTOCOL REFERENCE

A compendium of documents describing the

XMODEM and YMODEM

file transfer Protocols

Edited by Chuck Forsberg

________________________________________

We can E-mail to you this document if you need it.

X & Ymodem on the HP48sx

========================

As it was explain, you cannot use the Long Packet option because

the size of the HP48 receive buffer is only 255 bytes long. This is

not a drastic limitation because, in most softs availables (perhaps

all !), you can force the Short Packet option.

The reasons to implement X&Ymodem protocol on HP48 was :

- Kermit on HP48 is too slowly when you receive large files.

- Kermit don't permit to send directely an object stored in

a port (libraries or Backup objects).

- You can't receive an object and keep it on the stack. (usefull

when receiving librairies or backup)

For example, doing an ARCHIVE with the standard HP48 command take

ten minutes with Kermit for 90 kB. The time needed to download the

file take more then half an hour with the RECV command.

With Ymodem the operation is performed in three minutes to send and

three minutes to receive. Particulary usefull when hacking...

Unlike Kermit, there is no Garbage Collect during the transmission.

So the time needed to send or receive are approximatively equal and

are directely proportional to the size of the file and don't depend

of the low memory condition.

It's only possible to send or receive files in binary with this

version of X&Ymodem. Generaly, the files in ascii are short and the

time needed for the transmission with Kermit is acceptable. However,

if a file received with X&Ymodem don't contain the binary header

"HPHP48-x", the object is considered as a string and stored or keeped

on the stack as is. But, unlike Kermit, there is no possibility of

translate code :(

IMPORTANT REMARKS

=================

As previously described, the Xmodem protocol (not Ymodem) does not

send the size of a file. So if you send an object from the HP48 to

your favorite computer, the size of the received file will be aligned

to 128 bytes. So, in this case, there is a difference between the size

of the file and the size of the object.

After, if you download this file with Kermit, you will see a string

in place of the expected object. This string look like this :

"HPHP48-E#!'%@|{*.........

Why ? -Because Kermit perform a check on the size of the file and the

size of the object aligned to a byte. If there are equal, then the

object is considered as good. If not, the file is considerd as corrupted

and the representation is keeped in the string.

This test performed by Kermit is usefull to detect if a binary file

has been received as an ascii file and then send as an ascii or binary

file (in such a case some chars have been added or removed, like LF/CR).

This problem does not occur if you send the object from the HP48 to

your computer with Ymodem because the size of the file is transmitted

in the block zero. (see chapter Ymodem protocol) So you can receive it

later with Kermit without problems.

So, we can do this important remark :

DON'T SEND AN OBJECT WITH XMODEM, BECAUSE YOU CANNOT RECEIVE IT LATER

WITH STANDARD KERMIT COMMAND. USE YMODEM INSTEAD.

Now, what happends when you receive files with X&Ymodem ? The check

of the size between an object and a file is not done ! The only check

performed is the VERSTRING. There is two consequences :

- you can receive with Ymodem a file previously transmitted with

Xmodem.

- you can easily CORRUPT your memory if you don't know the

difference between ascii and binary modes.

If your are not familiar with ascii and binary modes, don't use this

library. However, if you are completely excited by the speed and the

comfort and you cannot resist to try the library, then read this :

- Select binary mode on your computer.

- Control if you have selected binary mode on your computer.

- Control if your previous control was correct.

Perhaps in next version, we will include the possibility to turn on

or off the size check.

Let us say again :

DON'T SEND AN OBJECT WITH XMODEM, BECAUSE YOU CANNOT RECEIVE IT LATER

WITH STANDARD KERMIT COMMAND. USE YMODEM INSTEAD.

REMEMBER : BINARY !

Preliminary settings

====================

In your computer, you must do the following settings :

- BINARY mode (have you forget ?)

- Short Packet (128 bytes)

- 8 bits data, 1 stop bit

- No XON/XOFF, no flow control

- Set the baud rate according with your IOPAR

Description of the commands

===========================

SETUPXYM

--------

This command call a menu to select some settings :

-------

|  CRC  |

-------

- Toggle flag 57. When flag is set (checked menu) the RECX

command begin the transmission with CRC. If the flag is

clear, then the Checksum is used.

-------

| REPL  |

-------

- Toggle flag -36. When flag is set (checked menu) the RECY

command overwrite existing object in memory (even directories).

If flag is clear, an extension is added to the name of the

object (like Kermit).

-------

|  STK  |

-------

- Toggle flag 58. When flag is set (checked menu) the RECY

command keeps objects on thestack. This feature is usefull

when transmitting libraries or Backup HOMEDIR.

-------

| EXIT  |

-------

- Go to previous menu ( does something like 0 LASTMENU ).

SENDM

-----

This command is similar to the SEND of Kermit but has been extended.

You can send an object stored in the current path, in a local name,

in a port or a Backup HOMEDIR ( like ARCHIVE ).

___________________________________________________

( 'Global'           ->    { } )

The object is sent in Ymodem Batch. If the object is not found

in the current path, then the transmission is ended without error,

but the output list contains the unsend Global Name.

If an error occurs during the transmission, the output list will

contains the Global Name and the transfert will be aborted. In this

case an error message appears on screen.

If the output list is empty the transmission was good.

___________________________________________________

( 'Local'            ->    { } )

Like a Global Name.

___________________________________________________

( :IO: Name          ->    { } )

Do a Backup HOMEDIR (like ARCHIVE). The output list is empty

if no errors occur during transmission. Else it contains the

:IO: Name. The transmission is in Ymodem Batch.

( :0: Name/Real/List ->    { } )

( :1: Name/Real/List ->    { } )

( :2: Name/Real/List ->    { } )

Send the object/library stored in the specified port. The

transmission modality is identical to a Global Name but the

name used to send the object has this structure :

P#$Name or P#$Real (for a library)

where # is 0,1 or 2 (see example)

ex: :0: ABC  SENDM

the object 'ABC' stored in port 0 is send with

the name 'P0$ABC'.

ex: :0: 1221  SENDM

the library 1221 stored in port 0 is send with

the name 'P0$1221'.

ex: :0: { ABC DEF GHI } SENDM

the var GHI stored in the subdirectory DEF of the

directory ABC which is stored in the port 0, is send

as 'P0$GHI'.

( :&: Name/Real/List ->    { } )

The char '&' is the wildchar. The search of the object begin

in the following order :

port 2, 1, 0 and in the current directory path.

The transmission modality is identical to a Global Name.

( :any: object       ->    { } )

Strip the mysterious tag and redo the dispatch to Global,

Local and List.

ex: :t: 'ABC'  SENDM

the object 'ABC' is send with the name 'ABC'.

___________________________________________________

( { }              ->    { } )

Do, for each element of the list, the previous dispatches with

one exception :

- if the current element is a list (or a tagged list) of two

elements { obj1 obj2 } then obj1 is send as obj2.

Where obj1 may be Global, Local or Tagged.

obj2 may be Global, Local or a String.

ex: { { ABC "DEF" } } SENDM

the object 'ABC' is sent with the name "DEF".

If the current element is not found, it is added to the output

list and the next element is processed. If an error occur during

transmission, the elements of the list not still processed are

added to the output list.

Another example for SENDM :

{ :w:A :x:y:IO:Apr17 :tag:0:1616 :z:{ :t:&:1221 "<-LIB->" }

:&:1092 :&:ABC { :0:{ dir subdir A } "NewName" } }

The object :w:A stored in the current path is send as 'A'.

The Backup HOMEDIR is sent as 'Apr17'.

The library 1616 stored in port 0 is send as 'P0$1616'.

The library 1221 stored somewhere is send as '<-LIB->'.

The library 1092 stored somewhere is send as 'P&$1092'.

The object :&:ABC stored somewhere is send as 'P&$ABC'.

if found in a port or as 'ABC' if found in the current

path.

The var A stored in the subdirectory of the var dir which is

stored in the port 0, is send as 'NewName'.

___________________________________________________

( any              ->        )

If the object on the stack has not been previously listed, it is

send in Xmodem protocol. But be carefull ! See IMPORTANT REMARKS.

RECX  (affected by flag 57. See SETUPXYM )

----

Receive a file with the Xmodem protocol. The received object is kept

on the stack. This is usefull for librairies because you don't need to

recall and purge the variable before storing in port. If the flag 57

is set, RECX request CRC error detection, else Cheksum.

RECY  (affected by flag -36 and 58. See SETUPXYM )

----

Receive one or more files with the Ymodem protocol. The files are

stored in the current directory if flag 58 is clear otherwise the

objects are kept on the stack. The flag -36, when set, allow you

to overwrite variables (even directories). If a non valid name is

received ( like '#abc' ), the object is stored with the name 'XYM'.

However, if 'XYM' already exists in the current dir, an extension

is added to 'XYM', even if flag -36 is set (replace enable).

ABOUTXYM

--------

This command is probably not the most usefull of the library.

Installation

============

As usual, send the library with the Kermit and store it in a port.

(smile, it's perhaps the last time you use it !)

BEFORE PLAYING, DO A BACKUP OF YOUR MEMORY WITH THE ARCHIVE !!!

Since six monthes, we only do transfer (backup included) with this

library and we had no problems with memory corrupt. But we used some

unsupported entry points. ( my HP48sx is a revision E )

We recommend to verify this library with the CHKIT command posted

to comp.source.hp48 by Detlef Mueller.

Informations

============

number : 1275

name   : XYMDM :v2.4 X&Y Modem

size   : #1914            \ command BYTES with

CRC    : 3623.5           / library on the stack

Flags

=====

-36  : This flag, when set, allows to overwrite existing files in your

current directory. (affects RECY; see commands SETUPXYM and RECY)

57  : This flag, when set, requests CRC error detection when receiving

in Xmodem. (affects RECX; see commands SETUPXYM and RECX)

58  : This flag, when set, allows to keep objects on the stack when

receiving in Ymodem. (affects RECY; see commands SETUPXYM and RECY)

Error messages

==============

# 1h     "Insufficient Memory"    The file to receive is too big !

# 5h     "Memory Clear"           Nooooooo, it's a joke !

# A03h   "Interrupted"            You have pressed the ATTN key or your

computer has cancelled the transmission.

# C02h   "Timeout"                Your computer is no responding. Verify

settings and connection.

# C06h   "Transfer Failed"        Too many retries when receiving a block.

# C07h   "Protocol Error"         When sending in Ymodem Batch, your

computer request Cheksum. Remember, in

batch mode always CRC.

# C12h   "Invalid IOPAR"          No comment...

# C14h   "Low Battery"            Place new batteries...

# 4FB01h "EOT Not Expected"       The HP has received a bad block (bad

CRC/Cheksum) and request, with a NAK,

for the same block. The NAK has been

garbled in an ACK and the sender close

the connection because there is no more

block to send. This error is similar to

# 4FB05h.

# 4FB02h "SOH Expected"           Your computer is sending Long Packets.

Select Short Packet instead.

# 4FB03h "#Block Expected"        The HP is waiting for Blk Number. Perhaps

an EOT has been garbled in a SOH and the

HP is waiting for an inexistant block.

# 4FB04h "Lost of sync."          The Blk Number and Blk /Number are not

correct. Try again with the clock

disabled.

# 4FB05h "Bad Block Number"       The Blk Number is not correct. Probably

the HP has received a bad block (bad

CRC/Cheksum) and request for the same

block with a NAK. The NAK has been

garbled in an ACK and the sender transmit

the next block which is not expected.

Unsupported entry points

========================

Here is the list of all the unsupported RPL entry points used in the

library.

=goodname?     EQU #2FDDE

=dupidlam?not  EQU #2E7A4

=togsysflag    EQU #3EDA2

=lastmenu      EQU #413B9

=splittag      EQU #05EC7

=rclplib@      EQU #09318

=WarnSTATU     EQU #04546

=RCLUSERF      EQU #1C64E

=STOUSERF      EQU #1C6F7

=STOSYSF       EQU #1C6E3

=STOF          EQU #1C6A2

=STIME         EQU #2EDF5

=BINT55h       EQU #64C66

=BINTC02h      EQU #2D5C3

=BINTC07h      EQU #65058

=STR_C         EQU #3D8AE

=$_REPL        EQU #3E0C9

=$_IO          EQU #2127D

=tok2          EQU #65364

=tok&          EQU #651B2

=CKEmbeddedDir EQU #41764

=DUPUserObj?   EQU #065E5

=PgDIR         EQU #09752

=PURGEcontents EQU #089FA

=CKPURGEcontents EQU #08C4A

=HOME>BAK      EQU #094A4

=BAK>HOME      EQU #09699

=REROOTHOME    EQU #0970A

There are not listed in the document previously posted by Joe Horn

concerning the difference between Rev E and Rev J. So, the library

should run in a Rev J but....

Edouard Westphal

Maurice Alou

E-mail : westphal@elde.epfl.ch