Codes for older Technics

Carrier: 32kHz

+--------+      +..

Header:   |        |      |

..+        +------+

8T     6T

+--+    +--+

1 is coded:  |  |    |  |

..+  +----+  +..

2T  6T  2T

+--+  +--+

0 is coded:  |  |  |  |

..+  +--+  +..

2T 2T 2T

T=420 us to approx 424 us (US of A and Canada)

T=454 us to approx 460 us (Europe and others)

Remote control transmitter data code (MN6030)

Head pulse  Custom code  Data code  Inverse custom code  Inverse data code

(8T high,   (5 bits)     (6 bits)   (1<==>0)             (1<==>0)

8T low

Followed by 90T low (for sync) = 41130us (Europe) = 37980us (USA)

To use the codes recall infosting and put the number on level 1 of

stack.

*********************************

Technics

---------------------------------

Amp

Code	Function

----	--------

#7E7	on/off

#79D	Muting

#7FB	Vol +

#70D	Vol -

#609	Eq on/flat

#649	Preset

---------------------------------

CD

Code	Function

----	--------

#14C	Play/Pause

#00C	Stop

#04C	Rew

#06C	FF

#0EC	Repeat

#52C	Time mode

#48C	Level +

#4AC	Level -

#54C	Memory

#16C	Clear

#20C	1

#22C	2

#24C	3

#26C	4

#28C	5

#2AC	6

#2CC	7

#2EC	8

#30C	9

#32C	0

---------------------------------

Tape

Code	Function

----	--------

#089	Deck a/b

#149	Play

#129	RewPlay

#009	Stop

#069	Rew

#049	FF

#109	Pause

---------------------------------

Tuner

Code	Function

----	--------

#209	1

#229	2

#249	3

#269	4

#289	5

#2A9	6

#2C9	7

#2E9	8

#309	9

#329	0

---------------------------------

Phono

Code	Function

----	--------

#5FB	Start

#3FB	Stop

---------------------------------

Bj�rn Gahm   -- Email: d89-bga@nada.kth.se

Snail: �ppelstigen 2

181 62 LIDING�

SWEDEN