Remote control info:	TEAC

Version:		3.1

Date:			4 May 93

Code length:		32 bits

Carrier: 		39.2kHz

The code is only transmitted once, and after that a stop code is transmitted.

+--------------------------------+                +..

Header:      |                                |                |

..+                                +----------------+

16T                       8T

8.8ms                     4.4ms

+--+      +--+

1 is coded:  |  |      |  |

..+  +------+  +..

T    3T   T

+--+  +--+

0 is coded:  |  |  |  |

..+  +--+  +..

T  T  T

+--------------------------------+                +--+

Stop code:      |                                |                |  |

..+                                +----------------+  +..

16T                      4T        T

8.8ms                   2.2ms

T = 550us

Space between transmissions: 11000us

To use the codes add the command with (the address * #10000h)

For more information please read the documentation.

**********************************

KENWOOD

----------------------------------

CD:

GROUP

Bj�rn Gahm   -- Email: d89-bga@nada.kth.se

Snail: �ppelstigen 2

181 62 LIDING�

SWEDEN