Remote control info:	CANNON

Version:		3.1

Date:			4 April 93

Code length:		32 bits

Carrier:		37.5kHz

+--------------------+

Header:      |                    |

..+                    +--------

16T            8T     (8.8ms + 4.4ms)

+--+      +--+

1 is coded:  |  |      |  |

..+  +------+  +....

T    3T   T

+--+  +--+

0 is coded:  |  |  |  |

..+  +--+  +....

T  T  T

T = 550us

Space between transmissions = 40ms

*********************************

CANON

---------------------------------

Videocamera

GROUP	#A16E

#C03F	Start/Stop

#B847	Zoom wide

#387C	Zoom tele

#807F	Play

#E817	Stop

#40BF	Rew

#00FF	FF

#20DB	Pause/Still

#E01F	REC

#708F	Char title

#F00F	Date

#B04F	Count reset

#609F	Tape return

#D827	On Screen

Bj�rn Gahm   -- Email: d89-bga@nada.kth.se

Snail: �ppelstigen 2

181 62 LIDING�

SWEDEN