Remote control info:	SONY

Version:		3.1.1

Date:			9 June 93

Code length:		12 bits (15 bits for some videocamera functions)

Carrier:		40kHz

+--------+

Header:      |        |

..+        +--

4T    T

+----+

1 is coded:  |    |

..+    +--

2T  T

+--+

0 is coded:  |  |

..+  +--

T  T

T = 550us approx

Space between data: 25ms

data: hhhhxxxxxxyyyyyy

^          ^

MSB        LSB

xxxxxx command

yyyyyy address ( the number of the apparatus )

tape deck a    # 20

tape deck b    # 1C

dat       # 38

cd        # 22

\<<

SONY              @ get info string

#8B2h             @ get the data

TRANSMIT          @ transmit

\>>

***************************

Tape

deck a	deck b

Code	Code	Function

----	----	--------

#830		auto play

#832	#71A	play

#833	#71B	rew

#834	#71C	ff

#835	#71D	rec

#836		rec pause

#837	#720	reverse play

#838	#718	stop

#839	#719	pause

#83A	#712	reverse rec

***************************

DAT

#E3A	play

#E3B	rew

#E3C	ff

#E3E	rec

#E38	stop

#E39	pause

#E00	1

#E01	2

#E02	3

#E03	4

#E04	5

#E05	6

#E06	7

#E07	8

#E08	9

#E14	0

#E20	open/close

#E21	counter mode

#E22	counter reset

#E25	display PGM time

#E26	display absolut time

#E27	display custom counter

#E28	display remaining time

#E29	date, present

#E2A	date, recorded

#E36	repeat one/all

#E4B	display mode (all/time/none) or E77

#E4E	margin reset

#E57	rewind and play

#E5B	skip play mode

#E5F	fader

#E6C	music scan

#E2D	record pause

#E3E	record at once

#E4F	record mute

#E69	title +

#E68	title -

#E50	start id auto

#E53	start id renumber

#E51	start id write

#E52	start id erase

#E54	skip id write

#E55	skip id erase

#E58	end id write

#E59	end id erase

#E71	display mode (level/sampling)

#E71	dimmer low

#E70	dimmer mid

#E6F	dimmer high

#E1B	RMS enter

#E1D	RMS check

#E1E	clear

#E56	?? <- I don't know what this exactly

#E73	?? <- is; maybe any service-features

***************************

Video

vtr 1     vtr 2     vtr 3

Code	Code	Code	Function

----	----	----	--------

#110	#390	#590	ch+

#111	#391	#591	ch-

#115	#395	#595	power

#118	#398	#598	stop

#119	#399	#599	pause

#11A	#39A	#59A	play

#11B	#39B	#59B	rew

#11C	#39C	#59C	ff

#11D	#39D	#59D	rec

#12A	#3AA	#5AA	ant

***************************

CD

Code	Function

----	--------

#880	1

#881	2

#882	3

#883	4

#884	5

#885	6

#886	7

#888	8

#889	9

#88A

#88B

#88C

#88D	Check	file text

#88E

#88F	Clear

#890	17

#891	18

#892	Line Out Volume Up

#893	Line Out Volume Down

#894

#895

#896	Open/Close

#897	19

#898	Custom Index

#899

#89A

#89B

#89C

#89D	Continue

#89E

#89F	Program

#8A0	10

#8A1	11

#8A2	12

#8A3	13

#8A4	14

#8A5	15

#8A6	16

#8A7	+10

#8A8	Time

#8A9	20

#8AA	Repeat A<->B

#8AB

#8AC	Repeat 1/All

#8AD

#8AE

#8AF

#8B0	AMS Backward

#8B1	AMS Forward

#8B2	Play

#8B3	Fast Audible Search Backward

#8B4	Fast Audible Search Forward

#8B5	Shuffle

#8B6	Index Backward

#8B7	Index Forward

#8B8	Stop

#8B9	Pause

#8BA	Slow Audible Search Backward

#8BB	Slow Audible Search Forward

#8BC	Music scan

#8BD

#8BE

#8BF

#8C0	Select Disc #1

#8C1	Select Disc #2

#8C2	Select Disc #3

#8C3	Select Disc #4

#8C4	Select Disc #5

#8CE	Auto space

#8DF	Fade

***************************

Amplifier

Code Function

----	--------

#631	dbfb

#642	rear lvl+

#638	off

#639	dolby

#63A	hall

#63B	simulated

#643	rear lvl-

#644	delay time

#812	vol+

#813	vol-

#814	muting

#815	power

#81E	video 2

#820	phono

#821	tuner

#822	video 1

#823	tape

#825	cd

#842	video 3

#843	video 4

#846	dat

***************************

Phono

Code	Function

----	--------

#828	start

#829	stop

***************************

Tuner

Code	Function

----	--------

#63B	shift

#690	pre+

#691	pre-

***************************

TV

Note:	Many of the codes were taken by Frank Hepmann

from a Sony monitor. But since the monitor has

the same adress as a normal TV these codes were

included here.

Code	Function

----	--------

#080	1

#081	2

#082	3

#083	4

#084	5

#085	6

#086	7

#087	8

#088	9

#089	0

#08B	enter

#090	ch+

#091	ch-

#092	vol+

#093	vol-

#095	power (toggle)

#0AE	power on

#0AF	power off

#0A5	tv/video

#098	picture +

#099	picture -

#09A	colour +

#09B	colour -

#09E	brightness +

#09F	brightness -

#0A0	hue + (purpur)

#0A1	hue - (green)

#0A2	sharpness +

#0A3	sharpness -

#0A6	balance L

#0A7	balance R

#096	normal values

#0C0	input line A

#0C1	input line B

#0C3	input AV

#0C5	input digital

#0C7	input vtr

#094	mute

***************************

Laser Disc Player

Code	Function

----	--------

#315	power

#318	stop

#31A	play

#31B	rew

#31C	ff

#335	skip rew

#334	skip ff

***************************

Videocamera

Code	Function

----	--------

#398	Stop

#399	Pause

#39A	Play

#39B	Rew

#39C	FF

#3DA	Data screen

#5C99	Start/Stop

#6C9A	Tele

#6C9B	Wide

Tele, Wide and Start/Stop uses 15 bits and requires

SONYCAM as infostring in order to work.

The other codes the normal SONY infostring.

Bj�rn Gahm   -- Email: d89-bga@nada.kth.se

Snail: �ppelstigen 2

181 62 LIDING�

SWEDEN