Codes for Panasonic.

The code is 48 bit long.

Carrier:  32kHz

+--------------------+

Header:      |                    |

..+                    +--------

10T            4T

+--+      +--+

1 is coded:  |  |      |  |

..+  +------+  +....

T    3T   T

+--+  +--+

0 is coded:  |  |  |  |

..+  +--+  +....

T   T  T

Space between transmissions 190T.

T = 400us approx

To use the codes add the command with (the address * #10000h)

*********************************

PANASONIC

---------------------------------

Video:

GROUP	#40040900

Code	Function

----	--------

#BCB5     VTR

#5059     Play

#0009     Stop

#4049     Rew

#C0C9     FF

#6069     Pause

#3039     Still adv

#1019     Rec

#F0F9     Slow

#7178     Slow+

#F1F8     Slow-

#0801     1

#8881     2

#4841     3

#C8C1     4

#2821     5

#A8A1     6

#6861     7

#E8E1     8

#1811     9

#9891     0

#DCD5     -/--

#6A63     Clock/Counter

#6C65     VTR/TV

#030A     Input select

#2A23     Reset

#8980     Monitor

#222B     Time search

#8D48     Tracking+

#4D44     Tracking-

Codes taken from a NV-FS100EG by Frank Heepmann.

They are tested on his VCR (German version).

#2C25     channel +

#ACA5     channel -

#CAC3     memory/repeat/search

#AAA3     remaining time

#CCC5     audio out

#0900     blank search

#2D24     timer

#020B     index            \  For the VISS (VHS index

#828B     address          | search system) and VASS

#D2DB     write            | (VHS address search

#323B     erase            | system) features.

#424B     renumber         /

#8188     jog/shuttle on   \  These codes allow to

#3039     jog >            | emulate the jog/shuttle

#BOB9     jog <            | button on the RC.

#1118     shuttle <<<      |

#6168     shuttle <<       | > = forward

#2128     shuttle <        | < = backwards

#4148     shuttle <|       |

#C1C8     shuttle |>       | examples:

#A1A8     shuttle >        | |>  = slow motion forward

#E1E8     shuttle >>       | <<< = very fast backwards

#9198     shuttle >>>      /

Group:    #40040980

#8009     check timer

#50D9     tape speed (sp/lp)

#0881     language select  \  These codes are only

#028B     text on          | successful if the VPT

#C24B     text off         | (timer programming by

#22AB     index            | videotext) option is

#A22B     f/t/b (text size)| installed. The module in

#62EB     reveal           | my VCR is called VW-VPT1.

#E26B     hold             | It also allows TOP (table

#52DB     list             | of pages) videotext, a very

#0A83     red              | fast page access system.

#8A03     green            | (videotext = teletext)

#4AC3     yellow           |

#CA43     blue             | In Germany the NV-FS100EG

#72FB     top/memory       | was Panasonic's one and only

#B23B     page preset      | VCR model with this option

#F27B     vpt              /  (unforunately).

---------------------------------

Amp:

GROUP	#40040543

Code      Function

----      --------

#FDBB     On/Off

#0D4B     Vol+

#8DCB     Vol-

#3D7B     Sleep

#F17B     Display

---------------------------------

CD:

GROUP	#40040543 (Same as amp)

Code      Function

----      --------

#E1A7     Play

#0147     Stop

#2167     Rew

#A1E7     FF

#D197     Program

#B1F7     Cancel

#4107     Random

#81C7     Repeat

#3094     1

#89CF     2

#490F     3

#C98F     4

#296F     5

#A9EF     6

#692F     7

#E9AF     8

#195F     9

#99DF     0

#F9BF     +10

---------------------------------

Tape:

GROUP	#40040543 (Same as amp)

Code      Function

----      --------

#6523     Play

#A5E3     RewPlay

#0543     Stop

#4503     Rew

#C583     FF

#7533     Reverse mode

#F5B3     Counter reset

#B5F3     Deck 1/2

---------------------------------

Tuner:

GROUP	#40040543 (Same as amp)

Code      Function

----      --------

#2761     Fm mode

#1D5B     Tune/Band

#3094     1

#89CF     2

#490F     3

#C98F     4

#296F     5

#A9EF     6

#692F     7

#E9AF     8

#195F     9

#99DF     0

Bj�rn Gahm   -- Email: d89-bga@nada.kth.se

Snail: �ppelstigen 2

181 62 LIDING�

SWEDEN