From: d89-bga@nada.kth.se (Bj|rn Gahm)

Subject: v05i050:  remote_bg - Remote Control v2.0, Part01/02

Date: 2 Jul 92 02:07:51 GMT

Hi all.

Here are new versions of the remote control programs.

I have made a more general progam to emulate more than one

remote control.

BEGIN_DOC remote.doc

----------------------------------------------------------------

WARNING!!!!

Even though this program works fine with my hp48-E I cannot be

responsible for memory loss or even hadware damage on your HP48.

----------------------------------------------------------------

Version 2.0

The only problem is that all the remotes I have tried have

different codings. (And I think that there are many more

codings that I haven't seen yet).

The types of codings that the program can generate is:

* Pulse modulated (the length of the pulse is the coding

1 is longer pulse, 0 is shorter)

* Space modulated (the space between the pulses is the

coding)

* Shift modulated (1 is space+pulse and 0 pulse+space or

or vice versa).

The code can be reversed.

The codelength can vary between 1 and 48 bits.

An inverted copy can be added to the code.

The number of repetitions can be set between 0 and 15

(it is useful whe you want the hp48 transmit a signal

as a batch job, since 1 transmission of the code is most of the

time not enough for the reciever to recognize)

I have added a read function to read codes from known remotes.

Main directory contains:

TRANSMIT  - the transmission program

SETUP     - subdirectory for setting up remote info (see below)

READ      - subdirectory for reading remote codes   (see below)

RC5REMOTE - Calculates the code and transmits (Philips)

GROUP     - Holds the group number (used by RC5REMOTE)

OLD       - used by RC5REMOTE

RC5       - Philips remote info

MATSU     - Panasonic and Technics info

OLDMATSU  - Panasonic and Technics info (Older equipment)

SONY      - Sony info

JVC       - JVC info

SETUP contains:

Show      - shows all the info from an infostring

HEADER    - shows or sets the header data

ONE       - shows or sets the 1 data

ZERO      - shows or sets the 0 data

LEN       - shows or sets the length

SPACE     - shows or sets the space between transmissions

REV       - toggles if the data is to be reversed

Inv       - toggles if an inverted copy is to be added

Mod       - toggels of a pulse is to be added after the code

REPS      - shows or sets the number of repetitions

T->P      - takes a time (in us) and converts it to pulse length

P->T      - takes a pulse length and converts it to time (in us)

S->T      - takes a space length and converts it to time (in us)

T->S      - takes a time (in us) and converts it to space length

READ contains:

RRC5      - read rc5 (Philips)

RMATSU    - read panasonic and technics

ROLDMATSU - read old panasonic and technincs

RSONY     - read sony remotes (needs the sony infostring)

RJVC      - read JVC remotes

RPION     - read Pioneer remotes

REVERSE

SAMPLE

T

IRSAMPLE

-----------------------------------------------------------------

To transmit a code just put the apropriate infostring on level 2

and the code on level 1 and press TRANSMIT.

The infostrings that i have managed to get so far are in the main

directory.

They are Sony, JVC, Panasonic, Technics, Pioneer and Philips.

See the separate listings for details on the different remotes.

Eg to play a sony CD:

Press SONY

Enter #8B2h

Press TRANSMIT

or in a program

<< SONY #8B2h TRANSMIT DROP2 >>

The DROP2 is needed since the transmit program does not alter the stack.

WARNING WARNING WARNING!!!!

The TRANSMIT program just checks that the data is there but not if it makes

sense. If an invalid string is used the hp48 might transmit for a very long

time. The only way to stop the program then is to press the reset button, since

the program turns off the display and the interrupts.

However it can not destroy any memory becauce is does not write to the memory

except to the irled.

----------------------------------------------------------------

To read a remote control, go to the READ directiory and press

the right read program (the screen will go blank).

Then hold the remote control about 4-6cm from the irport on the hp

and press the right button on the remote.

Now should the screen turn on again and after a few seconds you will

have a binary on level 1 that is the code.

If the code does not work the try with an another distance between

the hp and the remotecontrol.

----------------------------------------------------------------

To check or edit an infostring recall it to the stack and go to

SETUP directory.

In order to make new strings for unsupported remotecontrols you

have to have a oscilloscope to see how the codes looks like.

This is becauce the hp48 (as far as I know) can't detect the

width of the pulses.

Press Show to view all the information of the string.

Or press HEADER to see the header info

press ONE to see the 1 info and so on.

But don't press Rev Inv Mod since they change the info.

(Contents of an infostring see below)

Infostring: (16 chars)

|     |     |     |     |     |     |     |     |     |     |

|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|     |     |     |     |     |     |     |     |     |     |

|Pulse Space Reps |Pulse Space Order|Pulse Space Order|Data |

|  Header         |        1        |        0        |Len  |

|                 |                 |                 |     |

|     |     |     |     |     |     |

|  |  |  |  |  |  |  |  |  |  |  |  |

| LSB | MSB |     |     |     |     |

|   Trans   |Rev  |Inv  |Add  |Repeat

|   Space   |Data |Data |Pulse|     |

|           |     |     |     |     |

Head: Pulse: length of pulse, if length = 0 then there are no header

Space: time to next bit

Reps: 0 = send header always  1-9 send header that number of times

1,0:  Pulse: length of pulse

Space: time to next bit

Order: 83 "S" => space pulse

: 80 "P" => pulse space

Data len   : length of data in bits

Trans space: specifies the time between the repetitions of the data

Rev Data   : 82 "R" => reversed

32 " " => not reversed

Inv Data   : 73 "I" => the data is reversed and added to the data

-> data+inv(data).

32 " " => ignored

Add Pulse  : 65 "A" => a pulse is transmitted after the last bit.

32 " " => ignored

Repeat     : "0"-"?"=> the transmission is repeated (0-15) times

if no key is pressed

(if the program is run as an batch job and not manually)

To alter a infostring:

Put on stack:

Level 2: infostring

Level 1: data to change to

Press HEADER or ONE or ZERO or LEN ......

---------------------------------------

HEADER:

data is a list with pulse length space length and the number of repetitions.

If repetitions = 0 the the header is always transmited.

Ex.

2: infostring

1: { 30 40 0 };

HEADER

Will produce a header with a space that will be transmited all the time.

the data in the list are:

{ pulsewidth spacewidth repetitions }

The first item (30 in the example) is the header pulse width.

The second item (40 in the example) the space between the header pulse

and the first datapulse.

The third item (0 in the example) tells the program how many times it

shall transmit the header.

----------------------------------------

ONE and ZERO:

data is a list with pulse length space length and the number of repetitions.

If repetitions = 0 the the header is always transmited.

Eg.

2: infostring

1: { 27 18 "P" };

ONE

Will produce the coding for one with the pulse first.

the data in the list are:

{ pulsewidth spacewidth order }

The first item (27 in the example) is the header pulse width.

The second item (18 in the example) the space between the header pulse

and the first datapulse.

The third item ("P" in the example) tells the program the order of pulse

and space, P is pulse first.

----------------------------------------

LEN:

2: infostring

1: real number

Just put the number of bits as a real and press LEN.

Ex.

2: infostring

1: 12

LEN

Will set the code length to 12 bits.

-----------------------------------------

SPACE:

Sets the space between the transmissions.

2: infostring

1: real number

Just put the length of the space as a real and press LEN.

Ex.

2: infostring

1: 512

LEN

Will set the space between the transmissions to approx 23ms.

-----------------------------------------

Rev Inv Add:

Rev toggles between no reversion of the code and reversion.

You can see an "R" at the end of the infostring is the code is

to be reversed.

If the code is reversed then the msb becomes the lsb.

Inv toggles between add a inverted copy of the code and normal code.

You can see a "I" at the end of the code if this function is switched on.

Add pulse toggles between add a pulse after the last bit and normal transmission.

A pulse must be added if the code is spacewidth modulated.

-----------------------------------------

Bj|rn Gahm        Email: d89-bga@nada.kth.se

Snail: Forskarbacken 10/315

104 05 STOCKHOLM

END_DOC