Path: seq!spell

From: js12@uk.ac.cam.phx (Julian Taplin)

Newsgroups: comp.sources.hp48

Subject: v01INF3:  Learning Remote Controller Document v1.0

Message-ID: <2062@seq.uncwil.edu>

Date: 26 Aug 91 00:08:03 GMT

Sender: spell@seq.uncwil.edu

Followup-To: comp.sys.hp48

Lines: 114

Approved: spell@seq.uncwil.edu

Checksum: 3441055157 (verify with brik -cv)

Submitted-by: Julian Taplin <js12@uk.ac.cam.phx>

Posting-number: Volume 1, Info 3

Archive-name: lrn_rmt

BEGIN_DOC lrn_rmt.doc

Righto, I don't have very good newsgroup access, so much of what I

have to say might well have been said already, but maybe not, so here goes:

With all the talk of using the HP48 as a learning remote control,

it seems an appropriate time to share my experiences with remote control

applications in general, which are (admittedly) restricted to just two

types of IR Remote controls, those used by Philips devices, and Technics

(and probably other Matsushita) devices.

My original aim was to enhance the behaviour of my Hifi system,

which was comprised of both Philips and Technics parts, and (not

surprisingly), neither understood the others controller. I therefore planned

to use my old BBC Micro as a Remote control 'server', interpreting IR signals

from all sources, and sending out in turn the correct signals to the relevant

components, so that any CD Play button would not only start the CD playing,

but also set the Amp to the correct input, etc.

To accomplish this, I had to go into some depth into the workings of

IR RCD's in general. I should add that I have no formal training in

electronics, so all the folowing information is not necessarily as precise

as it could be, and I am only describing the methods as I discovered them,

and they may well be very well known to all those better qualified than myself!

I am aware of two different methods of encoding signals, both of which are

described below:

The first is the encoding type used by Philips devices, and is managed

by a SAA1050 chip (I think!). It sends a data word of about 24 bits, with each

bit being indicated by the frequency of the IR within a given time slot, so,

the data word looks like this (or as near as I can get in ascii!)

__    ____  __    __  ____    __

........

__  ____    __  ____  __    ____  __

That looks fairly meaningless, and for a while I was unable to work out

how it was encoded, but makes a lot more sense if you divide up the trace

into equal time slots, as shown below:

__|    |____|  __|    |__  |____|    |__  |

|    |    |    |    |    |    |    |    |

__  |____|    |__  |____|  __|    |____|  __|

|    |    |    |    |    |    |    |    |

Now, there are two distinct types of bit within the lines; those that have

a high frequency, or those that have a low frequency. Conveniently, the

low frequencies do not change state in mid timeslot, whilst the high freq.

ones do....

So, that particular fragment can be decoded as 100101001, which is fine.

you can then regenerate the original waveform from just the binary number,

which is exactly what was wanted for the Hifi project, and indeed, I have

6502 routines to sample, interpret, regenerate and transmit philips RCD Codes.

Technics, on the other hand, uses a different encoding method, that

seems slightly less robust (or it might just use a lower power transmitter!)

On the plus side though, the technics encoding is a lot easier to understand.

It simply uses the on pulses to split up patches of no IR. If the patch of

no signal is long, then it's one value, otherwise it's the other (I forget

which is which, and I'm not even sure if my definitions of one and zero would

square with the manufacturers definitions, as I worked it all out by

experimentation!)

Now, this is all very well if you are prepared to write code for each

device you wish to control, but all my experiences with anything to do with

hp machine code tell me that that is not a very good plan, so you can actually

sample any IR signal (as far as I know), in a more compact manner than just

dumping the data recieved from the diode to ram by simply timing the High-Low

and Low-High transitions in terms of code loops, and storing the number of

loops in memory. Providing that the transmit loop is nearly identical to the

receive loop in execution speed, then you can simply use that to retransmit

the signal, and this should only take about 50 bytes per sample (given a max

of 25bits data word, which is reasonable), as opposed to most other methods.

However, that needs a modicum of work, as you have to be able to work out when

one word ends, and the gap between words start - this is not too difficult, as

the gap is normally at least as big as the whole word itself.

Sadly, I am going on a years holiday away from Email and Newsgroups,

and although my trusty 48sx is going with me, I may not have the time or

facilities to write this code (for starters, I will not be with my Amp and CD

player, so my test bed is also not present!). I therefore hope that some

enterprising person, with rather more experience with Saturn code than me,

will get round to writing it instead - the ultimate use of this is, of course,

to be able to get the TV to automatically avoid all soap operas, by having

the 48 switch channels as they are about to come on!

Any feedback from this note should (sadly), be posted to me at the

address shown, as I am not currently able to read the newsgroups with any

reliability,so if you want to get back to me specifically, post to me

(although any mail after the 30th August will basically sit around for a year

without being read!)

PS. Remember to turn off interrupts, screens, or anything else that may eat

bus cycles unexpectedly while sampling or replaying!

----------------------------------------------------------------------------

Julian Taplin				|Home of <<The Computer Graveyard>>!

35 Shelly Garden, CAMBRIDGE, ENGLAND    |Retirement home for all old, tired,

(0223) 324815				|and generally dead hardware!

----------------------------------------------------------------------------

EMail: JS12@UK.AC.CAM.PHX     <<<--- No, it's not strictly my MBox, but it

will have to do for now!

END_DOC