The Memory Scanner/Editor       v 1.0

Copyright by Raymond Hellstern 1992,1993

General:

--------

Eight lines, divided into an address field and a data field (16 digits)

(The bottom row is reserved for menus)

Very fast display routine

Very fast incremental scan (see example)

Convenient Poke ability

...and more

The monitor will run on rev.A to E (of course), possibly rev.J machines

(rev.J and GX not tested yet)

Menu_Keys: Page 1

-----------------

SCAN    Start scan mode

GOTO	Ask for address to jump to

REREAD	Upate display

HELP	2 Help screens

ABOUT	self-explanatory

Menu_Keys: Page 2

-----------------

A..F

Hard_Keys:

----------

Cursor keys     (with fast repeat)

When you hold down a cursor key, it will repeat until you release.

The display is updated automatically.

NXT     a: In scan mode, search next occurence of Search$

b: In normal mode, switches from main menu to edit menu and back

<-	Corrction of erreanous input while doing a GOTO

ENTER	Push current address onto stack

ON	Exit program. In Scan mode, exit to Normal mode

0..9	In Edit mode, poke nib into memory

Example:

--------

Start program by pressing the corresponding key.

GOTO Key

--------

Press [GOTO] to get a five-Uscore prompt for an address. ("ADR:_____")

The menu will change to [A]..[F]. You can now key in five digits (0-9,A-F).

Key in 1,F,C. The calc shows "ADR:1FC__". Oh! The 'C' was wrong.

Press [<-] to delete the wrong number. ("ADR:1F___") Key in B,B,D.("ADR:1FBBD")

The input will be terminated by keying in the last digit and the

display will be updated. The first line will show "1FBBD:D9D20...".

From this address on, we want to scan for the nib sequence "11920".

SCAN Mode

---------

So press [SCAN] to activate Scan mode. The menu will change to [A]..[F].

Now press '1' on the num pad. The display will show "1FBC6:1.." with

the '1' highlighted.

Now press '1' again. The display will show "1FBD6:11.." with '11' highlighted.

Press '9'.  The display will show "20118:119.." with '119' highlighted.

Fortunately, this is the first occurence of '11920'.

Ok, we want to scan further for the whole five digits.

Press '2'. The display will show "20496:1192.." with '1192' highlighted.

This is the next occurence of '11920', of course.

But, press '0'. The display will show "20D2C:11920.." with '11920' highlighted.

Now you can use [NXT] to search for the next (or all following) occurences

of '11920'. You could even scan for very long combinations.

The search can be interrupted at ANY time. A warning will be displayed

and the display will then turn back to the last valid address.

If a pattern canno be found, a warning will be displayed, the last digit

input will be discarted and the current address is left unchanged.

No matter if you press the cursor keys. You can move around with them, and

when you press [NXT] again, the search for the given pattern will be restarted.

Press [ON] to leave Scan mode. You're back to Normal mode.

The search string is flushed.

HEX input mode

--------------

If you press [NXT] in Normal mode, you will enter HEX input mode.

The Menu keys and the numeric keypad are redefined to let you poke

the digits 0 through 9 and A through F into memory, each time

advancing the cursor to the next address.

This way you can key in a series of digits

without having to move the cursor manually.

ENTER Key

---------

Press [ENTER] to copy a system integer onto the

stack whose value reflects the current address.

STARTUP Value

-------------

A system integer in level 1 of the stack is interpreted

as the starting address.  Range: from 0h to FFFFFh.

SCN will start at address #100h if the stack is empty.

P.S.    Chksum/bytes of MEMSCAN: #AF64h/2595