Documentation for Routh-Hurwitz Stability Criterion

---------------------------------------------------

This is a routine to perform the Routh-Hurwitz test on a characteristic

equation.  It is mostly a re-write of a program by Robert Yoder and Paul

Dujmich (ryoder@ecn.purdue.edu and pauld@fs1.ece.cmu.edu).  It fixes a bug

I found in the special case when the coefficients in a derived row are

zero and the auxiliary polynomial must replace that row.  The previous

version took the derivative of the wrong row.  Also, I cleaned up the code

so it executes much faster.  I added a check that would determine from

the Routhian array whether the system was stable or unstable.  To use,

just enter the coefficients of the characteristic equation in an array

and execute Routh.  Shortly, the Routhian array is returned along with a

statement indicating the status of the characteristic equation (stable or

unstable).  Because variables can not be put into arrays, this routine can

not be used to find the range of gains to make the system stable.  Some

examples I found in the book, Modern Control Engineering by Katsuhiko Ogata

follow.  Thanks to the pioneers of this routine, Robert and Paul.  Please

send suggestions, complains, comments.

Dave

________________________________________________________________________________

David Jansen            |  INTERNET: eldorado@en.ecn.purdue.edu

Electrical Engineering  |  BITNET:   eldorado%ea.ecn.purdue.edu@purccvm

Purdue University       |  UUCP:     {purdue, pur-ee}!en.ecn.purdue.edu!eldorado

Example 1

4     3     2

s  + 2s  + 3s  + 4s + 5 = 0

enter the coefficients [1 2 3 4 5] on the stack and execute ROUTH

returned is the matrix [[1 3 5] [2 4 0] [1 5 0] [-6 0 0] [5 0 0]]

and the statement "UNSTABLE".

Example 2

3     2

1s  + 2s  + 1s + 2 = 0

enter [1 2 1 2] on the stack and execute ROUTH

Again, the matrix is returned and the status of the system.

2: [[1 1] [2 2] [1e-50 0] [2 0]]

1: "STABLE"

Example 3

5     4      3      2

s  + 2s  + 24s  + 48s  - 25s - 50 = 0

1: [1 2 24 48 -25 -50]

ROUTH

2: [[1 24 -25] [2 48 -50] [8 96 0] [24 -50 0] [112.7 0 0] [-50 0 0]]

1: "UNSTABLE"