CW: A Morse Code Practice Program for the HP-48SX

CW is a directory containing several programs and variables which

comprise a capable Morse code practice system for the user.  The

program can send code using the beeper at speeds slightly over 20

words per minute continuously, without encountering delays due to

calculator "garbage collection".  The user can set the overall

speed and element speed independently, or can manually set the

timing and delay for each element type.  The program will create,

send, and display five character random code groups, or the user

can store any size string in a variable to be sent and displayed

manually.  The following descriptions explain the programs and

variables and their use.

CGR (Code GRoups) is a program which creates a string of 25 five

character random groups, sends them using the present speed

parameters, and then displays them so the user can check his

copy.  The characters are selected from a string in CGR

which contains all the characters and prosigns used in amateur

radio license tests.  If the user wishes to practice only certain

characters, this string can be changed with 'CGR' RCL EDIT or 'CGR'

VISIT and the groups will then include only the characters from

the modified string.

SPD (SPeeD) is a program which calculates and sets the various

parameters affecting the timing of each type of element.  The

program prompts the user for the overall speed in words per

minute and the element speed, also in words per minute.  After

entering these two numbers on the stack, the user presses [LSHFT]

CONT and the program stores values in ST, LT, ET, CT, and WT.

SPD will not signal an error if speeds are set too high or if

element speed is less than overall speed, but one of the values

in ST, LT, etc. will be set to zero or negative and CWO will halt

or hang.  Calculator speed and overhead as well as time to do

the garbage collect operation limit the speed to slightly more

than 20 wpm if overall and element speeds are the same.  If

the element speed is set faster than the overall speed, somewhat

higher speeds can be reached, 26 wpm overall with 29 wpm element

speed being about the maximum in any case.  The speed will be

quite accurate, within 1% or so in most cases.  The user can

measure the speed if desired by storing the string

"paris paris paris ..." in TXT and using a stopwatch to time from

the first element of a word to the same first element a few words

later.  Divide the number of words timed by the number of seconds

on the stopwatch and then multiply that by 60 to obtain the speed

in words per minute.

CWO (CW Output) is the program which actually sends the Morse

code using the beeper.  CWO sends the characters contained in the

string TXT.  TXT is set and CWO is called automatically by the

CGR program, but the user can set TXT and then execute CWO

manually to practice something other than random code groups.

DSP (DiSPlay) is a program which display a screenful of

characters from the TXT string.  CGR calls this program to

display the code groups it creates and sends.  If TXT is more

than one screen long, DSP halts after each successive screen.

The user can view the next screen by pressing [LSHFT] CONT.

TXT (TeXT) is a variable which contains a string which will

be sent as Morse code by the CWO program.  Any ACSII character

with decimal code 32 through 128 can be contained in TXT.

Characters that have no representation in Morse code will be

sent as a space.  Note that some of the radio prosigns which

have no representation in ASCII have been placed on some of

those characters.  For a listing of these, see the CODE

variable.  Note that lower case characters are folded into

upper case.

CODE (CODE) is a variable which contains a list of short

programs, one for each character.  CWO disassembles this list

on the stack before sending so that it can access the

character codes much more quickly than by other methods.

Prosigns which have no associated ASCII character are rather

arbitrarily placed on the following characters:

understood (SN)           !

error (HH)                #

paragraph (AL)            *

starting signal (KA)      [

wait (AS)                 \

end of work (SK)          ]

ST (Short Time) is a numeric variable containing the basic

element (dit) time in seconds, normally set at 1.2/WPM.

LT (Long Time) is a numeric variable containing the longer

element (dah) time, set at 3*ST.

ET (interElement Time) is a numeric variable containing the

delay time between elements.  This is generally set at the

same value as ST, less the calculator overhead time between

elements, .04 sec.

CT (Character Time) is a numeric variable containing the

delay time between characters.  This is usually set to the

same value as LT, adjusted for various calculator overhead

times in the program.

WT (Word Time) is a numeric variable containing the delay

time between words.  This is normally set to 7*ST, adjusted

for various calculator overhead delays.

FR (FRequency) is the frequency in hertz of the beeper.

Set at 500 hertz, but the user may store any value which

suits individual taste.

S (Short beep) is a program which actually sends a single

dit element.  The user will not use this program.

L (Long beep) is a program which actually sends a single

dah element.  The user will not use this program.

W (Word wait) is a program which generates the delay for

a space character.  The user will not use this program.