#(8U#(s0p16.67h8.5v0s0b0T

Subject: HP Spice v3.0

BEFORE: Load library symmatsp.lib

This program is run from a CST menu and all you should need to do can be done from there.

There are thirteen ideal components that this program supports, and these are their component formats:

Resistors: { R n1 n2 value }        -n1 and n2 are the nodes around the resistor  and the value is  (obviously)  the value

the value may be complex (i.e.  an impedance)

Conductors: { G n1 n2 value }       -same as a resistor

Capacitors: { C n1 n2 value }       -same as a resistor only you can't have complex capacitances

Inductors:  { L n1 n2 value b }     -same as a resistor  only you are required to have a branch number  through  which the

current flows (from n1 to n2)

Current Source: { Is n1 n2 value }  -the current flows from n1 to n2 with a magnitude of value

Voltage Source: { Vs n1 n2 value b }

-the positive end of the source is at n1 and the current through b flows from n1 to n2

(if you don't like it, well, too bad!)

Current Controlled Current Source: { IcIs n1 n2 n3 n4 value b }

-this  controlled  source will produce a current of value times the current through b,

which flows from n1 to n2, from n3 to n4

Voltage Controlled Current Source: { VcIs n1 n2 n3 n4 value }

-this controlled  source will produce a current of value times the voltage  difference

between n1 and n2 from n3 to n4

Current Controlled Voltage Source: { IcVs n1 n2 n3 n4 value b1 b2 }

-this controlled  source will produce a voltage  difference of value times the current

b1, which flows from n1 to n2, with the  positive  end at n3 and a current  through b2

that flows from n3 to n4

Voltage Controlled Voltage Source: { VcVs n1 n2 n3 n4 value b }

-this controlled  source will produce a voltage  difference of value times the voltage

difference between n1 and n2 with the positive end at n3 and a current through b which

flows from n3 to n4

Short Circuit: { S n1 n2 b }        -b is the branch that current flow through in the short from n1 to n2

Operational Amplifier: { O n1 n2 n3 value b }

-The Op Amp is  semi-ideal.  This seemed like the best way to  implement  it so I did.

n1 is the non-inverting input, n2 is the inverting input, n3 is the output node, value

is the open loop amplifier  gain, and b is the output branch with a current that flows

out from n3.  If value is sufficiently large, the Op Amp will as though it's ideal.

Transformer:  { M n1 n2 n3 n4 l1 l2 m b1 b2 }

-n1 is the  entrance  node of the primary  winding, n2 is the exit node of the primary

winding, n3 is the entrance node of the secondary  winding, n4 is the exit node of the

secondary  winding, l1 is the inductance of the primary  winding, l2 is the inductance

of the secondary  winding, m is the mutual inductance  between coils, b1 is the branch

of the  current  that flows from n1 to n2, and b2 is the  branch of the  current  that

flows from n3 to n4

These components can be entered by pressing the component's  soft-key and responding to the prompts, or, if you really get

into it, by putting a component list like shown in the the above  definitions  and pressing the  orange-shifted  soft-key.

Using the regular  soft-key  will insure that you are putting  things in the correct  places but using the  orange-shifted

soft-key will not, so be careful.

Again, I must stress that, unlike the original  HPSpice, these components are ideal.  When entering values for a component

and a unit can be associated  with that value you may do so, otherwise  the program will assume normal units (i.e ohms for

resistances,  Farads for capacitance,  etc.).  To enter a unit type other than regular use the [_] key and a metric prefix

before the unit (i.e.  7.8_pF for a capacitor).

Please  insure that you have a variable  'Circuit'  when you start to make your  circuit  otherwise  the first  element in

'Circuit'  will be 'Circuit'.  This has slipped by me several  times and sent me on several  wild-bug-goose  chases.  Just

remember, create carefully!

When you have  finished  creating  your circuit you can see what you have by pressing  the soft key with the printer  icon

(thanks to A.R.  Duell) which will dump all of the components onto the stack for your viewing pleasure.

When you feel you have all the  components  you desire, press the compile key and the program will create three  matrices:

MatG, MatB, and Sources.  MatG is a matrix of the  conductances,  MatB is a matrix of the  susceptances,  and Sources is a

complex  1-dimensional  matrix of the complex  sources in the circuit  (not  controlled  sources!).  These will be in your

variable directory before 'Circuit'.  Anything before 'Circuit' you are welcome to erase but you will have problems if you

erase anything else.

After  compilation  you can  view  the  fruits  of your  labours  by  pressing  D.C.  for a D.C.  anlysis  of the  circuit

(interpreting  complex  sources as a D.C.  source of equal  magnitude).  Otherwise you can enter a frequency (not angular)

onto the stack, press the [F->V] key, and view the A.C.  analysis of your circuit.  Simple but efficient.

I have also  written a transient  routine that works much faster than any other I've seen around so far (around 1 3/4 mins

for a graph instead of about 8).  However, use of this function, and the following  transfer  function  routine,  requires

that you have my SYMMATRX.LIB which consists of various symbolic matrix manipulation  functions.  I decided not to combine

these  routines  into the code as they were  originally a completely  seperate  utility that happened to be useful in this

case.  Anyways,  this  transient  routine,  the  [V(F)]  soft-  key, will  prompt  you for a voltage  node and a range for

frequency.  I currently  have it set up to plot the real portion, then the imaginary  portion, and then the absolute value

of the voltage as a function of frequency (in Hz).  I hope ya like it.

The other function I put into this program is the transfer function routine.  If you press the [H(S)] soft-key you will be

prompted for the positive and negative nodes of both the input and output voltages for the transfer  function adn then the

calculator  will  think for quite a while and give you a messy  looking  algebraic  containing  's'.  s=jw  where w is the

angular  frequency.  H(s)=Vout/Vin.  I didn't write in any cleanup routines for the algebraic because I felt they took too

long.  I find it much easier to simply  clean it up on paper.  It isn't great but it is  occassionaly  useful.  Again, you

need the SYMMATRX.LIB to run this routine.