#

File: onkyo.info,  Node: Top,  Prev: (oldtech.info),  Next: (orion.info),  Up: (remotes.info)

******************************************************************************

ONKYO

******************************************************************************

Version:		3.2

Date:			20 Nov 93

* Menu:

* Technical info::

* Amp::

* CD::

#

File: onkyo.info,  Node: Technical info,  Prev: Top,  Next: Amp,  Up: Top

******************************************************************************

TECHNICAL INFO

******************************************************************************

Code length:		32 bits

Carrier: 		39.2kHz

The code is only transmitted once, and after that a stop code is transmitted.

+--------------------------------+                +..

Header:      |                                |                |

..+                                +----------------+

16T                       8T

8.8ms                     4.4ms

+--+      +--+

1 is coded:  |  |      |  |

..+  +------+  +..

T    3T   T

+--+  +--+

0 is coded:  |  |  |  |

..+  +--+  +..

T  T  T

+--------------------------------+                +--+

Stop code:      |                                |                |  |

..+                                +----------------+  +..

16T                      4T        T

8.8ms                   2.2ms

T = 550us

Space between transmissions: 11000us

#

File: onkyo.info,  Node: Amp,  Prev: Technical info,  Next: Tuner,  Up: Top

******************************************************************************

AMP

******************************************************************************

Device:	Onkyo amplifier (name: A-8780)

GROUP:	#4BB6h

Code	Function

----	--------

#20	power on/off

#40	volume +

#C0	volume -

#A0	muting

#10	tape 1

#E0	tape 2

#D0	tuner

#90	cd

#30	dat

#60	aux 1

#88	aux 2

#50	phono

#

File: onkyo.info,  Node: Tuner,  Prev: Amp,  Next: Amp,  Up: Top

******************************************************************************

TECHNICAL INFO

******************************************************************************

The Onkyo RI system allows to control devices like a tuner or a cd

(or tape 1&2) via cable link between the amplifier and the device.

This certainly works only on other Onkyo components with the IR-logo,

connected twith the amplifier.

tuner (via link from amplifier)

Code	Function

----	--------

#00	channel +

#80	channel -

#08	scan

#52	class

#A2	antenna

#62	fm

#E2	am

cd (via link from amplifier)

Code	Function

----	--------

#D8	play

#F8	pause

#38	stop

#78	track <<

#B8	track >>

Device:         Onkyo tuner (name: T-4670)

GroupCode:      #4BA4h

Function                        Code (hex)      Remarks

----------------------------------------------------------------------------

#00	channel +

#80	channel -

#08	scan

#32	class

#D8	shift

#62	rf mode

#E2	if band

#12	hi-blend

#92	mode (mono/stereo)

#A2	antenna

#90	fm

#50	am

#38	1

#88	2

#48	3

#C8	4

#18	5

#98	6

#58	7

#B0	8

#70	9

#F0	10

#0A	11

#8A	12

#4A	13

#CA	14

#2A	15

#AA	16

#6A	17

#EA	18

#1A	19

#9A	20

Device:         Onkyo cd (name: DX-6770)

GroupCode:      #4B34h

Code	Function

----	--------

#04	play

#D8	play

#F8	pause

#38	stop

#84	stop

#D0	open/close

#78	track <<

#B8	track >>

#80	search <<

#00	search >>

#40	level +

#C0	level -

#50	elapse/remain

#62	shuffle

#7A	fader

#90	memory/memory reverse

#A2	intro scan

#60	repeat

#E0	a-b repeat

#32	auto space

#BA	peak search

#B2	time edit

#C2	edit <-

#42	edit ->

#10	clear

#F0	+10

#70	0

#08	1

#88	2

#48	3

#C8	4

#18	5

#98	6

#58	7

#30	8

#B0	9

#F2	10

#0A	11

#8A	12

#4A	13

#CA	14

#2A	15

#AA	16

#6A	17

#EA	18

#1A	19

#9A	20