#

File: technics.info,  Node: Top,  Prev: (teac.info),  Next: (yamaha.info),  Up: (remotes.info)

******************************************************************************

TECHNICS

******************************************************************************

Version:	3.2

Date:		10 feb 94

* Menu:

* Technical info::

* Amp::

* CD::

* Tape::

* Tuner::

* Phono::

* Stereo Sound Processor::

#

File: technics.info,  Node: Technical info,  Prev: Top,  Next: Video,  Up: Top

******************************************************************************

TECHNICAL INFO

******************************************************************************

Code length:		48 bits

Carrier:		32kHz

+--------------------+

Header:      |                    |

..+                    +--------

10T            4T

+--+      +--+

1 is coded:  |  |      |  |

..+  +------+  +....

T    3T   T

+--+  +--+

0 is coded:  |  |  |  |

..+  +--+  +....

T   T  T

Space between transmissions 190T.

T = 400us approx

#

File: technics.info,  Node: Amp,  Prev: Technical info,  Next: CD,  Up: Top

******************************************************************************

AMP

******************************************************************************

GROUP	#40040500

Code      Function

----      --------

#0401     Vol+

#8481     Vol-

#4C49     Muting

#4346     Super Bass

#5154     Aux Digital

#191C     Dat

#797C     VCR

#4540     VDP

GROUP	#40040520

Code      Function

----      --------

#BC99     on/off

#694C     Sleep

#0326     CD

#83A6     Tape

#4366     Tuner

#

File: technics.info,  Node: CD,  Prev: Amp,  Next: CD,  Up: Top

******************************************************************************

CD

******************************************************************************

GROUP	#40040550

Code      Function

----      --------

#5005     Play

#6035     Pause

#0055     Stop

#5207     ff

#92C7     Rew

#5401     Program

#C590     Cancel

#80D5     Open/Close

#AAFF     Time mode

#085D     1

#88DD     2

#481D     3

#C89D     4

#287D     5

#A8FD     6

#683D     7

#E8BD     8

#184D     9

#98CD     0

#2174     >10

#

File: technics.info,  Node: Tape,  Prev: CD,  Next: Tuner,  Up: Top

******************************************************************************

TAPE

******************************************************************************

GROUP	#40040510

Code      Function

----      --------

#A9BC     Deck 1/2

#D0C5     Play

#5045     Rew Play

#0015     Stop

#C0D5     Rew

#4055     FF

#6075     Pause

#1005     Rec

#4145     Rec mute

#

File: technics.info,  Node: Tuner,  Prev: Tape,  Next: Tuner,  Up: Top

******************************************************************************

TUNER

******************************************************************************

GROUP	#40040520

Code      Function

----      --------

#082D     1

#88AD     2

#486D     3

#C8ED     4

#280D     5

#A88D     6

#684D     7

#E8CD     8

#183D     9

#98BD     0

#

File: technics.info,  Node: Phono,  Prev: Tuner,  Next: Stereo Sound Processor, Up: Top

******************************************************************************

PHONO

******************************************************************************

GROUP	#40040570

Code      Function

----      --------

#5025     Play

#0075     Stop

#

File: technics.info,  Node: Stereo Sound Processor,  Prev: Tuner,  Up: Top

******************************************************************************

STEREO SOUND PROCESSOR

******************************************************************************

Stereo Sound Processor SH-CH750

GROUP:	#40040548

#0C41	On/Flat

#410C	SFP

#414C	EQ		(stepping through equalizer-functions)

#A1AC	Effect down	(increasing equalizer-effect)

#212C	Effect up	(decreasing equalizer-effect)

#838E	SuperBass