#

File: sharp.info,  Node: Top,  Prev: (schneide.info),  Next: (sony.info),  Up: (remotes.info)

******************************************************************************

SHARP

******************************************************************************

Version:		3.2

Date:			10 feb 94

* Menu:

* Technical info::

* TV::

#

File: sharp.info,  Node: Technical info,  Prev: Top,  Next: TV, Up: Top

******************************************************************************

TECHNICAL INFO

******************************************************************************

Code length:		17 bits

Carrier:		32kHz

Transmits code + code XOR #7FE0h

header:	None

one:	275us pulse + 1900us space

zero: 	275us pulse + 775us space

Space between transmissions:	43ms

#

File: sharp.info,  Node: TV, Prev: Technical info,  Up: Top

******************************************************************************

TV

******************************************************************************

GROUP:	Not needed

Code	Function

----	--------

#2141	0

#2021	1

#2041	2

#2061	3

#2081	4

#20A1	5

#20C1	6

#20E1	7

#2101	8

#2121	9

#2221	Ch+

#2241	Ch-

#22C1	Power

#25E1	10+

#2601	20+

#2621	30+

#2261	TV/video

#22E1	Mute