#

File: yamaha.info,  Node: Top,  Prev: (technics.info),  Up: (remotes.info)

******************************************************************************

YAMAHA

******************************************************************************

Version:		3.2

Date:			4 April 93

* Menu:

* Technical info::

* CD::

* Tape::

#

File: yamaha.info,  Node: Technical info,  Prev: Top,  Up: Top

******************************************************************************

TECHNICAL INFO

******************************************************************************

Code length:		32 bits

Coding type:		Space

Carrier: 		39.2kHz

Space between trans:	11000us

+--------------------------------+                +--+

Header:      |                                |                |  |

..+                                +----------------+  +..

16T                       8T

8.8ms                     4.4ms

+--+      +--+

1 is coded:  |  |      |  |

..+  +------+  +..

T    3T   T

+--+  +--+

0 is coded:  |  |  |  |

..+  +--+  +..

T  T  T

+--------------------------------+        +--+

Stop code:      |                                |        |  |

..+                                +--------+  +..

16T                  4T     T

8.8ms                2.2ms

T = 550us

The code is only transmitted once.

After that a stop code is transmitted.

#

File: yamaha.info,  Node: CD, Prev: Technical info, Next: Tape, Up: Top

******************************************************************************

CD

******************************************************************************

GROUP	#9E61

Code	Function

----	--------

#08	0

#88	1

#48	2

#C8	3

#28	4

#A8	5

#68	6

#E8	7

#18	8

#98	9

#02	10

#82	11

#42	12

#C2	13

#22	14

#A2	15

#62	16

#E2	17

#12	18

#92	19

#52	20

#58	+10

#40	Play

#C0	Pause/Stop

#A0	Rew

#60	FF

#D0	Index

#80	Open/Close

#90	Pepeat A-B

#10	Pepeat S/F/Off

#E0	Track +

#20	Track -

Codes from Tom Bolt

#30	Program

#D8	Random

The second byte of the hex code is # ffh minus the first byte.

Of these 256 combinations, all of the useful commands have the first

byte x0, x2, x4, x8 or xA.

YAMAHA

There is no play-exchange remote command. ;-(

The first byte of the existing commands are:

0       2       4       8       A

0       mode    10      -       0       <D

1       repeat  18      D8      8       -

2       <T      14      D4      4       -

3       program 22      -       -       -

4       play    12      D2      2       -

5       time    20      D10     +10     +30

6       S>      16      D6      6       stop

7       -       24      -       -       -

8       open    11      D1      1       -

9       -       19      D9      9       +20

A       <S      15      D5      5       pause

B       clear   23      -       -       -

C       pau/stp 13      D3      3       -

D       index   21      -       random  -

E       T>      17      D7      7       -

F       -       D>      -       -       -

S=search, T=track, D=disk.

Tom Boldt

che043sa@sail.uwaterloo.ca

(519) 746-5163

#

File: yamaha.info,  Node: Tape,  Prev:  CD,  Up: Top

******************************************************************************

TAPE

******************************************************************************

GROUP	#FE01

Code	Function

----	--------

#00	Play

#C0	Stop

#80	Rew

#40	FF

#31	Rec/Pause

#C0	Rec/Mute

#B0	Reset

#70	Memo

#B8	Remain

#EB	Tape

#D0	Search rew

#50	Search ff

#F8	Intro rew

#78	Intro ff

#08	0

#88	1

#48	2

#C8	3

#28	4

#A8	5

#68	6

#E8	7

#18	8

#98	9