What are the different interactive self-tests?

Pressing ON-D enters the interactive self test function of the HP48. When you first press ON-D, your HP will beep and you will see 3 vertical lines, one on each side of the screen and one in the middle.

You can exit this mode with a Warmstart, ON-C.  The tests are:

[A]

Displays CPU speed

[B]

LCD test.  Press Enter to cycle through the tests

[C]

Internal ROM test

[D]

Internal RAM test

[E]

Keyboard test.  Starting with [A], press all the keys in order,

left to right, top to bottom.  If all goes well, the HP will

show "KBD1 OK".

[F]

Partial keyboard test

[G]

ESD test monitor.  Battery status is shown by bars.

[H]

UART loop-back test

[I]

Wired UART echo

[J]

Shows what cards are plugged in.  Press any key a few times.

[K]

Test RAM cards in ports

[L]

Blanks display

[M]

Sends system time from IR port

[N]

Receives system time from IR port (have another HP send it)

[O]

IR loop-back test

[P]

IR UART echo

[S]

Shows test start time

[T]

Shows test fail time

[U] [V] [W] [X] [Y] [Z]

Looping test

[ENTER]

Initialize test times

[DEL]

Test summary

[<-]

(Backspace) Enters memory scanner (SX only)

When the HP48 is in test mode (ON-D or ON-E), the test results that are displayed on the screen are also sent to the serial port at 9600 baud, 8 bits, no parity, 1 stop bit.  This seems to be unaffected by any settings in IOPAR, PRTPAR, or the system flags.  I can only guess that this feature was included for factory testing purposes.  This does explain, however, why the I/O annunciator lights up so much during the self test.

Excerpt from Andre Schoorl’s <aschoorl@engr.uvic.ca> "HP48 Frequently Asked Questions List (FAQ)", by Ricardo Miller.

The latest version of the faq can be downloaded at http://www.engr.uvic.ca/~aschoorl/faq/