PRE-RELEASE VERSION, CONTACT THE AUTHOR BEFORE DISTRIBUTING

-=---------------------------------------------------------------------------=-

Guide to the Saturn Processor

(With HP48 Applications)

Compiled by Matthew Mastracci

Rev. 1.0b

-=---------------------------------------------------------------------------=-

(c) Copyright 1995,96,97,98 by Matthew Mastracci

All Rights Reserved

-=---------------------------------------------------------------------------=-

Table of Contents

-=---------------------------------------------------------------------------=-

1.0 Introduction

1.1 Abstract

1.2 DISCLAIMER

1.3 Acknowledgements

1.4 Revisions

1.5 History of the Saturn Chip

2.0 Chip Information

2.1 Chip Specifications

2.2 Registers

2.3 Interrupts

2.4 Daisy-Chain Address Configuration

2.5 Yorke Chip

2.6 Internal 8-bit CRC

3.0 Instruction Set

3.1 Instruction Field Selection

3.2 Generic Register Instructions

3.3 Program Control Instructions

3.4 Chip-Interface Instructions

4.0 HP48GX/SX-Specific Peripheral Control

4.1 Hardware Registers

4.2 Display Driver Interface

4.3 Plug-In Card Interface

4.4 Bank Switching

4.5 UART Communication Controller

4.6 Serial Communication

4.7 IR LED/Receiver

4.8 Internal Speaker

4.9 Keyboard Interface

4.10 Timers

4.11 Miscellaneous

5.0 HP48 PCB Diagrams

5.1 Calculator Overview

5.2 Pinout Reference

5.3 74HC174: Hex D-type flip-flop with clear

5.4 74HC00: Quad 2-input NAND gate

5.5 RAM/ROM Memory

5.6 Ports

5.7 LCD Drivers

5.8 Keyboard

5.9 LCD Display

5.10 Yorke Chip

5.11 Printed Circuit Board

5.12 IR Receiver

5.13 IR Transmitter for the G/GX

5.14 IR Transmitter for the S/SX

5.15 RS232 Circuits

5.16 Power Circuits

5.17 Backup Power

5.18 Notes

6.0 How Do I...

6.1 ...Use Greyscale Graphics?

6.2 ...Output a Solid Tone?

6.3 ...Read the Keyboard Directly?

6.4 ...Speed Up Time-Critical Calculations?

6.5 ...Send text through the serial port?

6.6 ...Send text through the IR port?

6.7 ...Control the IR transmitter/receiver directly?

6.8 ...Create My Own Interrupt Handler?

7.0 Glossary

8.0 Contributing/Contacting

8.1 Contacting the Author

8.2 Contacting Contributors

-=---------------------------------------------------------------------------=-

1.0 - Introduction

-=---------------------------------------------------------------------------=-

1.1 - Abstract

The purpose of this guide is to be the most comprehensive and detailed

guide to the operation of the Saturn processor, useful for anyone trying to

write and optimize ML code, use some of the chip tricks or just to gain an

understanding of how the chip works.

1.2 - DISCLAIMER

This document is provided as is, with no warranty of any kind, either

expressed or implied.  You are free to copy and distribute the document

freely, in any format, provided no changes or additions are made to the text.

Matthew Mastracci or any contributing authors or sources shall in no event be

held liable to you or others for damages of ANY kind, incidental or

consequential, arising from the use or inability to use the information

contained herein.

WARNING: Improper experimentation with Saturn-architecture chips will cause

them to CATCH FIRE AND EXPLODE, KILLING YOU INSTANTLY.  Well, maybe not.  In

any case, be careful with undocumented hardware registers or ones that deal

with the LCD voltage in any way.  If you keep away from these, you shouldn't

have to worry about shelling out another $150 for a replacement calc.  Also,

opening your calculator's case is a DANGEROUS ACTIVITY and is in NO WAY

suggested by the author of this document, any of the contributors or Hewlett

Packard.  Don't try it if you don't know what you're doing!

1.3 - Acknowledgements

To create a guide like this, information was taken from many sources

published in many different formats.  The author would like to acknowledge

the following authors and sources for contributing directly or indirectly to

this project:

Alonzo Gariepy          Controlling the Beeper/HP28S Processor Notes

Dan Allen               HP Calculator History

Derek S. Nickel         Saturn Instruction Set Guide

Detlef Mueller          Speed Up Your HP48

Douglas R. Cannon       S/SX and G/GX Programming Tips

Eddie C. Dost           x48 Source

Gary Friedman           Plug-In Card Pinouts

Hewlett Packard         HP48 Software Development Tools

Jan Brittenson          UART Basic Usage

Joe Ervin               HP48SX Keyboard Input in ML

Mika Heiskanen          Sorted Entries Listing

Philippe Teuwen         HP48G/GX PCB Reference

Regis Duchesne          Les interruptions de la HP48

Dave Arnett             His zillions of informative postings

-and-

Various posters to comp.sys.hp48

Much thanks to the following people, who have offered large amounts of

written material and corrections:

Detlef Mueller

Martin Prajza

Philippe Teuwen

Eric Rechlin

Please notify the author of any errors or omissions.

1.4 - Revisions

The document has gone through the following revisions:

Revision    Date      Changes

0.00a-0.99  ??/??/??  - Early pre-release versions, incomplete

1.0         16/02/98  - A lot of information has been revamped and rewritten

1.0b        20/02/98  - Fixed many spelling mistakes, thanks to Eric Rechlin

1.5 - History of the Saturn Chip

The Saturn Microprocessor has been the core processor unit of many

calculators for the past ten years.  Hewlett Packard has used it for each and

every one of its most recent calculators to create powerful engineering

devices without sacrificing affordability.  The following list shows the

evolution of the Saturn chip as parts of Hewlett Packard calculators:

Calculator        Release Date          Chip Version     Analog/Digital IC

HP71B (early)     02/01/84              1LF2              -

HP71B (later)     ??/??/??              1LK7              -

HP18C             06/01/86              1LK7              -

HP28C             01/05/87              1LK7              -

HP17B             01/04/88              1LT8             Lewis

HP19B             01/04/88              1LT8             Lewis

HP27S             01/04/88              1LT8             Lewis

HP28S             01/04/88              1LT8             Lewis

HP48SX            03/16/91              1LT8             Clarke

HP48S             04/02/91              1LT8             Clarke

HP48GX            06/01/93              1LT8             Yorke

HP48G             06/01/93              1LT8             Yorke

HP38G             09/??/95              1LT8             Yorke

-=---------------------------------------------------------------------------=-

2.0 - Chip Information

-=---------------------------------------------------------------------------=-

All information contained in this section relates to the Saturn processor

as installed in Hewlett Packard calculators (either in general or dealing with

the HP48 series), including any coprocessors or peripherals that may be

attached to it.  Note that most of this information relates to any processor

with Saturn architecture.

2.1 - Chip Specifications

Universal Specifications

Data path width:       4 bits (nibble)

Maximum address width: 20 bits

Logical address space: 512kb (or 1024 kilonibbles)

Maximum register size: 64 bits (working and scratch registers)

HP48S/SX Specific

Integrated into: Clarke IC

Clock speed: 2Mhz

ROM size: 256kb

RAM size: 32kb (S, expandable to 128kb) or 128kb (SX)

Card ports: 2

Port 1: up to 128kb

Port 2: up to 128kb

HP48G/GX Specific

Integrated into: Yorke IC

Clock speed: ~4Mhz (varies with temperature)

ROM size: 512kb

RAM size: 32kb (G, expandable to 128kb) or 128kb (GX)

Card ports: 2

Port 1: up to 128kb

Port 2: up to 128kb accessible simultaniously, up to 4Mb with bank switching

2.2 - Registers

The Saturn processor has 19 registers:

- four 64-bit working registers, A, B, C and D;

- five 64-bit scratch registers, R0, R1, R2, R3 and R4;

- two 20-bit data address registers, D0 and D1;

- one eight-level, 20-bit FIFO stack, RSTK;

- one 4-bit pointer register, P;

- one 20-bit program counter register, PC;

- one 16-bit input register, IN;

- one 12-bit output register, OUT;

- one 1-bit carry flag;

- one 16-bit status register, ST; and

- one 4-bit hardware status register, HST.

Working Registers (A, B, C, D)

Most of the Saturn opcodes operate between two working registers or a

working register and another register.  The working registers are used for the

majority of all calculations and also as parameters for a lot of ML-code

subroutines.  Each of the working registers is 64-bits long, but can be

accessed and operated on in smaller parts with field-select specifiers.

Scratch Registers (R0, R1, R2, R3, R4)

The scratch registers are 64-bits long like the working registers, but

do not have many operations other than assignment to manipulate them.  They

are used to temporarily hold register contents in complex operations or to

act as additional parameters to some ML-code subroutines.

Address Pointer Registers (D0, D1)

The address pointer registers hold 20-bit addresses.  They are used mainly

for reading from and writing to memory, either directly or indirectly.

Return Stack (RSTK)

The return stack has a 20-bit levels for storing return addresses and

pushed register values.  Because the interrupt system requires a single level

for saving variable values, the stack must be used very carefully.

Pointer Register (P)

The pointer register can be used as a field selector for the working

registers.  It is often used for efficient program loops with small numbers

(many of its opcodes are two or three nibbles long).

Program Counter (PC)

The program counter contains the 20-bit address of the next instruction

to be executed by the CPU.  It is incremented by the chip after reading each

instruction.

I/O Registers (IN, OUT)

The input and output registers are 16 and 12 bits long, respectively.

They are used for communication with the system bus and to control any

peripherals connected to the chip.  The IN register is read-only and the OUT

register is write-only.

Carry Flag

The carry flag is used for program control.  In arithmetic operations, it

indicated whether a result has overflowed, underflowed, carried or borrowed.

In test operations, it indicates that the test is true.

Flag Register (ST)

The flag register is 16-bits long, representing 16 different flags.  The

top four bits are usually used to control the state of the operating system,

while the other 12 are used to control the state of the running program or

subroutine and can be accessed as a single register.

Hardware Status Register (HST)

The hardware status register contains four bits that represent the state

of the Saturn processor.  They can only by set by external events, so testing

for a particular state involves clearing it first.  Each of the individual

bits can be tested for and operated on by itself, however.  In order of least

to most significant, the bits are:

- XM: e(X)ternal (M)odule missing

This bit is set by the RTNSXM instruction.  Since the internal

representation of RTNSXM is 00, this bit is effectively set when a

gosub jumps to a nonexistent memory address.

- SR: (S)ervice (R)equest

This bit is set if a service request from the SREQ? instruction is

pending on the system bus.

- SB: (S)ticky (B)it

This bit is set when a non-zero bit is shifted off the right end of

a working register by a shift operation.

- MP: (M)odule (P)ulled

This bit is set whenever the *NINTX line is pulled low, regardless

of whether an interrupt is executed or not.

2.3 - Interrupts

When an interrupt is executed on the Saturn chip by an interrupt signal,

the chip disables interrupts, saves the program counter register on the return

stack and executes a jump to the memory location 0000Fh.  It also sets an

internal flag indicating that no more interrupts are to be serviced until a

RTI (return, reset interrupts) instruction is executed.  All interrupts that

occur during this state will cause the chip to flag an internal pending

interrupt flag, causing it to repeat the interrupt handling routine before

returning.  The first task of the ROM interrupt handler is to set bit 14 of

ST, meaning that there are pending interrupts left to be serviced if the

routine terminates prematurely (see below).  Because the OUT register is

write-only, the chip cannot save its state and therefore the ROM routine

expects that the OUT register be shadowed in RAM for recovery.

Some of the actions that can cause maskable interrupts are:

- Character received by the UART

- Character placed in the UART transmit holding register

- Keyboard button down/repeat

- Timer expiry

- Low battery condition

- IR emission/receipt

The actions that cause non-maskable interrupts (NMI) are:

- ON-key press

- VLBI (very low battery interrupt)

The ROM interrupt handler at 0000Fh must examine each of these

possibilities and act accordingly.

Disabling Interrupts

To disable interrupts completely, bit 15 of the status register must be

cleared.  If the interrupt handler finds this bit set, the interrupt routine

will immediately return without any interrupt processing to the previous

location.  Because the routine did not return with RTI, however, the chip

believes that it is still within the interrupt routine and will ignore all

interrupts until they are reenabled with RESET or RTI.  They will not be

serviced until bit 15 of ST is restored, however.

CAUTION: NO INTERRUPTS WILL BE SERVICED IF BIT 15 IS SET!

Use this method with extreme caution.  The ON, ON-C and ON-A-F key

combinations will not affect the program.  Only the RESET button, located

under one of the rubber feet of the calculator and connected to the reset line

of the chip, will do anything.  The consequence of this is that a low-power

state will not come to the attention of the interrupt handler.  If a program

is allowed to run continuously in this state, the calculator will not be able

to enter the battery-saving deep sleep and the batteries may be completely

drained, causing complete memory loss.

To disable the automatic CPU keyboard scan every 1ms, execute an INTOFF

instruction.  This will in turn disable the 16Hz key-repeat scan, which can

deplete system resources dramatically.

Interrupt Flags

ST Bit   Description

12       Set if DeepSleep should stay awake (forced wakeup request)

13       Set if interrupt has occurred (latched, clear to test)

14       Set if interrupt pending, cleared if no interrupts pending

15       Set if interrupts enabled, cleared if all interrupts disabled

These flags are similar to the internal Saturn interrupt flags, as

interrupts not serviced because of the cleared interrupt enable flag will

cause the pending interrupt flag to be set.

2.4 - Daisy-Chain Address Configuration

Owing to the 20-bit address space, there is half a megabyte of memory

available for use by all modules connected to the Saturn chip.  Each module

in use has a certain window in this 20-bit address space in which it can be

accessed.  In the HP48, these modules are:

S/SX                      G/GX

Device   Description               Description

ROM      ROM address space         ROM address space

HDW      Hardware registers        Hardware registers

RAM      RAM address space         RAM address space

CE1      Port 1 control            Bank select

CE2      Port 2 control            Port 1 control

NCE3     Unused                    Port 2 control

To divide the memory up, the Saturn uses a technique called daisy-chain

configuration.  This involves configuring each module in a specific order as

to which address window it takes up.  Each window must be at least four

kilonibbles (two kilobytes).  The opcodes used for configuring the modules

are:

RESET

Unconfigures all of the modules, places RAM at the default address.

CONFIG

Configures a module, using data in the C register.  For most of the

modules, two CONFIG instructions must be executed to completely configure it:

1)  The first CONFIG call specifies the size of the module's address space in

nibbles.  C is loaded with #100000 minus the value.  The size must be a

multiple of #00100 and must be greater than 4 kilonibbles (#01000).  The

hardware registers do not require a size CONFIG call and therefore the first

and only CONFIG call in this situation will be for its address.

2)  The second CONFIG call specifies the logical address of the module's

address space.  This logical address must be a multiple of #00100 and will

correspond to the physical address space of the module.

The devices are configured in the order of: ROM (always configured), HDW,

RAM, CE1, CE2, NCE3.  If two addresses overlap, however, the priority for

which device occupies the address space is: HDW, RAM, CE2, CE1, NCE3, ROM.

UNCNFG

Unconfigures a module.  The module is specified by placing its initial

logical address value in the address field of C.

C=ID

Gets the ID of the next module to be configured and the last

configuration address into the address field of the C register like so:

Bits (MSB to LSB)     Description

xxxxxxxxxxxx          Most significant three nibbles of last configuration

address specified

y         1 if the next CONFIG data will specify an address; 0 if

it will specify a size.

zzzzzzz  The ID of the next module to be configured.

If all modules are configured, the result in C will be #00000.  The ID

numbers returned are as follows:

ID      Module

#01     NCE3

#03     RAM

#05     CE1

#07     CE2

#19     HDW

2.5 - Yorke Chip

In the HP48G series, the Saturn processor is integrated onto a mixed

analog and digital chip called the Yorke.  The Yorke processor has an 8-bit

data bus and a 19-bit address bus structure.  See the PCB section for

information on the connections to the various peripherals.

2.6 - Internal 8-bit CRC

The CRC is updated every time a nibble is read from memory.  To CRC a

block of memory, simply write zero into the CRC hardware register at #00104h

and #00105h and read all of the bytes you wish to CRC.  The CRC hardware will

take the bytes straight of the bus and the register will contain the new CRC.

Remember to disable interrupts, as the interrupt handler will corrupt the CRC

value.  The CRC is calculated like so:

Saturn CRC Formula

NewCRC = (OldCRC shr 4) xor (((OldCRC xor NibbleValue) and #0Fh) * #1081h)

C Equivalent

crc = (crc >> 4) ^ (((crc ^ nibble) & 0xf) * 0x1081);

Pascal Equivalent

CRC := (CRC shr 4) xor (((CRC xor Nibble) and $0f) * $1081);

-=---------------------------------------------------------------------------=-

3.0 - Instruction Set

-=---------------------------------------------------------------------------=-

Note: for an excellent detailed explanations and timings of all Saturn

instructions, refer to the Saturn Assembly Language Guide packaged with

Hewlett Packard's HP48 Software Development Kit (SASM.DOC).

3.1 - Instruction Field Selection

Saturn instructions dealing with working registers can operate on

specific parts or fields of the working registers.  Some of the fields

represent portions of BCD numbers to simplify some real-number calculations.

Field Selectors:

Field   Nibbles   Selection

P       P         Nibble indicated by P register

WP      P-0       Nibbles P through 0, inclusive ("word thru pointer")

XS      2         BCD exponent sign

X       2-0       BCD exponent and exponent sign

S       15        BCD mantissa sign

M       14-3      BCD mantissa

B       1-0       Byte field (8-bit)

A       4-0       Address field (20-bit)

W       15-0      Word (64-bit)

Generally, fields in this document are communicated in one of two ways:

- Full-form, ie: "the address field of C" or "nibbles 3-0 of A"

- Parameter-form, ie: "C(A)" or "A(3:0)"

The first example refers to nibbles 4 to 0 of C and the second refers to

nibbles 3 to 0 of A.

3.2 - Generic Register Instructions

[incomplete]

3.3 - Program Control Instructions

[incomplete]

Opcode: PC=(A)           Instruction: 808C

Indirect jump, where A(A) is the address of the destination address.

Opcode: PC=(C)           Instruction: 808E

Indirect jump, where C(A) is the address of the destination address.

3.4 - Chip-Interface Instructions

The following instructions control the operation of and interface at a

low-level with the Saturn chip:

Opcode: ?HS=0  n         Instruction: 83nyy

Tests if the hardware status register, masked with n, is equal to zero.

Opcode: ?ST=0  n         Instruction: 86nyy

Tests if bit n of ST is cleared.

Opcode: ?ST=1  n         Instruction: 87nyy

Tests if bit n of ST is set.

Opcode: A=IN             Instruction: 802

Copies the value of the IN register to A(3:0).

Opcode: BUSCB            Instruction: 8083

Enters bus command B onto the system bus.  The bus commands are obsolete

in the HP48 series.

Opcode: BUSCC            Instruction: 80B

Enters bus command C onto the system bus.

Opcode: BUSCD            Instruction: 808D

Enters bus command D onto the system bus.

Opcode: C=ID             Instruction: 806

Returns the 5-nibble ID number of the next module to be CONFIGured.  See

the Daisy-Chain Configuration section for more information.

Opcode: C=IN             Instruction: 803

Copies the value of the IN register to C(3:0).  This opcode can only be

executed from an even-nibble address.  Use the =CINRTN entry point to call

it from code.

Opcode: C=RSTK           Instruction: 07

Pops the last return stack address into the address field of C.

Opcode: C=ST             Instruction: 09

Copies the lower 12 bits of ST into C(X).

Opcode: CLRHST           Instruction: 82F

Clears the XM, SB, SR and MP hardware status registers.

Opcode: CLRST            Instruction: 08

Clears the lower 12 bits of ST.

Opcode: CONFIG           Instruction: 805

Configures a module connected to the processor.  See the Daisy-Chain

Configuration section for more information.

Opcode: CSTEX            Instruction: 0B

Exchanges the lower 12 bits of ST with C(X).

Opcode: HS=0   n         Instruction: 82n

Generic form of MP/SB/SR/XM=0 opcodes.  Clears HST with mask n.

Opcode: INTON            Instruction: 8080

Enables maskable interrupts.  See the Interrupts section for more

information.

Opcode: INTOFF           Instruction: 808F

Disables maskable interrupts.  See the Interrupts sections for more

information.

Opcode: NOP3             Instruction: 420

Represents three "no operation" opcodes.  The chip will skip over these

three nibbles.

Opcode: NOP4             Instruction: 6300

Represents four "no operation" opcodes.  The chip will skip over these

four nibbles.

Opcode: NOP5             Instruction: 64000

Represents five "no operation" opcodes.  The chip will skip over these

five nibbles.

Opcode: OUT=C            Instruction: 801

Copies C(X) into the OUT register.

Opcode: OUT=CS           Instruction: 800

Copies C(0) to the lowest nibble of the OUT register.

Opcode: RESET            Instruction: 80A

Resets the address space of all modules connected to the processor.  See

the Daisy-Chain Configuration section for more information.

Opcode: RSTK=C           Instruction: 06

Pushes the address field of C onto the return stack.

Opcode: RSI              Instruction: 80810

Resets the interrupt-detection ability of the Saturn chip.

Opcode: RTI              Instruction: 0F

Returns from an interrupt routine and enables interrupt detection.

Opcode: SETDEC           Instruction: 05

Sets the chip into DEC mode.  Controls the operation of the carry flag

and affects the results of some arithmetic operators.

Opcode: SETHEX           Instruction: 04

Sets the chip into HEX mode.  Controls the operation of the carry flag

and affects the results of some arithmetic operators.

Opcode: SHUTDN           Instruction: 807

Stops the CPU at this instruction until a wake-up is requested.

Opcode: SREQ?            Instruction: 80E

Copies the a service request response from the system bus to C(0).  Sets

SR=1 if the request response exists.  The bits of C(0) set represent the

device that requests service:

Bit  Device

0    Display Driver (timer)

1    HP-IL Mailbox

2    Card Reader

3    Unused

Opcode: ST=0   n         Instruction: 84n

Clears bit n of ST.

Opcode: ST=1   n         Instruction: 85n

Sets bit n of ST.

Opcode: ST=C             Instruction: 0A

Sets the lower 12 bits of ST equal to C(X).

Opcode: UNCNFG           Instruction: 804

Unconfigures a module connected to the processor.  See the Daisy-Chain

Configuration section for more information.

-=---------------------------------------------------------------------------=-

4.0 - HP48GX/SX-Specific Peripheral Control

-=---------------------------------------------------------------------------=-

4.1 - Hardware Registers

For most of the devices below, hardware registers are used to control

many operational aspects.  Writing to one of the hardware registers, mapped

between the memory locations #00100h and #0013F, will affect the device

attached to it.  Note that some flags occupy only a single nibble within the

register, so care must be taken to preserve the states of the other bits.

Also, some registers are read-only or write-only, indicated by (R/O) and

(W/O), respectively.

4.2 - Display Driver Interface

The LCD attached to the HP48 is 131 pixels horizontally by 64 pixels

vertically.  In the HP48SX, the display is refreshed at 64Hz.  This causes

some flicker to occur when viewing the display under fluorescent lights in

North America, as power is regulated at 60Hz.

The following registers affect the LCD display:

Register   Bits   Description

#00100h    0-2    Control the eight-pixel vertical offset of the display

3      Display-enable flag

#00101h    All    Four LSBs of the five-bit contrast control value

#00102h    0      MSB of the contrast control value

1-3    Display test control

#00103h    All    Display test control

3      Enable no-refresh-mode (possibly dangerous!)

#0010Bh    0      Left-shift indicator

1      Right-shift indicator

2      Alpha indicator

3      Alert indicator

#0010Ch    0      Busy indicator

1      I/O indicator

2      XTRA - Reserved for future use

3      Indicator-enable flag

#00120     All    LSBs of the five-nibble display address base register (W/O)

...

#00124     All    MSBs of the display address base register

#00125     All    LSBs of three-nibble display line offset register (W/O)

...             (right margin of screen)

#00127     All    MSBs of the display line offset register

#00128     All    LSBs of vertical line count (R/O)

...

#00129     0-1    MSBs of vertical line count (R/O)

2      M32

3      DA19 Control (set is card port 2, cleared is upper ROM)

#00128     All    LSBs of screen height not including menu (W/O)

...

#00129     0-1    MSBs of screen height not including menu (W/O)

#00130     All    LSBs of menu GROB address (W/O)

...

#00134     All    MSBs of menu GROB address (W/O)

Note that registers #00128 and #00129 have many purposes.  When reading

from them, they return the current LCD retrace row (with DA19 and M32 in the

upper two bits).  When writing, they control the number of rows drawn of the

main screen grob before switching to the menu grob (again with DA19 and M32

controlled by the upper bits).

The display address base register at #00120 will only use even

addresses.  If you supply an odd address, it will zero the LSB and use the

result as the new base.

4.3 - Plug-In Card Interface

Both the HP48GX and HP48SX have two card ports.  Both of these

calculators use a modified version of the Seiko/Epson 40-pin card interface.

On the HP48GX, however, card port 2 can support a card of 32 x 128kb banks,

equaling 4Mb, through a technique called bank-switching.

The following hardware registers affect the plug-in card interface:

Register   Bits   Description

#0010Eh    0      Software Interrupt

1      Set Module Pulled

2      Run Card Detect

3      Enable Card Detect

#0010Fh    0      Card present in port 2  [note the reversal]

1      Card present in port 1

2      Writing on port 2 allowed

3      Writing on port 1 allowed

The pinouts on the card interface are as follows:

Pin    Signal   I/O    Description

1      VDD      Power  Power

2      VBB      Out    Card battery check (if Vbb is low, the calculator will

signal "low battery")

3      A0       In     Pins 3-19, 31, and 32 are the address lines (pin 32 is

inverted)

...

19     A16      In     Address line 16

20     NWE      In     Not write enable (write protect)

21     CE       In     Card Enable

22     NOE      In     Not output enable

23     D0       In/Out Pins 23-30 are the eight data lines

...

30     D7       In/Out Data line 7

31     A17      In     Address line 17 (for port 2)

32     NA18     In     Address line 18 (for port 2)

33     XSCL     In     Display data clock (A19 for port 2)

34     LP       In     Display data horizontal sync (A20 for port 2)

35     LD[0]    In     Display data (A21 for port 2)

36     LD[1]    In     Display data (BEN: Bank Switching Enabled for port 2)

37     CDT      Out    Card detect/type, must be tied high for writes to

operate.

38     NC              No connection [write-protect in Seiko/Epson]

39     NC              No connection [card-present in Seiko/Epson]

40     GND      Power  Ground

Note: In refers to input into the card and Out refers to output to the card.

The CDT line (pin 37) helps the calculator determine which of the

following states applies:

- Empty: the CDT pin is open

- ROM: the CDT pin is low (all write operations are disabled)

- RAM: the CDT pin is tied high and the card passes the RAM size test

- UNKNOWN DEVICE: the CDT pin is high and the card fails the RAM size test

(user-level reads and writes are disabled as if nothing were present, but

machine level reads and writes work on the memory)

In a normal Epson/Seiko card, pins 37 and 38 would represent the card's

write-protect and present state, respectively.  These two are represented by

the CDT line and so they serve no purpose on the HP48.

XSCL, LP, LD[0] and LD[1] are responsible for controlling overhead

projector displays.  The data is sent through LD[0] and LD[1] in two serial

bit streams in sync with the shift clock XSCL.  The LP signal provides

horizontal sync control.  For each row, 131 columns of display data are sent

left to right, contained in the 64 shifts before each LP pulse and the two

shifts immediately following the LP.  There is no vertical sync.  An overhead

projector interface must be plugged into port 1, as the four pins used to

control the display are converted into address lines in port 2.

4.4 - Bank Switching

(HP48GX only)

Bank switching on the HP48GX is handled through the CE1 controller, which

controls the external latch (the HC174).  It is usually configured at the

address of #7F000.  By reading a value from somewhere in logical memory space

assigned to the bank manager, one of the banks on the card will be activated

and can be then read from.

To Select a Bank

D0=#7F000 + #40 + 2 * n   Where n is the bank you wish to select, from 0-31

C=DAT0 B                  Throw away the byte just read

When the byte is read from the bank manager, the state of the latch chip

on the circuit board is changed so that the correct bank address is presented

on the memory card.  The data read from the bank manager is useless, but the

last few bits of address from which you read are transferred to the bank

address latch chip.  To access different banks on card port 2, the BEN line

must be high (by adding a value of #40).  The physical address of the bank

manager that you read from represents:

Bits (MSB to LSB)   Description

x                   BEN (Bank-switching Enabled)

yyyyy              Active bank select, 0-31

z             Not used (comes from nibble to byte conversion)

Because of the configuration of the Yorke IC inside the calculator, you

can access only card port 2 or upper ROM at any one time.  Upper ROM (NMA18)

and card port 2 (NCE3) are both controlled by pin 85 of the Yorke IC.  The two

functions are multiplexed onto pin 85.  DA19, in the control register,

controls the multiplexer and determines which is active.  DA19 is mirrored

into the BEN bit of the external latch (bank address latch chip), on bit 6

and can be controlled with bit 3 of hardware register #00129 (also used for

display purposes).  The lower five bits of the external latch represent the

active bank.

Upper ROM and card port 2 are mutually exclusive.  You cannot access any

instructions or data located in the upper half of ROM when card port 2 is

enabled.

To Enable Card Port 2 (disabling upper ROM)

1.  Set DA19 so that upper ROM is disabled and the NCE3 controller (card port

2) is enabled

2.  Read from an address in the bank manager so that bit 6 of the external

latch (BEN) is high (remember to keep bit 6 high when switching banks)

To Disable Card Port 2 (enabling upper ROM)

1.  Set the external latch so that bit six is a 0

2.  Set DA19 for ROM access

When programming the HP48GX, remember that card port 2 is only enabled

when it is required.  Ensure that when the card port is not going to be

accessed, upper ROM is reenabled.  This will prevent programs that assume

upper ROM is enabled from trying to read from the part that doesn't exist

(likely crashing the machine).

4.5 - UART Communication Controller

The UART controller in the HP48 is capable of handling I/O throughput of

up to 9600 baud.  It has two single-byte holding registers for reading and

writing and will send output to the active I/O device (either IR or the serial

port).

The following hardware registers affect the UART controller:

Register   Bits   Description

#0010D     0-2    3-bit baud rate register

Values are: 1200 1920 2400 3840 4800 7680 9600 15360

3      UART Clock (R/O)

#00110     All    UART Interrupt Control Register

0      Enable interrupt on RECV buffer receiving

1      Enable interrupt on RECV buffer full

2      Enable interrupt on XMIT buffer empty

3      Wire serial port enabled

#00111     All    UART RECV Control Register

0      Character present in receive buffer

1      UART receiving character

2      Error occurred while receiving

3      Not used

#00112     All    UART XMIT Control Register

0      XMIT buffer contains unsent character

1      XMIT is transmitting character in buffer

2      LPB

3      Break received

#00113     All    Clear receive error if written to (W/O)

#00114     All    LSBs of RECV holding buffer character (R/O)

#00115     All    MSBs of RECV holding buffer character (R/O)

#00116     All    LSBs of XMIT holding buffer character (W/O)

#00117     All    MSBs of XMIT holding buffer character (W/O)

Receiving

The interrupts associated with the serial port are controlled by register

#00110h.  When the first bit of a character is received, an interrupt will be

generated if bit 0 of #00110 is set and bit 1 of #00111 will be set.  When the

character has been received, bit 1 of #00111 will be cleared and bit 0 will

then be set.  If bit 1 of #00110 is set, an interrupt is generated.  The

interrupt routine captures the received character and sets bit 0 of #00111

to zero, indicating that the buffer is now ready to receive again.

Transmitting

When a character is written to the XMIT holding register, bit 0 of

#00112 is set.  The UART then clears bit 0 and sets bit 1, indicating that

the data is now being sent.  Once the UART has finished transmitting the

character, bit 1 is cleared.  If bit 2 of #00110 is set, an interrupt is

generated, which then puts the next byte into the XMIT holding buffer and

sets bit 0 of #00112.

4.6 - Serial Communication

The serial interface of the calculator is located in the center of the

front IR cover.  There are four pins in the jack, representing from right to

left:

- Shield: electrostatic protector

- TX: transmit (to other)

- RX: receive (from other)

- GND: ground

These are connected to pins 1, 3, 2 and 7 of a 25-pin jack and to the

casing and pins 2, 3 and 5 of a 9-pin jack, respectively.

4.7 - IR LED/Receiver

The IR LED and receiver are located on the front of the calculator, to

the right of the serial port (when viewed from the front).  The LED bulb is

on the left and the receiver assembly in on the right.  Note that IR I/O is

half-duplex, as reflections from the IR assembly cover can introduce feedback

into the system.

The following hardware registers affect the IR setup:

Register   Bits   Description

#0011A     All    IR Control register

0      IR interrupt occurred

1      IR interrupts enabled

2      Direct IR LED control disabled (RS232, UART-controlled mode)

3      Latched IR sampling bit/IR being received

#0011C     All    IR Status register

0      IR buffer full

1      IR buffer empty

2      IR empty-buffer interrupts allowed

3      IR LED enable flag

#0011D     0      LED buffer

1-3    (zeroes)

4.8 - Internal Speaker

The internal speaker of the HP48 is controlled through the upper two bits

of the 12-bit OUT register.  By setting the register to one of the following

values, the speaker can be controlled:

OUT value      Effect

#0xx           No speaker power (0)

#8xx           Positive displacement (+)

To generate steady tones, simply alternate between the two states of the

speaker, with a delay between them.  The delay is inversely proportional to

the frequency of the sound.  Note that the keyboard can be scanned at the same

time a tone is being generated with minimal overhead.

4.9 - Keyboard Interface

If an INTOFF instruction has not been executed, the CPU will

automatically scan the keyboard for key presses every 1ms.  The ON key is

scanned regardless of the interrupt state, however, as it is connected

directly to a pin on the processor.  This keyboard scanning is independent of

program operation and does not seriously affect the performance of the system.

If a key is pressed, an interrupt is generated and the ROM keyboard handler

scans the keyboard row by row to determine which keys are pressed.  It then

sets a timer interrupt for 1/16s in the future for the purposes of determining

which keys are held down and which are released, so that it can update the RAM

key buffer.

The keyboard interrupts can dramatically reduce system performance during

intense CPU calculations.  It is wise to use the INTOFF instruction to disable

the keyboard press and repeat scan during time-critical areas of code.  If the

code is important enough, the ON interrupt may also be disabled by setting bit

15 of ST to zero.  This will disable ALL interrupts, so that if the program

crashes or hangs, the only way to escape is by pressing the RESET button on

the back of the calculator.  In any case, use caution when disabling all

interrupts.

The keyboard is read row-by-row, by outputting the row identifier with

OUT and then reading the pressed keys with IN.   Note that the scan row does

not necessarily represent the keyboard row.  When a key is pressed, it

completes a circuit between two data lines.  Each bit set in the OUT register

corresponds to a keyboard data-line which is set high.  The bits set in the IN

register represent the output data-lines which are part of complete circuits.

OUT=   IN=#20    #10    #08    #04    #02    #01

#100      n/a    (f2)   (f3)   (f4)   (f5)   (f6)

#080      n/a    PRG    CST    VAR    (up)   NXT

#040      n/a    STO    EVAL   (left) (down) (right)

#020      n/a    COS    TAN    SQRT   POWER  INV

#010      ON     ENTER  +/-    EEX    DEL    BKSP

#008      ALPHA  SIN    7      8      9      /

#004      R-SHFT MTH    4      5      6      x

#002      L-SHFT (f1)   1      2      3      -

#001      n/a    '      0      .      SPC    +

Note that the ON-key is returning in bit 15 of OUT, whether you ask for

it or not.

For instance, if OUT=#020 and the SQRT and INV keys are pressed, the

value of IN would be (#01 or #04) or #05.  The row masks may also be ORed, but

this will only tell whether a key in each specific column was pressed.

Setting OUT=#1FF will return a non-zero value in the IN register if any keys

are currently being pressed.

4.10 - Timers

The following hardware registers affect the timers:

Register   Bits   Description

#0012Eh    All    Timer 1 Control Register

0      Extra function

1      Interrupt

2      Wake

3      Service request

#0012Fh    All    Timer 2 Control Register

0      [TRUN]

1      Interrupt

2      Wake

3      Service request

#00137h    All    Timer 1 value

#00138h    All    Timer 2 value

...

#0013Fh    All    Timer 2 value

TIMER1 is 4-bit clock running at 16Hz used by the interrupt handler to

detect keys being held down and released.  TIMER2 is a 32-bit (8-nibble) wide

clock running at 8192Hz.  The timer value will decrement each cycle, until it

reaches zero, at which time it will perform a function, depending on how it is

set up.

4.11 - Miscellaneous

The following hardware registers affect various aspects of the

calculator:

Register   Bits   Description

#00104h    All    LSBs of 8-bit CRC (see the CRC-calculation section)

...

#00105h    All    MSBs of CRC

#00108h    All    Power Status registers

0      Very-Low Battery Interrupt occurred

1      LowBat0 occurred

2      LowBat1 occurred

3      LowBat2 occurred

#00109h    All    Power Control registers

0      RST

1      GRST

2      Enable Very-Low-Battery Interrupt (VLBI)

3      Enable Low-Battery Interrupt (LBI)

#0010Ah    All    Chip mode (R/O)

#00118h    All    LSBs of service request

...

#00119h    All    MSBs of service request

#0011Bh    All    Base nibble offset

#0011Eh    All    Scratch pad (used by the interrupt handler)

#0011Fh    All    Base nibble (IRAM@, 7 for S/SX, 8 for G/GX)

-=---------------------------------------------------------------------------=-

5.0 - HP48 PCB Diagrams

-=---------------------------------------------------------------------------=-

The information in this section is adapted from version 0.03 of Philippe

Teuwen's incredible PCB documentation.  Most of the diagrams are drawn from

the perspective of the calculator viewed from the back with the batteries at

the bottom.

5.1 - Calculator Overview

-----!!!!----OO-----     Serial port/IR LED/IR receiver

|      &             |

| XXXX XXXXXXXX XXXX |    LCD driver/Processor/LCD driver

| XXXX XYORKEXX XXXX |

| XXXX XXXXXXXX XXXX |    & (4): Jumpers

|      XXXXXXXX &    |

|     &        RX  H |    Crystal (32kHz time X'tal)

|         &    IR  H |    IR RX circuits

RS232    |R                   |

TX      |S ################  |    Expansion connectors    ___________

circuits  |T ################  |    (port 2 above)      ___| port 2    |

|X ################  |                       /   |___________|__

|  !!!!!!!!!!!!!!!!  |    Small wires       /  __| port 1       |

Power    |P                   |                   __/__/__|______________|__

Circuits  |O  74 XXXX XXXX  CC |    RAM/ROM/Condensator 1mF

74HC174  |W  HC RAMX ROMX  HC |    74HC00

|R     XXXX XXXX  00 |

|--------------------|

| +                  |

|                    |    Battery case

|                  - |

-------------------

5.2 - Pinout Reference

A0-A16:    Address lines

A17-A21:   Extended lines for port 2 (A18 is inverted, actually NA18)

AR17-AR18: Extended lines for ROM

BEN:       Bank-switching Enable

buzz:      Buzzer (the second line of the buzzer is grounded)

CDT1-CDT2: Card Detect Type for port 1-2 (H:RAM L:ROM X:No Card)

CE1:       Card Enable 1 (In GX, is used to enable A17-A21 for port 2)

CE2:       Card Enable 2 (In GX, is used as CE for port 1)

CE2.2:     Card Enable port 2 (Not from CPU; CE2.2=BEN*N(AR18) )

D0-D7:     Data lines

GND:       Ground

LD(0):     Display: The first bit of display information

LD(1):     Display: The next bit

LP:        Display horiz sync (Fall. edge: a new line is about to be started)

NC:        Not Connected

NCE0:      Card RAM Enable

NCER:      ?Card ROM Enable

NOE:       Original Output Enable (Not used in G/GX)

NOE2:      Output Enable for all RAMs and ROM (Not from CPU; NOE2=N(NWE) )

NWE:       Write Enable for all RAMs

ON-key:    ON-key is wired directly to CPU

RX:        RS232 Reception

RXir:      IR Reception

SPD:       Processor Speed (H:4MHz L:2.4MHz)

sync1:     ?Display synchro line (between both drivers)

sync2:     ?Display synchro line (between both drivers)

sync3:     ?Display synchro line (between both drivers and the CPU)

sync4:     ?Display synchro line (between both drivers and the CPU)

sync5:     ?Display synchro line (between both drivers and the CPU)

TX:        RS232 transmission

TXir:      IR transmission

V0-V299:   Display data lines for the LCD

Vbb1-Vbb2: Card battery check (L:warning low bat port 1-2)

Vcc(on):   =5V  if HP is turned on else =0V

Vcc(10):   =10V if HP is turned on else =4.5V(bat)

Vcc:       =5V  if HP is turned on else =4.5V(bat)

+4.5V:     Directly wired from battery + pin

x3:        ? (links 84 and 124 of the CPU and power circuits)

x4:        ? (links 128 of the CPU and power circuits)

XSCL:      Display clock (Falling edge: 2 bits of display data may be read)

Xtal1:     To X'tal 32kHz

Xtal2:     To X'tal 32kHz

(GND):     \                              Ground

(RX):       | Pinouts of the RS232 I/O    Reception

(SH):       |                             Shield

(TX):      /                              Transmission

5.3 - 74HC174: Hex D-type flip-flop with clear

-----------

CE1 -| CLK   GND |- GND         NCLR CLK D Q

BEN -| 4Q     3Q |- A17           L   X  X L

A5 -| 4D     3D |- A0            H   ^  H H

A21 -| 5Q     2Q |- A18           H   ^  L L

A4 -| 5D     2D |- A1            H   L  X Q0

A3 -| 6D     1D |- A2

A20 -| 6Q     1Q |- A19       /-- 47k ---- Vcc(on)

Vcc(on)-| Vcc  NCLR |- Vcc 3V --<

----/^\---- 1           \-- 100k --- GND

5.4 - 74HC00: Quad 2-input NAND gate

----\_/---- 14

Vcc(on)-| 1A    Vcc |- Vcc(on)

AR18 -| 1B     4B |- Vcc(on)     Logic:   Y=N(A*B)

4(HC00) /-| 1Y     4A |- NWE

3(HC00) \-| 2A     4Y |- NOE2        Results: CE2.2=BEN*N(AR18)

BEN -| 2B     3B |- 6(HC00)              NOE2=N(NWE)

10(HC00) -| 2Y     3A |- Vcc(on)

GND -| GND    3Y |- CE2.2

-----------

5.5 - RAM/ROM Memory

RAM 32k or 128k                      ROM 512k

-----------                       -----------

D3 -| D3    GND |- GND            D3 -|           |- GND

D4 -| D4     D2 |- D2             D4 -|           |- D2

D5 -| D5     D1 |- D1             D5 -|           |- D1

D6 -| D6     D0 |- D0             D6 -|           |- D0

D7 -| D7     A0 |- A0             D7 -|           |- A0

NCE0 -| NCE    A1 |- A1           NCER -|           |- A1

A10 -| A10    A2 |- A2            A10 -|     ?     |- A2

GND -| NOE    A3 |- A3            GND -|           |- A3

A11 -| A11    A4 |- A4            A11 -|           |- A4

A9 -| A9     A5 |- A5             A9 -|           |- A5

A8 -| A8     A6 |- A6             A8 -|           |- A6

A13 -| A13    A7 |- A7            A13 -|           |- A7

NWE -| NWE   A12 |- A12           A14 -|           |- A12

Vcc -|CE/Vcc A14 |- A14          AR17 -|           |- A15

28 ....:^:.... 1   if 32k     AR18 -|           |- A16

Vcc(on)-|           |- GND

A15 -| A15   A16 |- A16              32 ----/^\---- 1

Vcc -| Vcc    NC |- GND

32 ----/^\---- 1   if 128k        Address: 512k=2^19 A0-AR18

5.6 - Ports

Port 1        Port 2

-----------------------

1 Vcc(on)       Vcc(on)

2 Vbb1          Vbb2

3-19 A0-A16        A0-A16

20 NWE           NWE

21 CE2           CE2.2

22 NOE2          NOE2

23-30 D0-D7         D0-D7

31 AR17          A17

32 AR18          A18

33 XSCL          A19

34 LP            A20

35 LD(0)         A21

36 LD(1)         BEN

37 CDT1          CDT2

38 NC            NC       In Seiko-Epson Cards: Card Present

39 NC            NC                             Card Type

40 GND           GND

5.7 - LCD Drivers

Refs: SED1181Fla Japan

Left(reversed)  Right

-----------------------------

1-28 V254-V281       V222-V249

29 sync2           NC

30 NC              NC

31 NC              NC

32 LD(0)           sync1

33 LD(1)           sync2

34 XSCL            XSCL

35 LP              LP

36 sync3           sync3

37 V87             V52

38 V86             V51

39 V85             V50

40 V83             V49

41-50 V82-V73         V47-V38

51 V71             V37

52 V70             V36

53-62 V69-V60         V34-V25

63 V58             V24

64-68 V57-V53         V22-V18

69 sync4           sync4

70\sync5           sync5

71/sync5           sync5

72 Vcc             Vcc

73 Vcc(10)         Vcc(10)

74 sync1           NC

75 NC              NC

76 NC              NC

77-80 V250-253        V218-221

5.8 - Keyboard

Refs: MXS 00048-80038

! on the other side of the PCB, so X'tal is on the left

1 Vcc           7 A3           13 A1

2 ON-key        8 A11          14 A15

3 A5            9 A12          15 A16

4 A4           10 A2           16 A0

5 A10          11 A13          17 AR17

6 A9           12 A14

Pressing a key makes a connection between 2 lines.

Correspondence Table:

-----------------------------------------------

| A     | B     | C     | D     | E     | F     |     ML correspondences:

|-------|-------|-------|-------|-------|-------|

| O A10 | O AR17| O AR17| O AR17| O AR17| O AR17|     OUT: #001 -> A9

| I A4  | I A4  | I A3  | I A2  | I A1  | I A0  |          #002 -> A10

|-------|-------|-------|-------|-------|-------|          #004 -> A11

| MTH   | PRG   | CST   | VAR   | up    | NXT   |          #008 -> A12

|-------|-------|-------|-------|-------|-------|          #010 -> A13

| O A11 | O A16 | O A16 | O A16 | O A16 | O A16 |          #020 -> A14

| I A4  | I A4  | I A3  | I A2  | I A1  | I A0  |          #040 -> A15

|-------|-------|-------|-------|-------|-------|          #080 -> A16

| '     | STO   | EVAL  | left  | down  | right |          #100 -> AR17

|-------|-------|-------|-------|-------|-------|     IN: #0001 -> A0

| O A9  | O A15 | O A15 | O A15 | O A15 | O A15 |         #0002 -> A1

| I A4  | A A4  | I A3  | I A2  | I A1  | I A0  |         #0004 -> A2

|-------|-------|-------|-------|-------|-------|         #0008 -> A3

| SIN   | COS   | TAN   | sqrt  | Y^X   | 1/X   |         #0010 -> A4

|-------|-------|-------|-------|-------|-------|         #0020 -> A5

| O A12 | O A14 | O A14 | O A14 | O A14 | O A14 |         #8000 -> ON-key

| I A4  | I A4  | I A3  | I A2  | I A1  | I A0  |

|---------------|-------|-------|-------|-------|

| ENTER         | +/-   | EEX   | DEL   | back  |

|---------------|-------|-------|-------|-------|

| O A13         | O A13 | O A13 | O A13 | O A13 |

| I A4          | I A3  | I A2  | I A1  | I A0  |

|---------------|-------|-------|-------|-------|

| alpha         | 7     | 8     | 9     | /     |

|---------------|-------|-------|-------|-------|

| O A12         | O A12 | O A12 | O A12 | O A12 |

| I A5          | I A3  | I A2  | I A1  | I A0  |

|---------------|-------|-------|-------|-------|

| shift left    | 4     | 5     | 6     | *     |

|---------------|-------|-------|-------|-------|

| O A11         | O A11 | O A11 | O A11 | O A11 |

| I A5          | I A3  | I A2  | I A1  | I A0  |

|---------------|-------|-------|-------|-------|

| shift right   | 1     | 2     | 3     | -     |

|---------------|-------|-------|-------|-------|

| O A10         | O A10 | O A10 | O A10 | O A10 |

| I A5          | I A3  | I A2  | I A1  | I A0  |

|---------------|-------|-------|-------|-------|

| ON            | 0     | .     | SPC   | +     |

|---------------|-------|-------|-------|-------|

| O +Vcc        | O A9  | O A9  | O A9  | O A9  |

| I ON-key      | I A3  | I A2  | I A1  | I A0  |

-----------------------------------------------

5.9 - LCD Display

Refs: LD-F8845A-23 363D Epson Japan

! on the other side of the PCB, so X'tal is on the left

up: 1-105 : V1-V105

down: 201 : V201

202 : NC

203-297 : V203-V297

298 : NC

299 : V299

5.10 - Yorke Chip

Refs: 00048-80063 D3004GD NEC Japan

1 NC                    90 NWE                137 LD(0)

2 NC                    91 RESET (if low)     138 LD(1)

3 TXir               92-99 D0-D7              139 V88

4-18 V217-V203            100 AR17               140 V282

19 V201             101-117 A16-A0             141 V89

20-35 V16-V1               118 Xtal2              142 NC

36 NC                   119 Xtal1              143 sync5

37 SPD                  120 NC                 144 sync4

38 NC                   121 NOE                145 +4.5V

39 V84                  122 TXir               146 RAM bat2

40 V72                  123 GND                147 RAM bat1

41-56 V90-V105             124 x3                 148 Xtal1

57 V299                 125 Vcc                149 Xtal2

58-72 V297-V283            126 Vcc(on)            150 CDT1

73-80 NC                   127 NCER               151 CDT2

81 RX                   128 x4                 152 V17

82 TX                   129 -1.5k-buzz         153 V23

83 XSCL                 130 Vcc(10)            154 V35

84 x3                   131 TX                 155 V48

85 AR18                 132 GND                156 V59

86 ON-Key               133 RX                 157 V72

87 CE2                  134 XSCL               158 V84

88 CE1                  135 LP                 159 GND

89 NCE0                 136 sync3              160 RXir

5.11 - Printed Circuit Board

Refs: 00048-80050

- Some tracks are interlaced to allow a bridge to be soldered on them.

(cf the 4 "&" on the PCB schematic)

- These jumpers exist to allow an SX to be easily constructed with this PCB.

Left one   :   CE1<->CE2.2

Right one  :   NOE<->NOE2

Middle one :   SPD<->Vcc   CPU at 4MHz  (soldered originally)

Upper one  :   SPD<->GND   CPU at 2.4MHz

CAUTION: Never solder the two last ones at the same time!

5.12 - IR Receiver

Note: schematic adapted from HP I/O Technical Interfacing Guide

+5V

_________________________________________________

|                           |                     |

10K    \                      18K  \               1M    \

1/8 W  / R6                   1/8W /  R8           1/8W  /  R7

\                           \                     \

RXir ----------|                           |                     |

|                            \                    |

|                             \|                  |

\                        Q4   |------------------|

\|           R4             /|                  |

Q3   |----------/\/\/----------V    <---,           |

/|     |   220K            |       '-----       |

V       |   1/8 W           |                    |

+5V _  ____   |       |                   |                    |

||       | 560K  /  R5        220K   /                    |

||       | 1/8W  \            1/8W   \  R3                /

GND  --       |       /                   /                  |/

|       |                   |------------------|  Q2

|       |                   |                  |\

|       |           .022uF ---  C1               V

|       |                  ___                   |

|       |                   |                    |

---------------------------|--------------------

---|---

--|--

-

Q4: Receiver              Q1,Q2,Q3:  2N3904

Refs: EG&G VATEC VTT 9112        R3-R8: 5%

___

| i |

|i|i|

|||

/ | \

E  B  C

5.13 - IR Transmitter for the G/GX

,-->                __

--'                   |  |

+4.5V --A->|-K--- TXir          |__|

||

| \

/   K

A

5.14 - IR Transmitter for the S/SX

Note: schematic adapted from HP I/O Technical Interfacing Guide

+5V

---

Transmit Circuit     |

39Ohms     /

1/8W       \  R1

+5V   __              /

||              |             CR1: NEC SE303A

||            -----  --,           or

GND    -  -----  CR1   \ /    '-->         HP HEMP-3301 (narrower beam)

---

|             R1-R2: 5%

/

OUT            R2        |/

--------------\/\/\------| Q1

---->         500 Ohms   |\

1/8 W        V

-----

---

-

5.15 - RS232 Circuits

Pinout:    I__|__|__|__|__I

|  |  |  |

/   |  |   \

(SH) (TX)(RX) (GND)

1nF

(SH)---| |--- GND

(TX)--------------------------------

|                         |

--- 1nF                   / \

---                      /   \

|          Vcc         |     |         Vcc

--|--         |         \ /   ---         |

-|-          |         ---   / \         |   2 diodes

-           \          |     |          \

47k /          |     |          / 47k

\         /       \         \

|       |/         \|       |

NPN           |-------|           |-------|          PNP

Transistors       |       |\         /|       |      Transistors

ref:1AM         |         V       A         |        ref:2A

\         |     |         /

\|       |     |       |/

|-------|-----|-------|

/|          |          |\

V            \            A

|             / 150ohms     |

|             \             |

|             |             |

|-------------|-------------|--------- TX

|

---

/ \    ref:H2M

|

--|--

-|-

-

5.6k

(RX)----/\/\/--------------- RX

|

-||-   ---  Capacitance diode (Varicap) ref:C10

/ \

|   75ohms

(GND)---------------/\/\/--- Vcc

5.16 - Power Circuits

+4.5V -----SSSSS-------------------------,

:   Self?   |      |               \|         PNP

:   (R=0)   |      |                |---   Transistor

,---'        \ /    \ /              /|   |    ref:YB51

Zener    / \         ---    ---       x4 -->'     |     or YB58

ref:Z3N    :           |      |                    |

omitted on   :           |      |-- Vcc(10)          |

recent PCB's  :          x3      |                    |

:                 XXX  Capa?            |

:                 ???  ref:336          |

:                 ???      16k          |

:                  |       429          |

' ' ' ' ' ' ' ' ' |--------------------

--|--

-|-

-

5.17 - Backup Power

+| |               | |

Vcc -------| |---------------| |-------- Vcc(on)

| |       |       | |

1mF       |      0.1uF

--|--

-|-

-

5.18 - Notes

- If a current larger than ~120mA flows through the CPU, all of the

indicators will be set independently of the HP's state

- See the "Contact" section for information on how to contact the original

author of this document for corrections or clarifications

-=---------------------------------------------------------------------------=-

6.0 - How Do I...

-=---------------------------------------------------------------------------=-

Before experimenting with any of the code or ideas presented in this

section, be sure to backup your data.  Typos can lead to the dreaded "Memory

Clear" message.

6.1 - ...Use Greyscale Graphics?

The HP48 is capable of displaying pseudo-four-color graphics without

seriously affecting performance.  This is accomplished by quickly flipping

between two different display GROBs of the same size.  Note that the second

(bottom half) of the GROB is given a higher weighting (displayed twice) to

allow for the four scale operation.

This code is handy for viewing GROBs from the stack, but is not the best

solution for all cases.  It assumes that the stack-based GROB could start at

either an even or an odd address, meaning that we'd have to shift it to make

sure it starts on an even one.  In a greyscale game, however, it is better

to just use AGROB, since this is guaranteed to be located properly.

* Adopted from unknown source, commented

* Views a 131x128 GROB in the first stack level as a 131x64 greyscale grob

CODE

* System Initialization

GOSBVL =SAVPTR                        * Save RPL pointers

GOSBVL =DisableIntr                   * Disable interrupts

* Display Initialization

D0=(5) #00128                         * Load D0 with display row count

LC(2) #3F                             * Load C with count of 63

DAT0=C B                              * Kill the menu

* Program Initialization

ST=0 0                                * Clear program flag 0

A=DAT1 A                              * Load A(A) with the address of the

A=A+CON A,10                          *  grob in level 1 and skip the

A=A+CON A,10                          *  prolog

?ABIT=0 0                             * Grob data on even address?

GOYES Align                           * Yes, jump

ST=1 0                                * No, set program flag 0

LC(5) #01100                          * Load C with copy count (#01100)

A=A+C A                               * Add this to A(A)...

D1=A                                  * ...and put it in D1

A=A-1 A                               * Subtract one from this...

D0=A                                  * ...and put it in D0

GOSBVL =MOVEUP                        * Copy up, D0 -> D1

AD1EX                                 * Swap A and D1

A=A+1 A                               * Add 1 to A

Align

R0=A A                                * Ensure grobs can be restored

R1=A A                                * Load R1 with first grob

LC(5) #00880                          * Load C(A) with offset

A=A+C A                               * Add grob offset to A(A)

R2=A A                                * Load R2 and R3 with second grob

R3=A A

D1=(5) #00128                         * Set D1 to display row count

* Main Program Loop

MainLoop

GOSUB RotateGrob                      * Run grob-swapping routine

GOSUB CheckKey                        * Check for a keypress

?C=0 A                                * No keys pressed?

GOYES MainLoop                        * Yes, loop again

?ST=0 0                               * Check if not moved before

GOYES NoMove                          * Yes, no need to move

A=R0 A                                * Restore A(A) from R0(A)

D0=A                                  * Put this into D0

A=A-1 A                               * Subtract 1 from A(A)

D1=A                                  * Put this into D1

LC(5) #01100                          * Load copy count into C(A)

GOSBVL =MOVEDOWN                      * Copy down, D0 -> D1

NoMove

GOSBVL =D0->Row1                      * Restore old screen address to A(A)

D0=(5) #00120                         * Load D0 with display address base

DAT0=A A                              * Restore original display address

D0=(5) #00128                         * Load D0 with display row count

LC(2) #37                             * Load C with new count

DAT0=C B                              * Restore the bottom menu

GOSBVL =AllowIntr                     * Restore interrupts

GOSBVL =FlushAttn                     * Flush ON-key presses

GOSBVL =Flush                         * Flush keypresses

GOVLNG =GETPTRLOOP                    * Return to RPL

* Rotates the current grob address through the three loaded before (R1..3)

RotateGrob

A=R3 A                                * Load A(A) with next grob

AR2EX A                               * Rotate current grob through the

AR1EX A                               *  other three loaded before

R3=A A                                * A(A) points to current grob

D0=(5) #00120                         * Load D0 with display address base

DAT0=A A                              * Show current grob

A=DAT1 B                              * Read byte from display scan row

LC(2) #C0                             * Load C(1-2) with mask #1100 0000h

A=A&C B                               * Save DA19 and M32 with mask

C=A B                                 * Load C with this value

LC(1) #A                              * Load C(1) with #Ah (LCD row 10)

* Vertical blanking loop: ensures that routine always exits with retrace on

* LCD row 10

VBlankLoop1

A=DAT1 B                              * Read a byte at [D1]

?A>C B                                * Is the retrace below row 10?

GOYES VBlankLoop1                     * Yes, wait until it returns to start

VBlankLoop2

A=DAT1 B                              * Read a byte at [D1]

?A<=C B                               * Is the retrace above row 10?

GOYES VBlankLoop2                     * Yes, wait until it passes row 10

RTN                                   * Return to main loop

* Check for keypress routine: required, as interrupts are disabled

CheckKey

LC(5) #001FF                          * Load C with keymask

GOVLNG #01EEC                         * Put key matrix into C(A), return

ENDCODE

6.2 - ...Output a Solid Tone?

This code will output a solid tone using the supported "makebeep" entry

in ROM:

CODE

GOSBVL    =SAVPTR         * Save RPL pointers

LC(5)     1000            *

D=C       A               * D(A): Frequency (Hz), 1000 Hz

LC(5)     500             * C(A): Duration (ms), 500 ms

GOSBVL    =makebeep       * Call ROM sound routine (supported entry)

GOVLNG    =GETPTRLOOP     * Jump back into RPL

ENDCODE

6.3 - ...Read the Keyboard Directly?

The C=IN opcode is only able to operate from an even nibble address.

The =CINRTN entry point allows us to call the opcode from a guaranteed even

nibble.  This sample code loops until ON is pressed and "processes" the

left-arrow key.

CODE

* Start of our key loop

GetKey

C=0 W                     * Just need ON key

OUT=C

GOSBVL =CINRTN            * C=IN (calls from even address)

* NOTE: the ON-key is returned in bit 15 of C at this point, whether we want

* it or not.  Make sure you don't make any assumptions about which key

* is pressed by testing with "C#0?"

?C=0 A                                                                                                                          * If zero, not pressed

GOYES CheckLeft

* ON pressed, quit program

Quit

INTON

GOSBVL =AllowIntr                     * Restore interrupts

GOSBVL =FlushAttn                     * Flush ON-key presses

GOSBVL =Flush                         * Flush keypresses

GOVLNG =GETPTRLOOP                    * Return to RPL

* Whatever we want to do now (maybe check other keys?)

CheckLeft

LC(3) #040                            * Mask row with left/right

OUT=C

GOSBVL =CINRTN                        * C=IN (calls from even address)

LA(2) #01                             * Left-arrow column mask

?C#A B                                * If not equal, key is not pressed

GOYES CheckMoreKeys                   *   or pressed with other key

* Process left arrow

* etc.

CheckMoreKeys

GOLONG GetKey

ENDCODE

Note that you can test for multiple keys on a single row using a single

in/out pair if you test the final value using the bitwise and (&) instead of

the bitwise non-equivalence.

6.4 - ...Speed Up Time-Critical Calculations?

While the Saturn CPU is processing your program, it is not always

devoting all of its attention.  It services interrupts, refreshes the LCD

display, scans the keyboard and coordinates all of the external activities of

the calculator!  To ensure you have the complete attention of the CPU for any

calculation, follow these steps:

1.  Disable the interrupt handler

2.  Disable the CPU keyboard scan

3.  Turn off the LCD

And to re-enable them, just:

1.  Turn on the LCD

2.  Enable the CPU keyboard scan

3.  Re-enable the interrupt handler

In an RPL program, it might look like this:

CleanDispOff   EQU  #01D44    * Unsupported entry, DispOff is supported if

* you'd rather use it

CODE

GOSBVL    =SAVPTR           * Save RPL pointers

GOSBVL    =DisableIntr      * Disables the ROM interrupt routine and the

* automatic keyboard scan

GOSBVL    =CleanDispOff     * Clear display enable flag -> Display Off

INTOFF                      * Disable the automatic CPU keyboard scan

* Your fast code goes here (be careful!)

INTON

GOSBVL    =DispOn           * Turn display back on

GOSBVL    =AllowIntr        * Re-enables the ROM interrupt routine and the

* keyboard scan, services any pending interrupts

GOVLNG    =GETPTRLOOP       * Jump back into RPL

ENDCODE

If you want to speed up some SysRPL code instead, put this in place of

your ML code:

CODE

[pre-code routine]          * Don't forget SAVPTR/GETPTRLOOP

ENDCODE

ERRSET

::

* Your RPL code

;

ERRTRAP                       * In case an error occurs, reenable everything

::

CODE

[post-code routine]     * Don't forget SAVPTR/GETPTRLOOP

ENDCODE

ERRJMP

;

CODE

[post-code routine]         * Don't forget SAVPTR/GETPTRLOOP

ENDCODE

Be careful about what you put in this routine!  If something goes wrong

with the internal routine, the calculator will be locked and the only way to

reset it will be with the RESET button under one of the rubber feet.  Refer to

the interrupt-handler section for more information.

6.5 - ...Send text through the serial port?

6.6 - ...Send text through the IR port?

6.7 - ...Control the IR transmitter/receiver directly?

6.8 - ...Create My Own Interrupt Handler?

Creating a customized interrupt handler on the HP48 is a fairly large

kludge.  The problems created with this process ensure that the only valid

application is a game requiring full attention of the CPU.  To set up a custom

interrupt handler with one of these methods, the program must be fairly

independent of ROM functions and be written in 100% ML.  The methods are:

1) Program takeover of lower memory

Advantages:

- Only ROM functions within program size (rounded up to multiple of #00100)

are overwritten

- More compact final code object

Disadvantages:

- Requires enough RAM to make a copy of what the entire program overwrites in

lower RAM (minimum free RAM required = program size)

Method:

1)  Disable interrupts.

2)  Create a temporary object and copy amount of lower RAM equal to size of

program and interrupt handler.

3)  Copy interrupt handler to physical address #0000F of RAM and program to

somewhere around #00100.

4)  Re-configure RAM to address #00000, re-configure hardware registers to

higher address if required.

5)  Re-enable interrupts, execute program at new address.

6)  Disable interrupts.

7)  Re-copy lower RAM from temporary object.

8)  Re-configure RAM at default address.

9)  Re-enable interrupts, quit.

2) Complete RAM takeover of lower memory

Advantages:

- Requires only enough RAM to mirror what interrupt handler overwrites

Disadvantages

- Most of lower ROM is overwritten

Method:

1)  Disable interrupts.

2)  Create a temporary object and copy amount of lower RAM equal to size of

a GOTO instruction.

3)  Put a GOTO instruction at physical address #0000F of RAM and have it

jump to the physical address of your interrupt handler.

4)  Re-configure RAM to address #00000, re-configure hardware registers to

higher address if required.

5)  Re-enable interrupts, execute program.

6)  Disable interrupts.

7)  Re-copy lower RAM from temporary object.

8)  Re-configure RAM at default address.

9)  Re-enable interrupts, quit.

3) RAM-Card Backup Object Method

Advantages:

- Only ROM functions within program size (rounded up to multiple of #00100)

are overwritten

- No RAM-saving required

- Extremely easy to implement

Disadvantages:

- Requires dedicated RAM-card

- SX/GX only

- Cannot access upper ROM if used in card port 2 (without RAM shadowing)

Method:

1)  Create a RAM/ROM card with the new handler starting at address #0000F.

2)  Disable interrupts

3)  Configure port 2 to start at address 0

4)  Re-enable interrupts

-=---------------------------------------------------------------------------=-

7.0 - Glossary

-=---------------------------------------------------------------------------=-

The following terms and acronyms are used throughout the document and

defined as they are mostly likely to be used in this context.  Most of them

are used commonly or have definitions that are difficult to find.

Bus Command:

?

CRER:

Clear Receive ERror.

GROB:

GRaphics OBject.  An internal representation of a two-color graphic.

Hz:

Hertz.  A measure of frequency, in cycles per second.

IR:

Infrared.  Used to transmit information invisible to the human eye.

LCD:

Liquid Crystal Display.  A display that displays information by using a

liquid crystal that polarizes light when a current is applied to it.  Commonly

used in portable electronic devices, such as calculators.

LED:

Light Emitting Diode.  An IR LED is used to generate IR light.

LSB:

Least Significant Bit.  The bit that has the smallest decimal

representation.  The rightmost bit.

ML:

Machine Language.  Refers to the basic assembly language of the Saturn

processor.

MSB:

Most Significant Bit.  The bit that has the largest decimal

representation.  The leftmost bit.

Nibble:

Four-bit data, half a byte.  As the Saturn chip has a four-bit data path,

the nibble is used extensively internally.

RBR:

Receive Buffer Register.  A hardware register that holds the last

received character.

RCR:

Receive Control Register.

RECV:

Serial receive function.  Refers to the transmission of a byte by the

UART via either IR or wire.

RPL:

"Reverse Polish Lisp" or "ROM-based Procedural Language".  A programming

format used uniquely in HP calculators, utilizing mainly a stack for

operations and calculations.  Used as a suffix, like SysRPL or UserRPL.

Scratch Register:

A 64-bit register used to hold working register values.

SREQ:

Service request.  Used on the older HP calculators to control card

readers and other devices.

SysRPL:

A medium-level HP programming language.

TBR:

Transmit Buffer Register.  A hardware register that holds the next

character to be sent by the UART.

TCR:

Transmit Control Register.

UART:

Universal Asynchronous Receiver/Transmitter.  A chip used to control

serial communication.

UserRPL:

A high-level HP programming language.

Wire transmission:

Information transmitted bidirectionally over a serial-link cable.

Working Register:

A 64-bit register used extensively in calculations.

XMIT:

Serial transmit function.  Refers to the transmission of a byte by the

UART via either IR or wire.

-=---------------------------------------------------------------------------=-

8.0 - Contributing/Contacting

-=---------------------------------------------------------------------------=-

8.1 - Contacting the Author

Please feel free to contact the author if you:

- would like to contribute something to this document;

- have found an error or omission; or

- would like to make a suggestion.

You can contact the author in one of the following ways:

Email: mmastrac@acs.ucalgary.ca

Newsgroup: comp.sys.hp48

The latest version of this document can always be found at

http://www.acs.ucalgary.ca/~mmastrac/files/saturn.html

8.2 - Contacting Contributors

The following contributors have offered their email addresses for

contact:

Name             Address                        Notes

Philippe Teuwen  s952365@student.ulg.ac.be      HP48 PCB Drawings

This manual compiled using MC v1.05 by Matthew Mastracci