(Comp.sys.handhelds)

Item: 3834 by catto at author.ecn.purdue.edu

Author: [Erin S Catto]

Subj: Fuzz & ML Blurbs

Date: Tue Aug 13 1991

Well I promised myself that I would pass on what I've learned once I figured

out what's going on with this machine language business.  Sorry, it's not a

game.  Anyway, here's a small program which simply draws random dots on the

stack display and exits when you press the drop key.

I thank Jan Brittenson for the STAR assembler (excellent) and the MLDL (way

excellent).

This program covers most of the key hardware questions a freshie might have.

+  Drawing on the stack display at a coordinate (0-127,0-63)

+  Data storage acquisition so that data tables are temporary

+  Turning off keyboard interupts (go ahead, press ON-A-F, nothing happens)

+  Machine code keyboard scanning

+  How to get random numbers you can't depend on

I'll briefly cover a few topics:

Preserving the RPL Operating System

===================================

The HP48 has an operating system just like any other computer.  It was written

in System RPL and machine code.  The HP48 OS uses the registers d0, d1, b, and

d to take care of business, such as memory management and RPL command pointing.

Thus you must save these important register and restore them at the termination

of your machine code program.  You must also set the OS back on track.  Here's

how it's done:

save_regs = #0679B      ;ml routines to handle these tasks

rest_regs = #067D2

begin

call.a     save_regs      ;save registers

.

.

.

call.a     rest_regs      ;restore registers

move.a     @d0, a         ;a = next RPL instruction

add.a      5, d0          ;point d0 to following instruction

jump       @a             ;jump to instruction in a

;address of object prolog?

end

Here's how I believe the last three instructions work:

d0

|

v

RPL instruction stream:  ... 11EC7|65547|EF9D7|83188|...

move.a    @d0, a       ;a gets 74556

add.a     5, d0        ;does the following

d0

|

v

RPL instruction stream:  ... 11EC7|65547|EF9D7|83188|...

Lets say at the address 74556 there is the RPL program prolog 02D9D then

jump     @a

will jump to the address 02D9D, a chain of pointers.

Now the operating system is back on track, refreshing the screen, handling the

keyboard, etc.  Evidence of this is seen when you exit the attached program,

the RPL OS automatically returns the screen to its normal state.

Drawing on the Stack Display

============================

The stack display contains two grobs: the stack-message grob and the menu grob.

Statistics:

+ Screen Grob:   location of grob contained at #7055B

size = 56 rows high by 136 bits wide (34 nibbles)

+ Menu Grob:     location of grob contained at #70551

size = 8 rows high by 136 bits wide

Once you get to the grob in memory you must skip the 20 nibble prolog to get to

the pixel data.  The pixel data begins at the upper left corner of the display

and goes left to right, wrapping to the next row after 34 nibbles.  While the

nibbles go left to right as the address increases the bits in each nibble are

in right to left order.

Example:         On Screen:          0101

In memory:          1010 = #A

It is useful to have a data table which contains the address of the start of

each row of pixels.  This way you can take an arbitrary y coordinate and

transform it into an index to the data simply by multiplying y by 5.  Lets

look at this:

Say y = 4, then  5 * y = 20, now the data table looks like this:

start y=0   y=1   y=2   y=3   y=4

|     |     |     |     |

v     v     v     v     v

04347|26347|48347|6A347|8C347.....  (stream of row addresses)

So you use start+20 to get the row address.  Below I talk about building a

data

string.

Here's my plug for using the stack display instead of PICT.

1)  Let PICT be the user's territory.

2)  PICT sits and takes up 1098 bytes, unless you PURGE it.

3)  The stack display is always the same size (131x64)

Making Data Space

=================

Quite often data tables are used in mcode programs.  If the data is going to be

generated by the program then it becomes a waste of space to attach non-

executing data to your program.  A routine at #05B7D will make a string object

whose data length (not including the header) is specified by the contents of

the c reqister.  The address is returned in the address register d0.

d0

v

C2A20|04000|123DF8734872394729834...   ...2312|

<- header->|<--- length = c -----...   ...--->|

All you need to do is:

move.p5  c, length      ;you specify length

call     make_str       ;make_str = #05B7D

swap.a   a, d0          ;d0 = string data area address

move.w   a, r0          ;store the address somewhere for reference

Turning Off Interupts

=====================

Turning off interupts means turning off the keyboard, in a manner of speaking.

It means your program will only be interupted when you want input.  It is

simple to turn the interupts off, but you must be sure to turn them back on.

Interupts Off:          clrb      15, st       ;clear bit 15 of status

register

intoff

Interupts On:           inton

setb      15, st       ;set bit 15 of status register

It would be nice if someone in-the-know would explain the flags the ST register

contains.

Keyboard input, the ML way

===========================

The instructions which must be used to grab key presses are OUT and IN.

Sounds simple enough, sure if you know how to do it.  Here's how it's done

1)  load register c with the row of keys you want to check (see table below)

2)  perform  OUT.X  C to tell your HP to check that row

3)  call IN_C = #01160, this address only contains:    IN.4   C

RET

It seems that IN.4   C must be called, I think it messes with the

return stack.

4)  move the adress field of register c somewhere safe, say register a

5)  clear register c

6)  perform OUT.X  C to tell your HP to stop checking

7)  now if you stored register c in register a in step (4) then the low

nibbles of reg. a will contain bits set corresponding to the keys pressed

in the row of interest.

The Keyboard Map:  (from an article by Jan Brittenson)

(IN) #20 #10 #08 #04 #02 #01

---------------------------

(OUT) |    6   5   4   3   2   1

#100  |        B   C   D   E   F

#080  |       PRG CST VAR  ^  NXT

#040  |       STO EVL  <   v   >

#020  |       COS TAN sqt pwr inv

#010  |   ON* ENT +/- EEX DEL <==

#008  |   alp SIN  7   8   9   /

#004  |   yel MTH  4   5   6   x

#002  |   blu  A   1   2   3   -

#001  |        '   0   .  SPC  +

(*) The ON key is actually in a column of its own.

#xxx refers to the keyboard scan bits. The column

scan bit for the ON key is #8000.

The numbers in the leftmost column are use to specify the row in the OUT

command and the numbers in the topmost row correspond to a positive result

from the IN command.  Note that these numbers have a binary representation

which has only one bit set.  So multiple bits set by the IN command mean

multiple key presses.

The Random Thing

================

Your HP48sx maintains a hardware checksum at #00104 which is constantly

changing in a seemingly unpredictable manner.  You can get four random

nibbles at a time from the above address which may be masked down to the

range of numbers you are interested in.

The program below takes two random numbers from the hardware crc, one

for the x coordinate and one for the y coordinate.  Two dummy instructions

were planted inbetween the two readings.  It turns out that without these

two dummy instructions, the static on the screen reduces to two predictable

lines.

Notice

======

I claim no proficiency at ML programming, but maybe this info will help others

get started.  I did not hack out much of this.  Most of it came from studying

the source code for SCHIP v1.0 by Erik Bryntse.  For those not familiar with

this type of programming I suggest you actually run the program below and

recognize that only one pixel is being draw at a time...it's FAST!!!  By the

way, press the drop key to EXIT.

The ML Freshie,

Zoom

catto@ecn.purdue.edu

P.S.  Has anyone finished Ant?  You must see the final round, it's a hoot.

The rolling (:() is actually quite easy, once you know how to get

by it.  There's a lot more game after that.

--------------------------begin STAR source------------------------------------

;FUZZ: makes random static on screen, drop key exits

;r0 = data string pointer

;r1 = x coordinate in pixels

;r2 = key data storage

make_str        =       #05B7D

save_regs       =       #0679B

rest_regs       =       #067D2

crc_val         =       #00104

screen_ptr      =       #7055B

menu_ptr        =       #70551

in_c            =       #01160

header

code

call            save_regs       ;save d0, d1, b, d

clr.w           c               ;c = 0

move.w          c, r2           ;initialize key storage

clr.w           a               ;a = 0

move.p3         #140, c         ;c = data string size (64*5+4)

call            make_str        ;make a data string

swap.a          a, d0           ;a = string address

move.w          a, r0           ;save string address in r0

move.a          a, d0           ;point to string

move.5          screen_ptr, d1  ;d1 = address of screen grob

move.p2         56, a           ;a = # rows in screen grob

call            fill_table      ;make data table

move.5          menu_ptr, d1    ;d1 = address of menu grob

move.p2         8, a            ;a = # rows in menu grob

call            fill_table      ;complete data table

clrb            15, st          ;interupt flag off

intoff                          ;turn off interupts

main:           move.5          crc_val, d0     ;d0 = address of hardware crc

move.a          @d0, c          ;c = pseudo random #

clr.a           a               ;a = 0

move.p2         #7F, a          ;a = mask for screen width

and.a           a, c            ;c = random and x mask

move.w          c, r1           ;save x coordinate

move.5          in_c, d0        ;these two lines do nothing

move.a          @d0, c          ;except make the crc random

;it's a numbers game

move.5          crc_val, d0     ;point to crc again

move.a          @d0, c          ;c = pseudo random #

move.p2         #3F, a          ;a = mask for screen height

and.a           a, c            ;c = random and y mask

move.a          c, a            ;a = c = y coordinate

add.a           c, c            ;c = 2*c

add.a           c, c            ;c = 4*c

add.a           a, c            ;c = 5*c

move.w          r0, a           ;get data string address

add.a           a, c            ;add offset for y coordinate

move.a          c, d0           ;point to row address

move.a          @d0, c          ;get row address

move.w          r1, a           ;get x coordinate in pixels

srb.w           a               ;a = x/2 (shift right bit)

srb.w           a               ;a = x/4 = x coordinate in nibs

add.a           c, a            ;a = row start + (x offset)

move.a          a, d0           ;d0 points to nib of interest

move.w          r1, c           ;c = x coordinate in pixels

clr.a           a               ;a = 0

move.p1         3, a            ;a = 3 (#11b)

and.a           c, a            ;get pixel position in nib

clr.a           c               ;c = 0

move.p1         1, c            ;set up c

;mirror_nib routine makes a nibble containing one bit set a x coordinate.

;While grob nibbles are in order from left to right (up in memory),

;the bits within the nibble are in reverse order.  So this routine

;puts the nibble through a u-turn.

mirror_nib:     dec.a           a               ;a = a - 1

;if a(old) = 0 then sets carry

brcs            ready           ;jump if carry is set

add.a           c, c            ;c = 2*c = shift left bit c

jump            mirror_nib      ;loop

;Now the old nibble of the lcd grob is xor-ed with the new nibble

;BTW a nibble is just a four bit chunk, a hex digit (ex. #1111b = #Fh = #15d)

ready:          clr.a           a               ;xor old and new nibble

move.1          @d0, a          ;a = old nibble (OLD)

clr.w           d

move.a          c, d            ;d = new nibble (NEW)

move.a          a, c            ;a = c = OLD

and.a           d, c            ;c = OLD and NEW

swap.a          c, d            ;d = OLD and NEW

or.a            a, c            ;c = OLD or NEW

not.a           d               ;d = not(OLD and NEW)

and.a           d, c            ;c = OLD xor NEW

move.1          c, @d0          ;place xor nibble

move.p3         #010, c         ;#010 specifies the row of

out.x           c               ;the drop key, the out.x

;instruction scans a row

call            in_c            ;in_c returns the results

;you have to call this command

;something to do with return

;stack

move.a          c, a            ;a = results

clr.a           c               ;c = 0

out.x           c               ;stop scanning

;the following checks if the drop was pressed AND released for program exit

;that way your not pushing the drop key when program exits

move.w          r2, c           ;get old key status

move.w          a, r2           ;put new key status

not.a           a               ;a = keys not pressed

and.a           c, a            ;and keys that were pressed

brbs            0, a, exit      ;bit 0 of contains flag

jump            main            ;for drop key column

;fill_table:  make a data table containing addresses of lcd rows

;IN:    d1 points to grob data

;       d0 points to data string

;       a = # rows to set up

;AFFECTS:       a, c, d0

fill_table:     move.a          @d1, c          ;c = grob adress

add.a           16, c           ;add 20 to skip grob header

add.a           4, c

ftlp:           move.a          c, @d0          ;put row address into table

add.a           16, c           ;add 34 to skip to next row

add.a           16, c           ;lcd grob width = 34 * 4 = 136

add.a           2, c

add.a           5, d0           ;skip to next address space

dec.b           a               ;decrement row counter

brnz.b          a, ftlp         ;continue if counter <> 0

ret

exit:           inton                           ;interupts on (keyboard)

setb            15, st          ;flag interupts on

call            rest_regs       ;recall d, b, d1, d0

move.a          @d0, a          ;return to RPL operating system

add.a           5, d0

jump            @a

endcode

-------------------------------end STAR source---------------------------------