Trig R US  (for G/GX only, due to specific G/GX commands)

Version 1.3

May 23, 1998

Copyright 1998 Don Trust

lib# 1020 (runs from any port)

size 9006 bytes

Preliminary documentation (real preliminary)

The programs in this library will be of use to machinists and probably no one else.

Printer required for some of these programs. I will probably change this in the

future to output to screen or printer.

*** FCHOOSE library by Matt Willis required. ver 3.3 used.***

You can find this on goodies disk #11 or at <http://hp48.ml.org/programming/misc/>

Thanks Matt, great program.

These are all User-RPL programs, except for a couple calls to SYSEVALs.  Nothing

here to mess with your memory, but I won't be responsible for any problems

you run into.  I won't take responsibility for any wrong answers you get,

but as far as I can tell, everything is correct.

Backup your memory if you want to be safe.  You have been warned.

I can say that this library has been used by many people where I work with no

problems whatsover.

Feel free to email me with any questions.  Better docs to come later, if there

is any interest.  This is a freebie, but I won't turn down money, cars, or other

valuables. :-)

Triangle Solutions - works only with right triangles.  tried to make this

as simple as possible, but needs some work.  Bigger

than it should be, but this was my first HP48 program,

and it definately ain't optimized.

Radius Moves       - Prints  how far in a Y+ direction to move grinding

wheel or cutter at a given angle to generate a radius

around a corner using small flats to "fake" in the curve.

For you machinists out there, it is meant to be used with

a Harig or dividing head.

Bolt Circle        - Print X-Y locations for holes to be drilled in a circular

Pattern. Program will offset the pattern from your present

position, if you want.

Cylindrical radius - Print moves for use on a surface grinder or a mill

to "fake in" a concave radius of a specified size, using

a "tolerance band" to keep the leftover scallops within

that range. You get to specify the radius on your cutter.

Serrations         - Prints wheel moves for grinding equal size grooves into

face (top) of a punch.

Change factor      - figures value to be multiplied by the amount of stock

you want to take off each side of groove to get the actual

change in MOW (measurement over wire).

Size over corners  - find the size over two same-size radii, given the length,

width, and corner radius of a rectangle.

Find radius size   - pretty much the opposite of the above.  If the length, width,

and over radius size is known, will figure the actual radius.

Set screw grooves  - figure the angle of the groove in the back of a punch.  Also

known as "cats ass".  The radius depth must be less than

the radius.

Circular segment   - figure either the depth, chord, or radius of a concave

depression, knowing 2 of the three.

Max wheel for face - computes the maximum diameter wheel that can be use to grind

an inside chamfer on the face of a punch. (diameter of

wheel touching face)

Punch blips        - a bit hard to explain.  docs sometime later, (needs a picture)

probably of no value to most people. Printed output.

Polygon sizes      - will figure over-radii size or over flat-radius size of

a polygon of any number of sides.  Gives flat-radius size for

odd number of sides, rad-rad size for even numbered sides.

Height above center- figures the distance between a line drawn through whatever

center point you choose (0,0) and a parallel line going through

the X-Y coordinates specified, at a particular angle

(needs a picture)

Cutter Angle       - figures the angle to grind on either 3 or 4 sides of a cutter

to get specified angle on cutting edge. Give the angle as

coming off the face of the cutter, and the angles returned

will be the same way.  Same for angle off the centerline.

Line-Line intersect- compute the intersect point of two lines.

Line tan to 2 circ - compute the angle that is tangent to 2 radii. The 2 radii can

be entered in any order, and there are 4 possible lines to

figure.  Think of the line being 'unwound' from the cirlce

and if it 'unwinds' CCW, that radius is entered as a positve

number.  If the line 'unwinds' CW that radius should be entered

as a negative number. (hmmm, easier to draw on paper than

explain in writing)

Line intersect, line tan to 2 circles, Height above center... these three programs

expect and return the angle a little different than normal.  For these

programs, the 0 degree point is at 12 0'clock, not 3 o'clock. The angles go from 0

degrees at 12 o'clock to +90 at 9 0'clock and to -90 at 3 o'clock.  This is a

convention I have used for years for certain programs, and is due to making

drawings on a type of drafting table I used to use years ago.  It was just easier

to do it this way for that machine.