QAG48

Version 1.0.1

March 13, 1998

QAG48 is a replacement for the internal numerical integration

of the HP48's.

Bytes: 4562.5

CRC:   #725Bh

Library #1548

To install:

Download the library to your calculator. Put the library on stack. Type

0 (or the port number of your choice) STO. Turn the calculator off & on.

Delete the old copy of the library to save space.

The library uses some unsupported but stable entries. They all appear in

Entries.doc. The library has been tested on a HP48S/SX rev J, and HP48G/GX

rev M and R. The library should not give any problems.

What it is:

QAG48 is a Numerical Integration library for the HP48's. QAG48 is based on

QAGS from the QUADPACK library of FORTRAN programs. This library uses

an adaptive scheme that estimates errors for intervals and bisects

only those whose errors are not within the limit set by the user.

For those who care; QAG48 (& QAGS) uses a Gauss Kronrod method with

10/21 points. The 10 point rule is exact for up to 21st order polynomials

and is identical to the 10 point Gauss-Legendre rule. The extra 11 points

are added in between the first 10 points & the endpoints - which

are reused with new weights for all 21 points to be exact for up to 31st

order polynomials. The difference between these two rules is the basis for

estimating the error of the 21 point rule.

Furthermore, QAG48 (& QAGS) also uses the Epsilon Extrapolation

method to speed up convergence of the results from the Gauss Kronrod

method.

The end result of all of this is a very efficient and accurate method

for calculating Definite Integrals numerically. It also means that we must

store lists of results, errors, integration ranges, and flags for each of

the intervals integrated over. This can result with up to 0.9 bytes stored

per function evaluation. I have tested some difficult integrals to about

20,000 function evaluations, thus resulting in about 18,000 bytes stored.

As long as you have about 5000 bytes free on your calc, QAG48 will work

well for most integrals.

Changes up to v1.0.1

1. Fix a bug that would leave extra stuff on the stack if the integrand (or a part of it for a subinterval) 	was a constant.

Changes since last release:

1. The QAG48 command was changed to just QAG.

2. This version of the library uses ML to speed up the various calculations

somewhat. This version takes about 60% of the time that the earlier the

version did. Also the shortest time needed for most integrals has been

reduced to only a fraction of a second more than the best that the built-in

HP48 integrator can do.

3. Eliminates false stops for certain integrals

4. Automatically handles some infinite range integrals (see below for more

information). The QAGO command is added to handle some other infinite

range integrals.

5.  LASTARG can be used to recover the integral after running the QAG command.

But LASTARG will not work (at the moment) if calculations stopped because of

an error in calculating results from the integrand, such as an overflow error.

6. The error calculations were changed to improve accuracy and reliability

of the error estimate, particularly for high accuracy results. Unfortunately this has

increased the time needed to calculate some integrals even with the ML code.

The error estimate now includes error incurred with the rounding to the

standard real numbers.

7. The time limit feature works now.

8. The number of loops used for the iteration limit is raised to 100 (was 37).

Syntax:

Enter the integral with the integral symbol. Then press or type QAG.

The library currently only works for a single integration. You can use

the HP48's internal numerical integration for the inner integration and

QAG for the outer integration of a double integral.

e.g.

'INTEGRAL(0,pi,sin(X),X)'    (where INTEGRAL is the integral sign)

then   QAG.  The command will run, returning the final result to stack

level 1, and the approximated error to the global variable IERR. (same as

the HP48's). At any time you can press CANCEL and the command will return

the latest result with the latest approximated error in IERR. The integral can

be recovered by using LASTARG.

The program uses a global variable named: 'intpar' and is defined as:

intpar: {time limit, relative error, display}

time limit: Number of seconds that you want to limit the command to run for.

Default = MAXR - essentially no limit.

relative error: 1E-11<= relative error <= .1 ; Default=1E-6

the error bound is approximated by

(relative error)*INTEGRAL(a,b,ABS(integrand),var)where the

integral is INTEGRAL(a,b,integrand,var)

This is the same way that the HP48's do it, except that

the relative error is given by the FIX/ENG/SCI modes.

display: 0= QAG48 display OFF

1= QAG48 display ON - Default

When QAG48 display is on, it shows the latest result,

error and the number of function evaluations. You can

then decide if the result is accurate enough & CANCEL.

QAG48 will then return the latest result with "BEST"

tagged onto it, and with the latest error in IERR.

Also on line 6, there is now a number that indicates

the iteration loop level of the Epsilon extrapolation routine.

'intpar' is not created automatically (I dislike programs that do this),

but does need to be within the directory tree up to HOME. Otherwise

QAG48 will use the default values.

Performance:

The command must do 21 function evaluations to get a result and thus

will take at least 0.9 seconds to return the first result. The HP48's

internal routine can give results in under a second, particularly

with 3 FIX (or a relative error of 1E-3).

I have tested QAG48 with 112 integrals. 76 are finite range integrals,

and the rest are infinite range integrals. These integrals test

a wide variety of integral types and difficulties. I also use them to

study the accuracy and reliability of QAG48 vs the HP48's built-in

numerical integrator, and the TI's (i.e. the TI-85 & TI-92).

With a relative error of 1E-3, QAG48 was faster than the HP48's routine

for 24 of 75. Within a reasonable time, QAG48 provided good results

for 73 of them and the HP48's routine provided good results for 69 of them.

For QAG48, the 3 integrals that gave poor results either gave a bad error

estimate or was not converging on a result. These integrals are discussed

below. For the HP48's routine, the integrals that gave bad results also

included integrals with singularities within or at the boundaries of the

integration range.

With a relative error of 1E-7, QAG48 was faster than the HP48's routine

for 54 of 73. Within a reasonable time, QAG48 provided good results

for 72 of them and the HP48's routine provided good results for 57 of them.

With a relative error of 1E-11, QAG48 was faster than the HP48's routine

for 50 of 69. Within a reasonable time, QAG48 provided good results

for 68 of them and the HP48's routine provided good results for ?? of them.

Most of the problems that QAG48 has for this accuracy are due to round off

errors, discussed below.

One of the integrals ran for 80 minutes on the HP48's routine and gave only

3 places accuracy, but QAG48 gave the answer in 23 seconds.

Comparison to the TI-85: The TI-85 uses a different method of which

I've been unable to determine. The TI-85 outperforms the HP48G's internal

routine about half the time. The TI-85 does do somewhat better with

singularities than the HP48's routine, but is way behind QAG48. QAG48 is

faster than the TI-85 for the few integrals I have studied (i.e. for 12 of 14)

at a relative error of 1E-11, but QAG48 is slower for a lower relative error

of .001. One awkward feature of the TI-85 is that the error limit appears to

be an absolute limit, thus you may have to adjust it after getting some results.

Comparison to the TI-92: The TI-92 has a very powerful integration

package. Most of it's "numerical" integration is apparently done by

first solving the symbolic integral (lookup tables?) and then applying

the limits. Thus it's fast for simple high accuracy integrals. Of the

8 integrals I tested that the TI-92 used a numerical method (method is unknown)

QAG48 was able to outperform the TI-92 on 6 of them and even then by

only half.

On Integrands:

The type of functions being integrated affects the speed of the results

and the accuracy of the results.

1.For "smooth" functions both QAG48 and the HP48's internal routines can

give quick and accurate results.

2.For Rapidly Oscillatory functions; for example functions including terms like

COS(50*X) over a range of 0 to Pi. Typically the simple Rectangular rule

or the Trapezoidal rule can give the best results. For the HP48  user, the HP48's

routine usually outperforms QAG48, except when the coefficient is a multiple

of 512 for certain ranges - the HP48's routine gives the wrong answer for

INTEGRAL(0,PI,COS(512*X),X).

3. Singularities on or near (outside) the endpoints: Only for certain types

of singularities can the HP48's routine give decent results. QAG48, however, can

and does make short work on these types of integrands, including infinite

singularities unless the integrand is oscillatory.

4. Singularities within the interval of integration: These types of integrands can

be difficult to get results in a reasonable time. For finite singularities, QAG48 can

give results eventually (perhaps up to 3-4 minutes). For infinite singularities, QAG48

can give ballpark figures - indeed virtually no numerical integration routines can

give even decent results on such an integrand. The solution is to break up the

integrating interval into intervals spanning between the singularities. This can

considerably help for finite singularities also.

5. A class of integrals that gives QAG48 problems combine an infinite singularity

with oscillations going to infinity at the same location. For example the integral:

INTEGRAL(0,1,SIN(1/X)/X,X). The result using QAG just floats around with

no indication of converging.

Infinite oscillations gives enough problems as it is anyways: an example is: 	INTEGRAL(0,1,SIN(1/X),X); QAG gives an answer at a relative error of .001, but

the error estimate was bad.

Infinite singularities inside the integrating range also gives problems. For example:

INTEGRAL(0,2,1-9/XROOT(3,SQ(X)),X); QAG gives an answer at relative error

of .001, but the error estimate is bad.

6. If a number of singularities occur within the integrating range, then QAG48 starts

to have problems. For example is INTEGRAL(0,3,FP(SQ(X)),X) which will give

some good results at lower relative accuracy, but becomes difficult at a higher

relative accuracy.

7. Round off error can be a problem at high relative accuracy (1E-11). For example:

INTEGRAL(-1,1,SQRT(1-SQ(X)),X) will not stop in a reasonable time. Related to this

is round off occurring at the range limits. For example PI is rounded by the calculator

to 3.14159265359. This can cause a problem if there is an infinite singularity at that

location

Infinite Range Integrals:

QAG48 now can recognize infinite range integrals. As long as either or both upper or

lower limits of the integral are +-MAXR or +-9.99999999999E499. QAG48 does

this in this version by using the MATCH commands and a substitution. The version

I'm working on now will place a little secondary before and after the function and

hence will run faster.

My investigation into infinite range integrals showed me that it's not possible to

come up with one single substitution that will work well for most integrals. In particular,

the substitution I used with QAG does not work well when trigonometric functions

are present in the integrand (unless combined with a exponentially decaying function).

So....  I have provided QAGO to be able to get results for these types of integrals.

QAGO is currently set up to work only on infinite range integrals, and will error

for finite range integrals. In the version I plan on extending this to work for finite

range integrals of the type stated in (5) above.  QAGO does not run very fast - It will

take about 10 mins to get a result to 11 places. Better algorithms exist, but they require

to know where the zeros of the integrand occur.

Future improvements:

1. Add tests for detecting divergent integrals, detecting roundoff error,

and detecting slowly convergent integrals.

2. Expand the commands to work with Units, Complex number range

and outputs.

3. A new command to allow the user to subdivide the integral at user

defined locations (for use with integrals with internal singularities).

4. Support for Multiple Integration.

5. Low memory support - to detect when a low memory condition may

occur and to end the command with the best result on the stack.

6. The Epsilon extrapolation routine can be used to accelerate results to

infinite summations, so I will add a command to do this.

7. QAGO will be extended to work on finite range integrals to give some

results for rapidly oscillating integrands.

8. I will try to write up a PostScript version of the documentation.

Send comments, suggestions, etc. to:

John Edry, JEEjohn@aol.com