DAVID ELFORD'S LEAST SQUARES RESECTION PROGRAM

(C) Copyright 1995 DAVID ELFORD

These programs are not guaranteed to be free from defects.  Use

them at your own risk.  I shall not be held responsible for any

loss caused by these programs or their use, incidental or otherwise.

===============================================

Files you need : RESECT     2190    08-10-95

IFTST      173     08-10-95

OCTST      277     08-10-95

===============================================

This program solves for resections, which are sometimes used by surveyors.

The program uses Least Squares Estimates to calculate the 'best' solution

for a resection.

The program calculates the final easting, northing, and orrientation

(in D.MMSS) format of the resected point, as well as the number of

iterations done. This information is placed in the following variables -

Easting :        'EOCC'

Northing :       'NOCC'

Orientation :    'ORR'

Iterations :     'ITT'

************************************************************************

Running the program

When the program is started, you are asked the standard deviation

of a single observation. This must be entered as seconds of arc.

Typically, for say a Wild T2 theodolite, the standard error may be 1

or 2 seconds.

The number of observations and the approximate coords of the

occupied station are asked for next. Note: use the arrow keys to move

to the next input prompt. When all the data is entered then press the

ENTER key.

Finally, the direction to and coordinates of the known point is asked

for. The direction should be entered in D.MMSS format.

**********************************************************************

** Important Notes **

1. Place the this program in an empty directory. The program creates a

lot of variables, then purges most of them when the calculations are

complete.

2. The calculations are NOT fast. However, this can be reduced by using

closer approximations of the coordinates of the point being resected.

Typical calculations can take around 10-30 seconds to complete. If it

takes more than this, the calcs. may be diverging. Press cancel and

check the variables 'DELE' and 'DELN'. If these values are large then

you can assume the calulation is diverging.

3. Other statistical data such as Aposteriori Variance Factor, error

ellipses etc are not calculated (This will be done in the next version

of this program).