Statistical Process Control

for The G\GX\S\SX

version 2.5

By Othniel Rawlins

ostcroix@aol.com

This program is freeware; it is copyright (c) 1995

by Othniel Rawlins. This program my be  Redistributed

freely as long there is no exchange of cash. Although

I spent hours debugging this program, use it at your

own risk; I am not responsible for any damages or memory

lost to your calc as a result of using this program.

I    Revision History

II   Acknowledgements

III  Introduction

IV   Setting up SPC

V    What is SPC?

VI   Menu definition

VII  Keyboard definition for Main display

VIII If you use this program

I     REVISION HISTORY

=====================

Version2.5

Got rid of all unsupported entries;

Now works on S/SX

Made changes to output to Left Shift X&R

Added a SEND feature to export data to

computer

Histogram is no longer cleared from PICT

Version 2.4                     ==

ADDED                 The ATTN has been error trap to pop

back to the previous display during

execution of STAT HIST X&R and PRINT.

ADDED                 SPC can now tell difference between

printing via wire and printing via Infrared.

That change was influence by

Jarno Peschier (jpeschie@cs.ruu.nl)

HP82240B infrared printer emulator

ADDED                 Depressing LS with uparrow or

downarrow repeats a page

FIXED                 fix insufficient memory problems after

control charts are printed

FIXED                 Backarrow key to abort better

CHANGED               Rewrote routines of browsers for faster

scrolling and updating of display

CHANGED               TDS variable is no longer used. I switched

to vardat to avoid conflicts with future

versions.

CHANGED               Got rid of a lot of nagging unsupported

entries; however, a few still lurks

======

Version 2.3a         The ENTER key crashed the calc

when COPY or MOVE are suspended. FIX!

I also forgot to mention that x&r chart scrolls

horizontally using left arrow & right arrow

========

Version 2.3          First release to the public

II      Acknowledgements

================

William Kitchen - For showing how to write a browsing proram

back in my SX days

Jacklevy       - For posting this SPC program to the ftp sites for me.

Robert Tiismus  -  I am using his fast grob rotation program

that rotate my control chart grobs so I can

PRINT. Thanks for the other insights

Richard Steventon -  For sharing his source files for SOL

& JAVA of which help me to better

understand how to use MAKEROM.

On that note, source files for this program is avaliable upon request

III    INTRODUCTION

============

This is a real world program on the subject of

Statistical Process Control. It is assume that the

user have a basic knowledge of SPC from a manufacturing

perspective. My intent for this project is simply to

provide HP48 users with the capability of performing

Statistcal Process Control on their HP48. Explaining how

to interpret the many different Satistical outputs from

this package is far beyond the scope of this documentation.

IV  What is SPC?

============

Statistical Process Control is nothing more than

statistical tools that are used to analyze, control,

and reduce variability within a process. The different

kinds of tools used are run charts, control charts,

histograms, normal distribution & sometimes

confidence limits. The most common tools used are

control charts, histogram & normal distirbution, which

are included in this package.

V     Setting up SPC

==============

Download SPCV25.LIB and DATA to your calculator.

If your are not familiar with using a library, refer

to owners manual under library. Data is a HP48 directory of

example data that I have collected from the book: Statistical

Quality Control by Eugene L. Grant & Richard S. Leavenworth,

the six edition. It is a good idea to use that directory to get

familiar with how to use the program

Execute SPC to get into the main display.

The main display shows the current directory path

along with two types of objects: directories and arrays.

Arrays will always be displayed in the following format:

XXXXXX:[SSxSG]

XXXXXX - data name

SS     - Sample size

SG     - Subgroup Size

VI  Menu definition

=================

=

[NEW] -  Collects new data for current directory.

New prompts for data name. Enters HP's

Matrix Manager for data collection. New then

enters you in the database collector so important

info about your array can be stored in a database.

NOTE: When collecting data, remember column are SS and rows are SG.

Here is the database input definitions.

NAME  - Person who collected the data.

UTL   - Upper tolerance limit

LTL   - Lower tolerance limit

MACH# - Machine number

TYPE  - Machine type

PART# - Part number

LOT#  - Lot number

NOTE  - Any notes about the data you may want to keep track of.

NOTE: Each directory uses a variable call vardat. vardat stores database

information for the current directory. Altering the contents of

vardat may cause a MEMORY LOST!!! Let SPC do all the altering

===

[X&R] - Plots X&R charts for the current highlighted data

control charts can be horizontally by using

Left and right arrow keys

NOTE: maximum sample size 10. maximum subgroup size 52

[X&R]    Key assignments

-----------------------

DIVIDE   -  Is also assigned to [HIST] and [STAT]

this is the print key for printing to HP's

infrared printer or Jarno's infrared printer emulator.

DIVIDE prompts if you would like to print the header.

selecting YES prints header then continue to print info. NO

just skips printing header then continue print info.

ON pops back to previous display.

MULTIPLY - From left to right: upper control limit,

center line, and lower control limit

SUBTRACT - Connect pixel; disconnect pixel

PLUS     - Toggles Rchart and Xchart

ON       - Pops back to main environment

RS X&R   -  Allows you to enter Xbar, Rbar and sample size

so trial control limits can be calculated. It also

calculated the estimate of the population based

on those parameters. note that the population sign

with the hat is the estimate of the population

=

[HIST] -  Plots histogram along with normal curve and tolerance

limits. Minimum bin size 6 maximum 23.

DIVIDE -  Print key

RS HIST - Enter your bins size then histogram is plotted

Hist uses variable m & S to plot normal curve

m is MEAN

s is PSDEV

=

[STAT]  - displays statistics.

%AUTL - percentage above Upper Tolerance Limit

%BLTL - percentage below Lower Tolerance Limit

%AUTL & %BLTL are estimated based on the normal curve

and assumes data is normally distributed

PCI is seen in some text as CPK which is called a Process

capability Index. A minimum value of 1.33 or greater

is desired for a process to be satisfactory

ON    - Pops back to previous display

DIVIDE - Print key

=

[SEND]  -  Exports data to your Computer. Pressing

Send will prompt "Press ENTER to EXPORT"

Invoke Kermit server mode on your Computer

and press ENTER on your 48 to transfer data.

If your Computer SPC software have a import

feature, you shouldn't have any problem

anaylzing data from your 48 on your computer

Format for data transfered to PC is a follows:

Horizontal will be your sample size & vertically

will be your subgroup size. I will include

different formating option depending on user

interest

=

[CRDIR] - Creates a directory in the current path

=

[COPY]  - Copies data to name user specifies on input.

you can still navigate through directories

all key assignment remains except ENTER.

Copy menu keys

[HERE]- Copies data to new name

[EXIT]- Returns to original directory

=

[MOVE]  - Move data to a new directory. you can still

navigate through directories

=

[EDIT]  - Edit data at highlight using matrix manager

=

[PURGE] - Deletes data at highlight

=

VII   Keyboard definition for Main display

================================

LS UP   - Move directory up

LS HOME - Goes to the home directory

LS DOWN - Page display down; Will also repeat

if depressed

LS UP   - page display up; Will also repeat if depressed

RS DOWN - Down to the end of display

RS UP   - Up to the beginning of display

ALPHA   - Search Key

RS ON   - off

ON      - Aborts program

BACKARROW  - Aborts program

SPC  - ?

ENTER -  Enter does three things:

If the highlight Bar is pointed to a directory, that

directory is selected. If the highlight Bar is pointed to

a array, you will enter the database environment. If

the highlight bar is pointed to a array without dbase

info, you will be prompt to create a header for that data.

Database Menus

--------------

[EDIT]  - Edit or allows entry of header info

[VIEW]  - View data too long for display the view

command have no scrolling capabilities.

The entire screen is viewed only

[INDX]  - Display entire field. ENTER selects current

data to the database. ON pops back to the

database environment

[TRACE] - Display all data names of highlighted field.

ENTER selects current data to the database.

ON pops back to the database environment

[EXIT]  - Pops back to previous browser with name of

highlighted

ON      - Pops back to previous browser

RS HOME - the key works with INDX and TRACE also. this key

pops back to the array directory browser.

LEFT ARROW - scroll left

RIGHT ARROW - Scroll right

VIII     If you use this program

========================

If you find yourself using this program on a regular basis ,

I would appreciate if you E-mail me explaining how & where you

use this program. if you E-mail me I will put you on my mailing

list and you will be the first to evaluate future versions of

this software. Thanks