Author: Frank Plate

Subj:   solving differential equations

Date: 1 April 1992

[Note: received by mail on a floppy.  Thanx, Frank!  -jkh-]

This program solves every common differential equation and system

(every potential and degree also with variable coefficients)

on repetitive use of the Runge-Kutta-algorithm.

* Attention :

���������������������������������������������������������ͻ

�  - avoid discontinuity on the sampled x-range !         �

�  - periodic functions require stepsize << one period!   �

���������������������������������������������������������ͼ

* It generates the following variables :

- 'degree'

- 'functions'    ( list of a deq-system )

- 'initvalues'

- 'xmax' 'xmin'

- 'h'            ( stepsize )

* y0 in this program equals y, y1 equals y', y2 equals y'' ect.

* The progam is controlled by menu and will take the value of the last

calculation when invalid or no input-data is entered at any step.

* the result can be found in �DAT :

x  y  y'  y''  y''' ...

1. values [[                         ]

2. values  [                         ]

....               ...

[                         ]]

* As result the second column of �DAT ( = y(x) ) is drawn.

y', y'' etc. can be plotted by changing YCOL.

���������������������ͻ

�     Example #1 :    �

���������������������ͼ

-------------------------------------------------------------------------

deq:      y'' = -y' - 1.25 * y           �    y (3/2*�)=1

�    y'(3/2*�)=0

solution: y = e^(0.25*(3*�-2*x)) * ( 0.5*cos(x) - sin(x) )

-------------------------------------------------------------------------

- press "DEQ"

- enter 2 when asked for degree

- enter highest derivative :          :�X(Y1)=Y2: '-Y1-1.25*Y0'

in 'functions' stored    :        { :�X(Y0):    'Y1'

:�X(Y1)=Y2: '-Y1-1.25*Y0' }

- enter initial values     :          :   x0 : '3/2*�'

:Y0(x0): 1

:Y1(x0): 0

stored in 'initvalues'   :          [ 4.71238898038 1 0 ]

- enter x-range            :          :xmin: 0

:xmax: '2*�'

- enter stepsize           :            '�/4'

���������������������ͻ

�     Example #2 :    �

���������������������ͼ

--------------------------------------------------------------------------

deq:        y''= ( 2 / y ) * (y')� - ( 2 / x ) * y'   �   y (1) = 1

�   y'(1) = 0.5

solution :  y  = 2 * x / ( 1 + x )

--------------------------------------------------------------------------

- press "DEQ"

- degree : 2

- enter the highest derivative :      :�X(Y1)=Y2: '2/Y0*Y1^2-2/X*Y1'

stored in 'functions'        :    { :�X(Y0):    'Y1'

:�X(Y1)=Y2: '2/Y0*Y1^2-2/X*Y1'  }

- enter initial values         :      :   x0 : 1

:Y0(x0): 1

:Y1(x0): 0.5

stored in 'initvalues'       :      [ 1  1  0.5 ]

- enter x-range                :      :xmin: 0

:xmax: 5

- enter stepsize               :       1.1

see what's happening when you change the x-range :

- press "DEQ"

- press "ENTER" three times

( 'degree' 'functions' 'initvalues' remain unchanged )

- enter x-range :                     :xmin: -2

:xmax: 3

- press "ENTER" ( leave 'h' unchanged )

Calculated values are quite accurate untill the program passes the

discontinuity x = -1. Values beyond x = -1 are calculated and stored in

�DAT but are not correct anymore.

If you now change stepsize 'h' to 0.5 you'll get in stack:

1: program killed: "infinite result"    and stuff in other levels.

This error is caused by y'' when y-values for x = 0 were calculated.

But values already stored in �DAT are correct.

���������������������ͻ

�     Example #3 :    �

���������������������ͼ

-------------------------------------------------------------------------

deq-system :    u''(t) = -v'(t)        �     u(0)  = 0  ;   u'(0) = 1

v''(t) = 4 * u'(t)     �     v(0)  = 0  ;   v'(0) = 0

solution :      u(t) = 0.5 * sin(2*t)

v(t) = 1 - cos(2*t)

-------------------------------------------------------------------------

substitution for the program :

Replace all functions except the derivatives of highest degree.

t   ->    X

u (t)  ->   Y0

u'(t)  ->   Y1

v (t)  ->   Y2

v'(t)  ->   Y3

with these new variables you get :

original deq-system         �  new deq-system for calculator

������������������������������������������������������������

�t(u)  =  u'                �        �X (Y0) =      Y1

�t(u') =  u'' =   - v'      �        �X (Y1) =    - Y3

�t(v)  =  v'                �        �X (Y2) =      Y3

�t(v') =  v'' = 4 * u'      �        �X (Y3) =  4 * Y1

- press "DEQ"

- degree = 4

- enter the deq-system :              :�X(Y3)=Y4: '4*Y1'

:�X(Y2)   : 'Y3'

( use PgDn )                     :�X(Y1)   : '-Y3'

:�X(Y0)   : 'Y1'

- enter initial values :              :   x0 : 0

:Y0(x0): 0

( use PgDn )                     :Y1(x0): 1

:Y2(x0): 0

:Y3(x0): 0

- enter x-range        :              :xmin: 0

:xmax: '�'

- enter stepsize       :               '�/10'

u(t) is plotted at the end of the program and v(t) can be found in

column 4 of �DAT.

**********************************************************************

As I've got only some of the GOODIES disks I don't know if someone has

already written a similar program.  [Note: See also DIFFEQ on Goodies

Disk #1 for another programmer's approach to differential equations.

-jkh-]

Although I didn't take much time to speed up the program I hope it

can be useful to someone. Don't forget this is only a numerical

guess process and therefore only useful to control the manually

calculated result  ( how to find out discontinuities or periodic

times before getting to know y(x) ??). But nonsymbolic results can

be calculated with any run-of-the-mill calculator as well.

Frank Plate

Im Hachegrund 3a

2808 Syke

Germany - W