Instituto Tecnológico de Morelia.

Ingeniería Electrónica 2000

NEW VERSION  LIB

ENGLISH

Routh-Hurwitz

User's manual.

Installation.

To lower to install the program it decompresses the file routh.zip, it installs the bookstore Routh.lib in your HP 48, and I list.

it passes all the files, except for this word document, to a directory of your HP48.

Execution.

To execute the program the variable ROUTH that is in the memory presses.

Use.

After the presentation you will have to introduce the polynomial in the same way that you introduce a polynomial in SOLVE POLY...

Example:

If the polynomial is 3s3+23s2-324 you will have to introduce [3 23 0 -324]

You will have to select the degree of the polynomial in this case it is of degree 3.

note: this program manages degree polynomials 2 up to 8.

Lastly you will see the values of the chart of Routh-Hurwitz:

In this case B1=42.26 and C1 = -324

line1       line2

s3         3                0

s2       23            324

s1       42.26

s0          -324

As in the line1 a negative number exists the system it is unstable, only when the constants of the fila1 are positive the system is stable.

For a degree system 8 with As8 + Bs7 + Cs6 + Ds5 + Es4 + Fs3 + Gs2 + Hs + I

line1	line2	line3	line4	line5

s8 	A	C	E	G	I

s7  	B	D	F	H

s6  	B1	B2	B3	B4

s5  	C1	C2	C3

s4  	D1	D2	D3

s3  	E1	E2

s2  	F1	F2

s1  	G1

s0  	H1

Warning: if the constants of the fila1 give 0 then the system will have to divide among zero (according to the formula of Routh-Hurwitz) and this will interrupt the program and it will show an infinite result, for what the system can be in a value near the stability or uncertainty.

Nueva Versión en libreria

(menos pasos)

ESPAÑOL

Criterio de estabilidad de Routh-Hurwitz

Manual de usuario.

Instalación.

Para bajar instalar el programa descomprime el archivo routh.zip, instala la librería Routh.lib en tu HP 48, y listo.

Ejecución.

Para ejecutar el programa presiona la variable ROUTH que está en la memoria.

Utilización.

Después de la presentación tendrás que introducir el polinomio de la misma forma que introduces un polinomio en SOLVE POLY...

Ejemplo:

Si el polinomio es 3s3+23s2-324  tendrás que introducir  [3 23 0 -324]

Tendrás que seleccionar el grado del polinomio en este caso es de grado 3.

Nota: este programa maneja polinomios de grado 2 hasta 8.

Por último verás los valores de la tabla de Routh-Hurwitz:

En este caso B1=42.26 y C1= -324

Fila1       Fila2

s3         3                0

s2       23            324

s1       42.26

s0          -324

Como en la fila1  existe un número negativo el sistema es inestable, sólo cuando las constantes de la fila1 son positivas el sistema es estable.

Para un sistema de grado 8 con As8 + Bs7 + Cs6 + Ds5 + Es4 + Fs3 + Gs2 + Hs + I

Fila1	fila2	fila3	fila4	fila5

s8 	A	C	E	G	I

s7  	B	D	F	H

s6  	B1	B2	B3	B4

s5  	C1	C2	C3

s4  	D1	D2	D3

s3  	E1	E2

s2  	F1	F2

s1  	G1

s0  	H1

Nota: si las constantes de la line1 dan 0 entonces el sistema tendrá que dividir entre cero (según la fórmula de Routh-Hurwitz) y esto interrumpirá el programa y mostrará un resultado infinito, por lo que el sistema puede estar en un valor cerca de la estabilidad o inestabilidad.

Écsar García Sandoval

Quiero Dar las gracias al Dr. José Antonio Gutiérrez Gnecchi por su explicación sobre este criterio.