## Index       Manjo

ÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄ

p Field Selection

p Pointer Registers

p Special Registers

p Status Flags

p Arithmetic Mode

p Loading Data from Memory

p Storing Data in Memory

p Mnemonics

ÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄo Field Selection

Subfields of the working registers A, B, C, and D may be

accessed by the use of field selection.  The possible field

selections range from the entire register to any single

nibble of the register.  Certain subfields are designed for

use in BCD calculations.  Others are used for data access or

general data manipulation.

Field Selection Description

P Nibble  P    indicated by P register

WP Nibbles 0-P  first (P+1) nibbles

XS Nibble  2    xponent sign

X Nibbles 2-0  exponent including exponent sign

S Nibble  15   mantissa sign

M Nibbles 14-3 mantissa

B Nibbles 1-0  byte

A Nibbles 4-0  address field (20 bits)

W Nibbles 15-0 word (entire 64-bit)

Register Nibbles

ÚÄÄÂÄÄÂÄÄÂÄÄÂÄÄÂÄÄÂÄÄÂÄÄÂÄÄÂÄÄÂÄÄÂÄÄÂÄÄÂÄÄÂÄÄÂÄÄ¿

³15|14|13|12|11|10| 9| 8| 7| 6| 5| 4| 3| 2| 1| 0³

ÃÄÄÁÄÄÁÄÄÁÄÄÁÄÄÁÄÄÁÄÄÁÄÄÁÄÄÁÄÄÁÄÄÁÄÄÁÄÄÁÄÄÁÄÄÁÄÄ´

³ S³                             ³     ³XS³  B  ³

³  ³<ÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄ M ÄÄÄÄÄÄÄÅÄÄÄÄ>³ <Ä X Ä>³

³                                ³<ÄÄÄÄÄ A ÄÄÄÄ>³

³<ÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄ W ÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄ>³

o Pointer Registers

The 20 bit Data Pointer registers D0 and D1 are used to

contain addresses during memory access, and are used in

conjunction with the working registers.

The 4 bit Pointer register P is used in Field Selection

operations with the working registers.

o Special Registers

The input/output registers are used to communicate with the

system bus.  The program counter points to the next

instruction to be executed by the CPU.  The input register

IN is 16 bits, the output register OUT is 12 bits, and the

PC register is 20 bits.

o Status Flags

The Carry bit is adjusted when a calculation or logical test

is performed.  During a calculation, such as incrementing or

decrementing a register, it is set if the calculation

overflows or borrows; otherwise it is cleared.  During a

logical test, such as comparing two registers for equality,

it is set if the test is true; otherwise it is cleared.

The upper 4 Program Status bits are typically used to

indicate the state of the operating system.  The remaining

12 Program Status bits are generally available to

applications software, and may be manipulated collectively

as the ST register.

The four Hardware Status bits are set (but not cleared) by

hardware-related events, and must therefore be cleared

beforehand in order to detect a particular occurrence.  They

are individually accessible by name.  The Module Pulled bit

(MP) is set whenever the *NINTX CPU input is pulled low

(regardless of whether an interrupt is actually executed).

The Sticky Bit (SB) is set when a non-zero bit shifts off

the right end of a working register as the result of a shift

instruction.  The Service Request (SR) bit is set as a

result of the SREQ?  instruction if any hardware service

request is pending.  The external Module Missing bit is set

by execution of a ``00'' opcode (RTNSXM instruction).

Hardware Status:  4 bits

Bit  Symbol  Name

ÄÄÄÁÄÄÄÄÄÄÄÁÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄ

3     MP    Module Pulled (*NINTX pulled low)

2     SR    Service Request

1     SB    Sticky Bit

0     XM    External Module Missing

o Arithmetic Mode

The arithmetic mode is set by the SETHEX and SETDEC

instructions.  When SETHEX is executed, the arithmetic mode

is set so that all register arithmetic is performed in

hexadecimal mode.  When SETDEC is executed, the arithmetic

mode is set so that most register arithmetic is performed in

decimal mode.  The following instructions are *always*

performed in hexadecimal mode, regardless of the arithmetic

mode setting:

P=P+1

P=P-1

C+P+1

D0=D0+     n   D0=D0-     n

D1=D1+     n   D1=D1-     n

A=A+CON fs,n   A=A-CON fs,n

B=B+CON fs,n   B=B-CON fs,n

C=C+CON fs,n   C=C-CON fs,n

D=D+CON fs,n   D=D-CON fs,n

The arithmetic mode is not ``readable'', but can be inferred

by doing an appropriate operation, followed by a test.  For

example:

LCHEX   9

C=C+1   P

sets the carry if, and only if, the arithmetic mode is

decimal.

o Loading Data from Memory

When data is read from memory into a register, the CPU

places the lowest addressed nibble in the least significant

nibble of the register.

For example, if the data shown below in memory is read into

the C register using the C=DAT1  4 instruction, the data in

the register will be arranged as shown.

Memory

Location  Value            C Register

ÄÄÄÄÄÄÄÄÁÄÄÄÄÄÄ      ÚÄÄÂÄÄÄÄÄÂÄÄÂÄÄÂÄÄÂÄÄ¿

1000      6        ³  | ... | 9| 8| 7| 6³

1001      7        ÀÄÄÁÄÄÄÄÄÁÄÄÁÄÄÁÄÄÁÄÄÙ

1002      8         15  ...   3  2  1  0

1003      9

This principle applies also to loading constants into a CPU

register such as C, D0, or D1, since the CPU must read the

constant from the instruction opcode in memory.  For

example, the instruction LCHEX 9876 produces the opcode

336789 and the C register is loaded as shown above

(assuming P= 0).

o Storing Data to Memory

When data is written from a register to memory, the CPU

places the least significant nibble of the register in the

lowest nibble of the addressed memory location.  For

example, if the data shown above in the C register is

written to memory using the DAT1=C  4 instruction, the data

will be written to memory as shown.

o Mnemonics

?A~=0  fs -  Test for A not equal to 0

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of A is not equal to 0.

Adjusts Carry.

?A~=B  fs -  Test for A not equal to B

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of A is not equal to the fs field of B.

Adjusts Carry.

?A~=C  fs -  Test for A not equal to C

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of A is not equal to the fs field of C.

Adjusts Carry.

?A<=B  fs -  Test for A less than or equal to B

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of A is less than or equal to the

fs field of B.

Adjusts Carry.

?A<B   fs -  Test for A less than B

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of A is less than the fs field of B.

Adjusts Carry.

?A=0   fs -  Test for A equal to 0

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of A is equal to 0.

Adjusts Carry.

?A=B   fs -  Test for A equal to B

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of A is equal to the fs field of B.

Adjusts Carry.

?A=C   fs -  Test for A equal to C

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of A is equal to the fs field of C.

Adjusts Carry.

?A>=B  fs -  Test for A greater than or equal to B

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of A is greater than or equal to

the fs field of B.

Adjusts Carry.

?A>B   fs -  Test for A greater than B

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of A is greater than the fs field of B.

Adjusts Carry.

?B~=0  fs -  Test for B not equal to 0

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of B is not equal to 0.

Adjusts Carry.

?B~=A  fs -  Test for B not equal to A

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of B is not equal to the fs field of A.

Adjusts Carry.

?B~=C  fs -  Test for B not equal to C

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of B is not equal to the fs field of C.

Adjusts Carry.

?B<=C  fs -  Test for B less than or equal to C

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of B is less than or equal to the

fs field of C.

Adjusts Carry.

?B<C   fs -  Test for B less than C

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of B is less than the fs field of C.

Adjusts Carry.

?B=0   fs -  Test for B equal to 0

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of B is equal to 0.

Adjusts Carry.

?B=A   fs -  Test for B equal to A

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of B is equal to the fs field of A.

Adjusts Carry.

?B=C   fs -  Test for B equal to C

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of B is equal to the fs field of C.

Adjusts Carry.

?B>=C  fs -  Test for B greater than or equal to C

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of B is greater than or equal to

the fs field of C.

Adjusts Carry.

?B>C   fs -  Test for B greater than C

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of B is greater than the fs field of C.

Adjusts Carry.

?C~=0  fs -  Test for C not equal to 0

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of C is not equal to 0.

Adjusts Carry.

?C~=A  fs -  Test for C not equal to A

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of C is not equal to the fs field of A.

Adjusts Carry.

?C~=B  fs -  Test for C not equal to B

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of C is not equal to the fs field of B.

Adjusts Carry.

?C~=D  fs -  Test for C not equal to D

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of C is not equal to the fs field of D.

Adjusts Carry.

?C<=A  fs -  Test for C less than or equal to A

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of C is less than or equal to the

fs field of A.

Adjusts Carry.

?C<A   fs -  Test for C less than A

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of C is less than the fs field of A.

Adjusts Carry.

?C=0   fs -  Test for C equal to 0

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of C is equal to 0.

Adjusts Carry.

?C=A   fs -  Test for C equal to A

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of C is equal to the fs field of A.

Adjusts Carry.

?C=B   fs -  Test for C equal to B

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of C is equal to the fs field of B.

Adjusts Carry.

?C=D   fs -  Test for C equal to D

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of C is equal to the fs field of D.

Adjusts Carry.

?C>=A  fs -  Test for C greater than or equal to A

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of C is greater than or equal to

the fs field of A.

Adjusts Carry.

?C>A   fs -  Test for C greater than A

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of C is greater than the fs field

of A.

Adjusts Carry.

?D~=0  fs -  Test for D not equal to 0

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of D is not equal to 0.

Adjusts Carry.

?D~=C  fs -  Test for D not equal to C

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of D is not equal to the fs field

of C.

Adjusts Carry.

?D<=C  fs -  Test for D less than or equal to C

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of D is less than or equal to the

fs field of C.

Adjusts Carry.

?D<C   fs -  Test for D less than to C

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of D is less than the fs field of C.

Adjusts Carry.

?D=0   fs -  Test for D equal to 0

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of D is equal to 0.

Adjusts Carry.

?D=C   fs -  Test for D equal to C

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of D is equal to the fs field of C.

Adjusts Carry.

?D>=C  fs -  Test for D greater than or equal to C

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of D is greater than or equal to

the fs field of C.

Adjusts Carry.

?D>C   fs -  Test for D greater than C

fs = A                       cycles:   13 + d (YES)

6 + d (NO)

fs = (P,WP,XS,X,S,M,B,W)     cycles:   13 + d (YES)

6 + d (NO)

Test whether the fs field of D is greater than the fs field

of C.

Adjusts Carry.

?MP=0  - Test Module Pulled bit (MP)

cycles:   13 (YES)

6 (NO)

Test whether the Module Pulled bit (MP) is zero.  This

hardware status bit is set whenever a module-pulled

interrupt occurs (the *INT line of the CPU is pulled high),

and must be explictly cleared by the MP=0 mnemonic. See the

"HP-71 Hardware Specification" for more information.

Adjusts Carry.

?P~=   n    - Test if P pointer not equal to n

cycles:   13 (YES)

6 (NO)

Test whether the P pointer is not equal to n.

Adjusts Carry.

?P=    n    - Test if P pointer is equal to n

cycles:   13 (YES)

6 (NO)

Test whether the P pointer is equal to n.

Adjusts Carry.

?SB=0  - Test Sticky Bit (SB)

cycles:   13 (YES)

6 (NO)

Test whether the Sticky Bit (SB) is zero. This hardware

status bit is set on right shifts when a non-zero nibble or

bit is shifted off the end of the field.  The Sticky Bit

must be cleared explicitly.

Adjusts Carry.

?SR=0  - Test Service Request bit (SR) for zero

cycles:   13 (YES)

6 (NO)

Test whether the Service Request bit (SR) is zero.  This

hardware status bit is set by the SREQ? mnemonic, and must

be cleared explicitly by the SR=0 instruciton.

Adjusts Carry.

?ST=0  n   - Test status bit n equal to 0

cycles:   14 (YES)

7 (NO)

Test whether Program Status bit n is clear.

Adjusts Carry.

?ST=1  n   - Test status bit n equal to 1

cycles:   14 (YES)

7 (NO)

Test whether Program Status bit n is set.

Adjusts Carry.

?XM=0  - Test External Module Missing bit (XM)

cycles:   13 (YES)

6 (NO)

Test the whether the External Module Missing bit (XM) is

zero.  This hardware status bit is set by the RTNSXM

mnemonic, and must be explicitly cleared by the XM=0

mnemonic.

Adjusts Carry.

A=-A   fs -  Two's complement of A into A

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Complement the specified fs field of A. Complement is two's

complement if in HEX mode, ten's complement if in DEC mode.

Carry is set if the field is not zero, else Carry is

cleared.

A=~A   fs -  One's complement of A into A

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Perform a one's complement on the specified fs field of A.

Carry is always cleared.

A=0    fs -  Set A equal to 0

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Set the specified fs field of A to zero.  Carry is not

affected.

A=A!B  fs -  A OR B into A

fs = A                       cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     cycles:    4 + d

Set the fs field of register A to its logical OR with the

corresponding field of register B.  Carry is not affected.

A=A!C  fs -  A OR C into A

fs = A                       cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     cycles:    4 + d

Set the fs field of register A to its logical OR with the

corresponding field of register C.  Carry is not affected.

A=A&B  fs -  A AND B into A

fs = A                       cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     cycles:    4 + d

Set the fs field of register A to its logical AND with the

corresponding field of register B.  Carry is not affected.

A=A&C  fs -  A AND C into A

fs = A                       cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     cycles:    4 + d

Set the fs field of register A to its logical AND with the

corresponding field of register C.  Carry is not affected.

A=A+1  fs -  Increment A

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Increment the specified fs field of register A by one.

Adjusts Carry.

A=A+A  fs -  Sum of A and A into A

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Double the specified fs field of register A. Adjusts Carry.

A=A+B  fs -  Sum of A and B into A

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Set the specified fs field of register A to the sum of

itself and the corresponding field of register B. Adjusts

Carry.

A=A+C  fs  -  Sum of A and C into A

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Set the specified fs field of register A to the sum of

itself and the corresponding field of register C. Adjusts

Carry.

A=A-1  fs -  Decrement A

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Decrement the specified fs field of register A by one.

Adjusts Carry.

A=A-B  fs -  A minus B into A

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Set the specified fs field of register A to the difference

between itself and the corresponding field of register B.

Adjusts Carry.

A=A-C  fs -  A minus C into A

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Set the specified fs field of register A to the difference

between itself and the corresponding field of register C.

Adjusts Carry.

A=B    fs -  Copy B to A

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Copy the fs field of register B into the corresponding field

of register A. Carry is not affected.

A=B-A  fs -  B minus A into A

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Set the specified fs field of register A to the inverse

difference between itself and the corresponding field of

register B. Adjusts Carry.

A=C    fs -  Copy C to A

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Copy the fs field of register C into the corresponding field

of register A. Carry is not affected.

A=DAT0 fsd -  Load A from memory

fs = A                       cycles:   18

fs = B                       cycles:   15

fs = (P,WP,XS,X,S,M,W)       cycles:   17 + d

fs = d                       cycles:   16 + d

The amount of data (d nibbles) specified by fsd will be

transferred from the memory address pointed to by D0 into

the specified field of register A.  The lowest-addressed

nibble will be transferred into the lowest-order nibble of

the register field, proceeding toward the higher-order

nibbles. If fs = d, d nibbles are transferred into the

register starting at nibble 0. See the section on "Loading

Data From Memory".

A=DAT1 fsd -  Load A from memory

fs = A                       cycles:   18

fs = B                       cycles:   15

fs = (P,WP,XS,X,S,M,W)       cycles:   17 + d

fs = d                       cycles:   16 + d

The amount of data (d nibbles) specified by fsd will be

transferred from the memory address pointed to by D1 into

the specified field of register A.  The lowest-addressed

nibble will be transferred into the lowest-order nibble of

the register field, proceeding toward the higher-order

nibbles. If fs = d, d nibbles are transferred into the

register starting at nibble 0. See the section on "Loading

Data From Memory".

A=IN   - Load A with IN      cycles:    7

Load the low-order 4 nibbles of the A register with the

contents of the Input register.

A=R0   - Copy R0 to A         cycles:   19

The contents of the scratch register R0 is copied to the

working register A.

A=R1   - Copy R1 to A         cycles:   19

The contents of the scratch register R1 is copied to the

working register A.

A=R2   - Copy R2 to A         cycles:   19

The contents of the scratch register R2 is copied to the

working register A.

A=R3   - Copy R3 to A         cycles:   19

The contents of the scratch register R3 is copied to the

working register A.

A=R4   - Copy R4 to A         cycles:   19

The contents of the scratch register R4 is copied to the

working register A.

ABEX   fs -  Exchange Registers A and B

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Exchange the fs fields of registers of A and B. Carry is not

affected.

ACEX   fs -  Exchange Registers A and C

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Exchange the fs fields of registers of A and C. Carry is not

affected.

AD0EX  - Exchange A and D0 (nibs 0-4)

cycles:    8

Exchange the A field of register A with Data pointer D0.

Carry is not affected.

AD0XS  - Exchange A and D0 short (nibs 0-3)

cycles:    7

Exchange the lower 4 nibbles of A with the lower 4 nibbles

of Data pointer D0.  Carry is not affected.

AD1EX  - Exchange A and D1 (nibs 0-4)

cycles:    8

Exchange the A field of register A with Data pointer D1.

Carry is not affected.

AD1XS  - Exchange A and D1 short (nibs 0-3)

cycles:    7

Exchange the lower 4 nibbles of A with the lower 4 nibbles

of Data pointer D1.  Carry is not affected.

AR0EX  - Exchange A and R0    cycles:   19

Exchange the contents of the working register A and the

scratch register R0.

AR1EX  - Exchange A and R1    cycles:   19

Exchange the contents of the working register A and the

scratch register R1.

AR2EX  - Exchange A and R2    cycles:   19

Exchange the contents of the working register A and the

scratch register R2.

AR3EX  - Exchange A and R3    cycles:   19

Exchange the contents of the working register A and the

scratch register R3.

AR4EX  - Exchange A and R4    cycles:   19

Exchange the contents of the working register A and the

scratch register R4.

ASL    fs -  A Shift Left

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Shift the contents of the specified fs field of register A

left one nibble, without affecting the rest of the register.

The nibble shifted off the left end of the field is lost.

The new low-order nibble of the field is zero. The Sticky

Bit (SB) is not affected.

ASLC   - A Shift Left Circular

cycles:   21

Circular shift register A left one nibble.  Operates on all

16 digits.  The Sticky Bit (SB) is not affected.

ASR    fs -  A Shift Right

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Shift the contents of the specified fs field of register A

right one nibble, without affecting the rest of the

register. The nibble shifted off the right end of the field

is lost, but the Sticky Bit (SB) is set if the nibble was

non-zero.  The new high-order nibble of the field is zero.

ASRB   - A Shift Right Bit

cycles:   20

Shift register A right one bit.  Operates on all 16 digits.

The bit shifted off the end is lost, but the Sticky Bit (SB)

is set if it was non-zero.  The new high-order bit of the

register is zero.

ASRC   - A Shift Right Circular

cycles:   21

Circular shift register A right one nibble.  Operates on all

16 digits. The Sticky Bit (SB) is set if the nibble shifted

from low-order around to high-order position was non-zero.

B=-B   fs -  Two's complement of B into B

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Complement the specified fs field of B. Complement is two's

complement if in HEX mode, ten's complement if in DEC mode.

Carry is set if the field is not zero, else Carry is

cleared.

B=~B   fs -  One's complement of B into B

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Perform a one's complement on the specified fs field of B.

Carry is always cleared.

B=0    fs -  Set B equal to 0

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Set the specified fs field of B to zero.  Carry is not

affected.

B=A    fs -  Copy A to B

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Copy the fs field of register A into the corresponding field

of register B. Carry is not affected.

B=B!A  fs -  B OR A into B

fs = A                       cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     cycles:    4 + d

Set the fs field of register B to its logical OR with the

corresponding field of register A.  Carry is not affected.

B=B!C  fs -  B OR C into B

fs = A                       cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     cycles:    4 + d

Set the fs field of register B to its logical OR with the

corresponding field of register C.  Carry is not affected.

B=B&A  fs -  B AND A into B

fs = A                       cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     cycles:    4 + d

Set the fs field of register B to its logical AND with the

corresponding field of register A.  Carry is not affected.

B=B&C  fs -  B AND C into B

fs = A                       cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     cycles:    4 + d

Set the fs field of register B to its logical AND with the

corresponding field of register C.  Carry is not affected.

B=B+1  fs -  Increment B

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Increment the specified fs field of register B by one.

Adjusts Carry.

B=B+A  fs -  Sum of B and A into B

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Set the specified fs field of register B to the sum of

itself and the corresponding field of register A. Adjusts

Carry.

B=B+B  fs -  Sum of B and B into B

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Double the specified fs field of register B. Adjusts Carry.

B=B+C  fs -  Sum of B and C into B

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Set the specified fs field of register B to the sum of

itself and the corresponding field of register C. Adjusts

Carry.

B=B-1  fs -  Decrement B

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Decrement the specified fs field of register B by one.

Adjusts Carry.

B=B-A  fs -  B minus A into B

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Set the specified fs field of register B to the difference

between itself and the corresponding field of register A.

Adjusts Carry.

B=B-C  fs -  B minus C into B

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Set the specified fs field of register B to the difference

between itself and the corresponding field of register C.

Adjusts Carry.

B=C    fs -  Copy C to B

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Copy the fs field of register C into the corresponding field

of register B. Carry is not affected.

B=C-B  fs -  C minus B into B

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Set the specified fs field of register B to the inverse

difference between itself and the corresponding field of

register C. Adjusts Carry.

BAEX   fs -  Exchange Registers B and A

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Exchange the fs fields of registers of B and A. Carry is not

affected.

BCEX   fs -  Exchange Registers B and C

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Exchange the fs fields of registers of B and C. Carry is not

affected.

BSL    fs -  B Shift Left

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Shift the contents of the specified fs field of register B

left one nibble, without affecting the rest of the register.

The nibble shifted off the left end of the field is lost.

The new low-order nibble of the field is zero. The Sticky

Bit (SB) is not affected.

BSLC   - B Shift Left Circular

cycles:   21

Circular shift register B left one nibble.  Operates on all

16 digits.  The Sticky Bit (SB) is not affected.

BSR    fs -  B Shift Right

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Shift the contents of the specified fs field of register B

right one nibble, without affecting the rest of the

register. The nibble shifted off the right end of the field

is lost, but the Sticky Bit (SB) is set if the nibble was

non-zero.  The new high-order nibble of the field is zero.

BSRB   - B Shift Right Bit

cycles:   20

Shift register B right one bit.  Operates on all 16 digits.

The bit shifted off the end is lost, but the Sticky Bit (SB)

is set if it was non-zero.  The new high-order bit of the

register is zero.

BSRC   - B Shift Right Circular

cycles:   21

Circular shift register B right one nibble.  Operates on all

16 digits. The Sticky Bit (SB) is set if the nibble shifted

from low-order around to high-order position was non-zero.

BUSCC  - Bus Command "C"

cycles:    6

Enters the Saturn bus command "C" onto the system bus (this

command is reserved for later use).  No other operation is

performed.  See the "HP-71 Hardware Specification" for more

information.

C+=P+1 - Increment C by One Plus P Pointer

cycles:    8

The A field of the C register is incremented by one plus the

value of the P pointer.  This instruction is always executed

in HEX mode.  Adjusts Carry.

C=-C   fs -  Two's complement of C into C

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Complement the specified fs field of C. Complement is two's

complement if in HEX mode, ten's complement if in DEC mode.

Carry is set if the field is not zero, else Carry is

cleared.

C=~C   fs -  One's complement of C into C

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Perform a one's complement on the specified fs field of C.

Carry is always cleared.

C=0    fs -  Set C equal to 0

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Set the specified fs field of C to zero.  Carry is not

affected.

C=A    fs -  Copy A to C

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Copy the fs field of register A into the corresponding field

of register C. Carry is not affected.

C=A-C  fs -  A minus C into C

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Set the specified fs field of register C to the inverse

difference between itself and the corresponding field of

register A. Adjusts Carry.

C=B    fs -  Copy B to C

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Copy the fs field of register B into the corresponding field

of register C. Carry is not affected.

C=C!A  fs -  C OR A into C

fs = A                       cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     cycles:    4 + d

Set the fs field of register C to its logical OR with the

corresponding field of register A.  Carry is not affected.

C=C!B  fs  -  C OR B into C

fs = A                       cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     cycles:    4 + d

Set the fs field of register C to its logical OR with the

corresponding field of register B.  Carry is not affected.

C=C!D  fs -  C OR D into C

fs = A                       cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     cycles:    4 + d

Set the fs field of register C to its logical OR with the

corresponding field of register D.  Carry is not affected.

C=C&A  fs -  C AND A into A

fs = A                       cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     cycles:    4 + d

Set the fs field of register C to its logical AND with the

corresponding field of register A.  Carry is not affected.

C=C&B  fs -  C AND B into C

fs = A                       cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     cycles:    4 + d

Set the fs field of register C to its logical AND with the

corresponding field of register B.  Carry is not affected.

C=C&D  fs -  C AND D into C

fs = A                       cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     cycles:    4 + d

Set the fs field of register C to its logical AND with the

corresponding field of register D.  Carry is not affected.

C=C+1  fs -  Increment C

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Increment the specified fs field of register C by one.

Adjusts Carry.

C=C+A  fs - Sum of C and A into C

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Set the specified fs field of register C to the sum of

itself and the corresponding field of register A. Adjusts

Carry.

C=C+B  fs -  Sum of C and B into C

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Set the specified fs field of register C to the sum of

itself and the corresponding field of register B. Adjusts

Carry.

C=C+C  fs - Sum of C and C into C

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Double the specified fs field of register C. Adjusts Carry.

C=C+D  fs -  Sum of C and D into C

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Set the specified fs field of register C to the sum of

itself and the corresponding field of register D. Adjusts

Carry.

C=C-1  fs -  Decrement C

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Decrement the specified fs field of register C by one.

Adjusts Carry.

C=C-A  fs -  C minus A into C

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Set the specified fs field of register C to the difference

between itself and the corresponding field of register A.

Adjusts Carry.

C=C-B  fs -  C minus B into C

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Set the specified fs field of register C to the difference

between itself and the corresponding field of register B.

Adjusts Carry.

C=C-D  fs -  C minus D into C

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Set the specified fs field of register C to the difference

between itself and the corresponding field of register D.

Adjusts Carry.

C=D    fs -  Copy D to C

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Copy the fs field of register D into the corresponding field

of register C. Carry is not affected.

C=DAT0 fsd -  Load C from memory

fs = A                       cycles:   18

fs = B                       cycles:   15

fs = (P,WP,XS,X,S,M,W)       cycles:   17 + d

fs = d                       cycles:   16 + d

The amount of data (d nibbles) specified by fsd will be

transferred from the memory address pointed to by D0 into

the specified field of register C.  The lowest-addressed

nibble will be transferred into the lowest-order nibble of

the register field, proceeding toward the higher-order

nibbles. If fs = d, d nibbles are transferred into the

register starting at nibble 0. See the section on "Loading

Data From Memory".

C=DAT1 fsd -  Load C from memory

fs = A                       cycles:   18

fs = B                       cycles:   15

fs = (P,WP,XS,X,S,M,W)       cycles:   17 + d

fs = d                       cycles:   16 + d

The amount of data (d nibbles) specified by fsd will be

transferred from the memory address pointed to by D1 into

the specified field of register C.  The lowest-addressed

nibble will be transferred into the lowest-order nibble of

the register field, proceeding toward the higher-order

nibbles. If fs = d, d nibbles are transferred into the

register starting at nibble 0. See the section on "Loading

Data From Memory".

C=ID   - Request chip ID      cycles:   11

The chip which has its DAISY-IN line high and its

configuration flag low will send its 5 nibble ID register to

the system bus which will be loaded into the low-order 5

nibbles (A field) of the C register.

C=IN   - Load C with IN       cycles:    7

Load the low-order 4 nibbles of the C register with the

contents of the Input register.

C=P    n    - Copy P Pointer into Nibble n of C

cycles:    6

Copy P pointer into C register at digit position specified

by n.

C=R0   - Copy R0 to C         cycles:   19

The contents of the scratch register R0 is copied to the

working register C.

C=R1   - Copy R1 to C         cycles:   19

The contents of the scratch register R1 is copied to the

working register C.

C=R2   - Copy R2 to C         cycles:   19

The contents of the scratch register R2 is copied to the

working register C.

C=R3   - Copy R3 to C         cycles:   19

The contents of the scratch register R3 is copied to the

working register C.

C=R4   - Copy R4 to C          cycles:   19

The contents of the scratch register R4 is copied to the

working register C.

C=RSTK - Pop stack to C       cycles:    8

Pop the top-most address off of the hardware return stack,

placing the address in the lower 5 nibbles (A field) of

register C. The high-order nibbles of C are unchanged.  As

the address is popped from the return stack, a zero address

is inserted at the bottom of the stack.  Compare with the

RTN mnemonic.

C=ST   - Status to C              cycles:    6

Copy the low-order 12 bits of the status register into the

low-order 12 bits (X field) of the C register.

CAEX   fs -  Exchange Registers C and A

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Exchange the fs fields of registers of C and A. Carry is not

affected.

CBEX   fs -  Exchange Registers C and B

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Exchange the fs fields of registers of C and B. Carry is not

affected.

CD0EX  - Exchange C and D0 (nibs 0-4)

cycles:    8

Exchange the A field of register C with Data pointer D0.

Carry is not affected.

CD0XS  - Exchange C and D0 short (nibs 0-3)

cycles:    7

Exchange the lower 4 nibbles of C with the lower 4 nibbles

of Data pointer D0.  Carry is not affected.

CD1EX  - Exchange C and D1 (nibs 0-4)

cycles:    8

Exchange the A field of register C with Data pointer D1.

Carry is not affected.

CD1XS  - Exchange C and D1 short (nibs 0-3)

cycles:    7

Exchange the lower 4 nibbles of C with the lower 4 nibbles

of Data pointer D1.  Carry is not affected.

CDEX   fs -  Exchange Registers C and D

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Exchange the fs fields of registers of C and D. Carry is not

affected.

CLRHST - Clear Hardware Status bits

cycles:    3

Clears the 4 Hardware Status bits XM, SB, SR and MP.  Note

that the opcode is actually 82x, where x is merely a mask

for which Hardware Status bits to clear, as follows:

bit

0 -  External Module Missing (see XM=0 mnemonic)

1 -  Sticky Bit              (see SB=0 mnemonic)

2 -  Service Request         (see SR=0 mnemonic)

3 -  Module Pulled           (see MP=0 mnemonic)

For example opcode 829 clears XM and MP.  Although there is

no mnemonic for this, the opcode can be inserted into the

code by using, for example, NIBHEX  829.

CLRST  - Clear Program Status

cycles:    6

Clear the low-order 12 bits (S0 through S11) of the Program

Status register ST.

CONFIG - Configure

cycles:   11

Copy the low-order 5 nibbles (A field) of the C register

into the Configuration register of the chip which has its

DAISY-IN line high and its configuration flag low. See the

"HP-71 Hardware Specification" for information.

CPEX   n    - Exchange Nibble n of C With P Pointer

cycles:    6

Exchange the P pointer with digit n of the C register.

CR0EX  - Exchange C and R0    cycles:   19

Exchange the contents of the working register C and the

scratch register R0.

CR1EX  - Exchange C and R1    cycles:   19

Exchange the contents of the working register C and the

scratch register R1.

CR2EX  - Exchange C and R2   cycles:   19

Exchange the contents of the working register C and the

scratch register R2.

CR3EX  - Exchange C and R3   cycles:   19

Exchange the contents of the working register C and the

scratch register R3.

CR4EX  - Exchange C and R4   cycles:   19

Exchange the contents of the working register C and the

scratch register R4.

CSL    fs -  C Shift Left

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Shift the contents of the specified fs field of register C

left one nibble, without affecting the rest of the register.

The nibble shifted off the left end of the field is lost.

The new low-order nibble of the field is zero. The Sticky

Bit (SB) is not affected.

CSLC   - C Shift Left Circular

cycles:   21

Circular shift register C left one nibble.  Operates on all

16 digits.  The Sticky Bit (SB) is not affected.

CSR    fs -  C Shift Right

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Shift the contents of the specified fs field of register C

right one nibble, without affecting the rest of the

register. The nibble shifted off the right end of the field

is lost, but the Sticky Bit (SB) is set if the nibble was

non-zero.  The new high-order nibble of the field is zero.

CSRB   - C Shift Right Bit

cycles:   20

Shift register C right one bit.  Operates on all 16 digits.

The bit shifted off the end is lost, but the Sticky Bit (SB)

is set if it was non-zero.  The new high-order bit of the

register is zero.

CSRC   - C Shift Right Circular

cycles:   21

Circular shift register C right one nibble.  Operates on all

16 digits. The Sticky Bit (SB) is set if the nibble shifted

from low-order around to high-order position was non-zero.

CSTEX  - Exchange Status

cycles:    6

Exchange the low-order 12 bits (S0 through S11) of the

Program Status register ST with the low-order 12 bits of the

C register.

D0=HEX2 nn    - Load 2 Nibbles Into D0

cycles:    4

Load the low-order two nibbles of D0 with nn. The upper

nibbles of D0 remain unchanged. Any overflow is ignored by

the assembler. The assembled digits of nn are stored in the

opcode in reverse order so that when the instruction is

executed the data will be loaded into the register with the

intended orientation.  See the section on "Loading Data From

Memory".

D0=HEX4 nnnn  - Load 4 Nibbles Into D0

cycles:    6

Load the low-order four nibbles of D0 with nnnn.  The upper

nibble of D0 remains unchanged. Any overflow is ignored by

the assembler. The assembled digits of nnnn are stored in

the opcode in reverse order so that when the instruction is

executed the data will be loaded into the register with the

intended orientation.  See the section on "Loading Data From

Memory".

D0=HEX5 nnnnn - Load 5 Nibbles Into D0

cycles:    7

Load all five nibbles of D0 with nnnnn. Any overflow is

ignored by the assembler. The assembled digits of nnnnn are

stored in the opcode in reverse order so that when the

instruction is executed the data will be loaded into the

register with the intended orientation.  See the section on

"Loading Data From Memory".

D0=A   - Copy A to D0 (nibs 0-4)

cycles:    8

The A field of register A is copied into Data pointer

register D0.  Carry is not affected.

D0=AS  - Copy A to D0 short (nibs 0-3)

cycles:    7

The lower 4 nibbles of A are copied into the lower 4 nibbles

of Data pointer register D0.  Carry is not affected.

D0=C   - Copy C to D0 (nibs 0-4)

cycles:    8

The A field of register C is copied into Data pointer

register D0.  Carry is not affected.

D0=CS  - Copy C to D0 short (nibs 0-3)

cycles:    7

The lower 4 nibbles of C are copied into the lower 4 nibbles

of Data pointer register D0.  Carry is not affected.

D0=D0+  n   - Add n to D0 (1<=n<=16)

cycles:    7

Increment D0 by n.  This instruction is always executed in

HEX mode.  Adjusts Carry.

D0=D0-  n   - Subtract n from D0 (1<=n<=16)

cycles:    7

Decrement D0 by n.  This instruction is always executed in

HEX mode.  Adjusts Carry.

D1=HEX2 nn    - Load 2 Nibbles Into D1

cycles:    4

Load the low-order two nibbles of D1 with nn. The upper

nibbles of D1 remain unchanged. Any overflow is ignored by

the assembler. The assembled digits of nn are stored in the

opcode in reverse order so that when the instruction is

executed the data will be loaded into the register with the

intended orientation.  See the section on "Loading Data From

Memory".

D1=HEX4 nnnn  - Load 4 Nibbles Into D1

cycles:    6

Load the low-order four nibbles of D1 with nnnn. The upper

nibble of D1 remains unchanged.  Any overflow is ignored by

the assembler. The assembled digits of nnnn are stored in

the opcode in reverse order so that when the instruction is

executed the data will be loaded into the register with the

intended orientation.  See the section on "Loading Data From

Memory".

D1=HEX5 nnnnn - Load 5 Nibbles Into D1

cycles:    7

Load all five nibbles of D1 with nnnnn. Any overflow is

ignored by the assembler. The assembled digits of nnnnn are

stored in the opcode in reverse order so that when the

instruction is executed the data will be loaded into the

register with the intended orientation.  See the section on

"Loading Data From Memory".

D1=A   - Copy A to D1 (nibs 0-4)

cycles:    8

The A field of register A is copied into Data pointer

register D1.  Carry is not affected.

D1=AS  - Copy A to D1 short (nibs 0-3)

cycles:    7

The lower 4 nibbles of A are copied into the lower 4 nibbles

of Data pointer register D1.  Carry is not affected.

D1=C   - Copy C to D1 (nibs 0-4)

cycles:    8

The A field of register C is copied into Data pointer

register D1.  Carry is not affected.

D1=CS  - Copy C to D1 short (nibs 0-3)

cycles:    7

The lower 4 nibbles of C are copied into the lower 4 nibbles

of Data pointer register D1.  Carry is not affected.

D1=D1+  n  - Add n to D1 (1<=n<=16)

cycles:    7

Increment D1 by n.  This instruction is always executed in

HEX mode.  Adjusts Carry.

D1=D1-  n  - Subtract n from D1 (1<=n<=16)

cycles:    7

Decrement D1 by n.  This instruction is always executed in

HEX mode.  Adjusts Carry.

D=-D   fs -  Two's complement of D into D

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Complement the specified fs field of D. Complement is two's

complement if in HEX mode, ten's complement if in DEC mode.

Carry is set if the field is not zero, else Carry is

cleared.

D=~D   fs -  One's complement of D into D

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Perform a one's complement on the specified fs field of D.

Carry is always cleared.

D=0    fs -  Set D equal to 0

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Set the specified fs field of D to zero.  Carry is not

affected.

D=C    fs -  Copy C to D

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Copy the fs field of register C into the corresponding field

of register D. Carry is not affected.

D=C-D  fs -  C minus D into D

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Set the specified fs field of register D to the inverse

difference between itself and the corresponding field of

register C. Adjusts Carry.

D=D!C  fs  -  D OR C into D

fs = A                       cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     cycles:    4 + d

Set the fs field of register D to its logical OR with the

corresponding field of register C.  Carry is not affected.

D=D&C  fs  -  D AND C into D

fs = A                       cycles:    4 + d

fs = (P,WP,XS,X,S,M,B,W)     cycles:    4 + d

Set the fs field of register D to its logical AND with the

corresponding field of register C.  Carry is not affected.

D=D+1  fs -  Increment D

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Increment the specified fs field of register D by one.

Adjusts Carry.

D=D+C  fs -  Sum of D and C into D

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Set the specified fs field of register D to the sum of

itself and the corresponding field of register C. Adjusts

Carry.

D=D+D  fs -  Sum of D and D into D

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Double the specified fs field of register D. Adjusts Carry.

D=D-1  fs -  Decrement D

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Decrement the specified fs field of register D by one.

Adjusts Carry.

D=D-C  fs -  D minus C into D

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Set the specified fs field of register D to the difference

between itself and the corresponding field of register C.

Adjusts Carry.

DAT0=A fsd -  Load memory from A

fs = A                       cycles:   17

fs = B                       cycles:   14

fs = (P,WP,XS,X,S,M,W)       cycles:   16 + d

fs = d                       cycles:   15 + d

The amount of data (d nibbles) specified by fsd will be

written to the memory address pointed to by D0 from the

specified field of register A.  The lowest-order nibble of

the register field will be written to the lowest-addressed

nibble of memory, proceeding toward the higher-order

nibbles. If fs = d, d nibbles are written to memory starting

from nibble 0 of the register. See the section on "Storing

Data Into Memory".

DAT0=C fsd  -  Store into memory from C

fs = A                       cycles:   17

fs = B                       cycles:   14

fs = (P,WP,XS,X,S,M,W)       cycles:   16 + d

fs = d                       cycles:   15 + d

The amount of data (d nibbles) specified by fsd will be

written to the memory address pointed to by D0 from the

specified field of register C.  The lowest-order nibble of

the register field will be written to the lowest-addressed

nibble of memory, proceeding toward the higher-order

nibbles. If fs = d, d nibbles are written to memory starting

from nibble 0 of the register. See the section on "Storing

Data Into Memory".

DAT1=A fs  -  Store into memory from A

fs = A                       cycles:   17

fs = B                       cycles:   14

fs = (P,WP,XS,X,S,M,W)       cycles:   16 + d

fs = d                       cycles:   15 + d

The amount of data (d nibbles) specified by fsd will be

written to the memory address pointed to by D1 from the

specified field of register A.  The lowest-order nibble of

the register field will be written to the lowest-addressed

nibble of memory, proceeding toward the higher-order

nibbles. If fs = d, d nibbles are written to memory starting

from nibble 0 of the register. See the section on "Storing

Data Into Memory".

DAT1=C fsd -  Store into memory from C

fs = A                       cycles:   17

fs = B                       cycles:   14

fs = (P,WP,XS,X,S,M,W)       cycles:   16 + d

fs = d                       cycles:   15 + d

The amount of data (d nibbles) specified by fsd will be

written to the memory address pointed to by D1 from the

specified field of register C.  The lowest-order nibble of

the register field will be written to the lowest-addressed

nibble of memory, proceeding toward the higher-order

nibbles. If fs = d, d nibbles are written to memory starting

from nibble 0 of the register. See the section on "Storing

Data Into Memory".

DCEX   fs -  Exchange Registers D and C

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Exchange the fs fields of registers of D and C. Carry is not

affected.

DSL    fs -  D Shift Left

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Shift the contents of the specified fs field of register D

left one nibble, without affecting the rest of the register.

The nibble shifted off the left end of the field is lost.

The new low-order nibble of the field is zero. The Sticky

Bit (SB) is not affected.

DSLC   - D Shift Left Circular

cycles:   21

Circular shift register D left one nibble.  Operates on all

16 digits.  The Sticky Bit (SB) is not affected.

DSR    fs -  D Shift Right

fs = A                       cycles:    7

fs = (P,WP,XS,X,S,M,B,W)     cycles:    3 + d

Shift the contents of the specified fs field of register D

right one nibble, without affecting the rest of the

register. The nibble shifted off the right end of the field

is lost, but the Sticky Bit (SB) is set if the nibble was

non-zero.  The new high-order nibble of the field is zero.

DSRB   - D Shift Right Bit

cycles:   20

Shift register D right one bit.  Operates on all 16 digits.

The bit shifted off the end is lost, but the Sticky Bit (SB)

is set if it was non-zero.  The new high-order bit of the

register is zero.

DSRC   - D Shift Right Circular

cycles:   21

Circular shift register D right one nibble.  Operates on all

16 digits. The Sticky Bit (SB) is set if the nibble shifted

from low-order around to high-order position was non-zero.

GOC    label - Go relative on carry

cycles:   10 (GO)

3 (NO)

Short relative jump to label if Carry is set. label must be

in the range:

addr - 128  <=  label  <=  addr + 127

where addr is the address of the second nibble of the

opcode. The address offset aa is in two's complement form

and is relative to addr.

GOLONG label  - Go Long

cycles:   14

Long relative jump to label unconditionally.  label must be

in the range:

addr - 32768  <=  label  <=  addr + 32767

where addr is the address of the third nibble of the opcode.

The address offset aaaa is in two's complement form and is

relative to addr.

GONC   label - Go relative on no carry

cycles:   10 (GO)

3 (NO)

Short relative jump to label if Carry is clear. label must

be in the range:

addr - 128  <=  label  <=  addr + 127

where addr is the address of the second nibble of the

opcode. The address offset aa is in two's complement form

and is relative to addr.

GOSUBVL label   - Gosub very long to label

cycles:   15

Absolute subroutine jump to aaaaa, which is the absolute

address of label.  See the GOSUB mnemonic.

GOSUB  label - Gosub to label

cycles:   12

Relative subroutine jump to label.  label must be in the

range:

addr - 2048 <= label <= addr + 2047

where addr is the starting address of the next instruction.

The address offset aaa is in two's complement form and is

relative to addr.

As with all subroutine jumps, the address (addr) of the

instruction following the gosub opcode is pushed onto the

hardware return stack, so that when a corresponding return

is executed, control resumes with the instruction at address

addr.

As the return address is pushed onto the return stack, the

bottom-most address on the stack is discarded.  Therefore,

the return stack always contains 8 addresses, and if pushes

exceed pops by 8 levels, the bottom-most return addresses

are lost. Since the interrupt system requires one level to

process interrupts, only 7 levels of the return stack can be

used by code which must execute when interrupts are enabled.

See the RTN mnemonic for further information.

GOSUBL label   - Gosub long to label

cycles:   15

Long relative subroutine jump to label.  label must be in

the range:

addr - 32768  <= label <= addr + 32767

where addr is the starting address of the next instruction.

The address offset aaaa is in two's complement form and is

relative to addr. See the GOSUB mnemonic.

GOTO   label - Jump relative

cycles:   11

Relative jump to label unconditionally. label must be in the

range:

addr - 2048  <=  label  <=  addr + 2047

where addr is the address of the second nibble of the

opcode. The address offset aaa is in two's complement form

and is relative to addr.

GOVLONG label  - Jump very long

cycles:   14

Unconditional jump to aaaaa, which is the absolute address

of label.

INTOFF - Interrupt Off        cycles:    5

Disable the keyboard interrupt system.

INTON  - Interrupt On         cycles:    5

Enable the keyboard interrupt system. See the "Hp-71

Hardware Specification" for more information.

LCHEX  h...h - Load C with hex constan

cycles:    4+n

Load up to 16 hex digits into the C register beginning at

the P pointer position, and proceeding toward higher-order

nibbles, with the ability to wrap around the register.  The

hex digits are stored in the opcode in reverse order so that

when the instruction is executed the data will be loaded

into the register with the intended orientation.  See the

section on "Loading Data From Memory".

MP=0   - Clear Module Pulled bit (MP)

cycles:    3

Clears the Module Pulled bit (MP) and pulls the Module

Pulled Interrupt line low. See CLRHST mnemonic.

NOP3   - Three nibble No-op

cycles:  10 (YES)

3 (NO)

This mnemonic generates a GOC to the next instruction,

effectively skiping three nibbles.

NOP4   - Four nibble No-op

cycles:   11

This mnemonic generates a GOTO to the next instruction,

efectively skiping four nibbles.

NOP5   - Five nibble No-op

cycles:    11

This mnemonic generates a relative GOTO to +4 nibbles. The

fifth nibble in the opcode is a place holder and is jumped

over.  The mnemonic effectively skips five nibbles.

OUT=C  - Load 3 nibbles of OR

cycles:    6

All nibbles of the Output register are loaded with the low-

order three nibbles of C (X field).

OUT=CS - Load 1 nibble of OR

cycles:    4

The least significant nibble of the Output register is

loaded with the least significant nibble of the C register.

PC=(A) - Jump (Set PC) indirectly thru A field of A register

cycles:    23

This instruction causes the CPU to jump to the address

pointed to by memory at the address specified by the A field

of the A register.  In symbolic form, the operation is

PC=mem(A[A]).  This opcode is not available on the 1LF2

version of the Saturn CPU.

P=C    n    - Copy P pointer into C at Nibble n

cycles:    6

Copy nibble n of register C into the P pointer.

P=P+1  - Increment P Pointer

cycles:    3

Increment the P pointer.  If P is incremented past F it will

automatically wrap around to 0. This instruction is always

executed in HEX mode.  Adjusts carry.

P=P-1  - Decrement P Pointer

cycles:    3

Decrement the P pointer. If P is decremented past 0 it

automatically wraps around to F. This instruction is always

executed in HEX mode.  Adjusts Carry.

P=     n    - Set P Pointer to n

cycles:    2

Set the P pointer to n.

R0=A   - Copy A to register R0

cycles:   19

The contents of the working register A is copied to the

scratch register R0.

R0=C   - Copy C to register R0

cycles:   19

The contents of the working register C is copied to the

scratch register R0.

R1=A   - Copy A to register R1

cycles:   19

The contents of the working register A is copied to the

scratch register R1.

R1=C   - Copy C to register R1

cycles:   19

The contents of the working register C is copied to the

scratch register R1.

R2=A   - Copy A to register R2

cycles:   19

The contents of the working register A is copied to the

scratch register R2.

R2=C   - Copy C to register R2

cycles:   19

The contents of the working register C is copied to the

scratch register R2.

R3=A   - Copy A to register R3

cycles:   19

The contents of the working register A is copied to the

scratch register R3.

R3=C   - Copy C to register R3

cycles:   19

The contents of the working register C is copied to the

scratch register R3.

R4=A   - Copy A to register R4

cycles:   19

The contents of the working register A is copied to the

scratch register R4.

R4=C   - Copy C to register R4

cycles:   19

The contents of the working register C is copied to the

scratch register R4.

RESET  - System reset

cycles:    6

The System Reset Bus Command is issued with all chips

performing a local reset.  The reset function will vary

according to the chip type.

RSI    - Reset Interrupt System

cycles:    6

This instruction causes CPU to consider any input line (ie

input register bits) presently high as a new interrupt.  If

the CPU is presently in the interrupt routine it will wait

for an RTI before vectoring, otherwise the CPU will vector

immediately following the RSI instruction.  For a complete

discussion on the interrupt system see the CPU Hardware

Specification (A-1LK7-9005-1).  This instruction is not

available on the 1LF2 version of the Saturn CPU.

RSTK=C - Push C to Return Stack

cycles:    8

Push the low-order 5 nibbles (A field) of the C register

onto the Return Stack. See the GOSUB mnemonic.

RTI    - Return from interrupt

cycles:    9

Return and re-enable the interrupt system.  See the RTN

mnemonic.

RTN    - Return   cycles:    9

Return control to the top address on the hardware return

stack. The top address on the hardware return stack is

popped off and placed in the program counter PC.  As the

address is popped off the stack, a zero address is inserted

at the bottom of the stack.

Therefore the the hardware return stack always contains 8

addresses, and if more pops (returns) than pushes (gosubs)

are performed, zeros will be read off the stack.  Such an

attempt to "return" to address 0 results in a memory reset,

since the memory reset code of the operating system resides

at address 0.

RTNC   - Return on carry

cycles:   10 (RTN)

3 (NO)

Return if Carry is set. See RTN mnemonic.

RTNCC  - Return, clear carry

cycles:    9

Return and set Carry. See RTN mnemonic.

RTNNC  - Return on no carry

cycles:   10 (RTN)

3 (NO)

Return if Carry is not set.  See RTN mnemonic.

RTNSC  - Return, set carry

cycles:    9

Return and set Carry.  See RTN mnemonic.

RTNSXM - Return, set External Module Missing bit (XM)

cycles:    9

Return and set the External Module Missing bit (XM).  Since

the opcode is zero, this mnemonic is executed on a jump to a

non-existent memory device.  See the "HP-71 Hardware

Specification" for more information.  See also the RTN

mnemonic.

SB=0   - Clear Sticky Bit (SB)

cycles:    3

Clear the Sticky Bit (SB).  See CLRHST mnemonic.

SETDEC - Set decimal

cycles:    3

Set CPU arithmetic mode to decimal.

SETHEX - Set hexadecimal mode

cycles:    3

Set CPU arithmetic mode to hexadecimal.

SHUTDN - System Shutdown

cycles:    5

When this mnemonic is executed the CPU sends out the

Shutdown Bus Command and stops its clock.  Issuing the

SHUTDN command with the output register=000 (see OUT=C or

OUT=CS) will cause the PC to be set to zero, causing an

automatic cold-start in some systems, or a system halt

in the HP 48.

SR=0   - Clear Service Request bit (SR)

cycles:    3

Clear the Service Request bit (SR).  See the CLRHST

mnemonic.

SREQ?  - Service Request

cycles:    7

This mnemonic sets the Service Request bit (SR) if any chip

on the system bus requests service.  When it is executed, a

Service Request Bus Command is issued on the system bus to

poll all chips for a Service Request.  If any chip requests

service, a bus line will be pulled high during the next

strobe following the Service Request Bus Command.  This

value of the bus will be latched into the least significant

nibble of the C register. The bus line pulled high

determines the device type according to the following table.

Bit           Device

ÄÄÄÄÄÁÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄÄ

3     Unused

2     Card Reader

1     HP-IL Mailbox

0     Display Driver (timer)

If any bus line is high, the Service Request bit (SR) will

be set. See the "HP-71 Hardware Specification" for more

information. See also the ?SREQ and SR=0 mnemonics.

ST=0   n  - Clear Program Status bit n

cycles:    4

Clear the Program Status bit selected by n.

ST=1   n  - Set Program Status bit n

cycles:    4

Set the Program Status bit selected by n.

ST=C   - C to Status              cycles:    6

Copy the low-order 12 bits of the Status register (X field)

into the low-order 12 bits  of the C register.

UNCNFG - Unconfigure       cycles:   12

Load the low-order 5 nibbles (A field) of the C register

into each Data pointer with the device addressed by the Data

pointer unconfiguring.  See the "HP-71 Hardware

Specification" for more information.

XM=0   - Clear External Module Missing bit (XM)

cycles:    3

Clear the External Module Missing bit (XM).  This hardware

status bit is set by the RTNSXM mnemonic.  See the CLRHST

mnemonic.