*****************************************************************************

*                      Digital Setting Circles v2.0                         *

*****************************************************************************

The following application allows anyone with a telescope equipped with

an equatorial mount and setting circles to point and view without any polar

alignment.  The need for polar alignment is eliminated by a unique system

which relies on 2 guide stars.  The 2 guide star method takes a few minutes

to setup with a little practice.

The user has access to the 113 object database which includes

49 bright stars, 39 Messier objects, 11 variable stars and 14 double stars.

The user can also enter his/her own coordinates, thus allowing for a

limitless database of objects for observing.

*****************************************************************************

*****************************************************************************

I.                   PERFORMING THE FIXING PROCEDURE

*****************************************************************************

*****************************************************************************

Before any observing can take place, you must "FIX" the telescope.  The

term "FIX" refers to the procedure in which you will be performing to obtain

reference points from which your observations will take place.  The FIXing

procedure involves selecting two guide stars.  A data base of bright stars

are provided or the user can enter custom coordinates for their favorite

stars.

Note: The FIXing procedure needs to be done only once and you will be

setup to observe objects as long as you DO NOT move the tripod on

which your telescope stands.

Press [FiX] to begin FIXing procedure:

You will be prompted to select the first guide star.

Choose a Method For

Guide Star # 1

FIND: keyword finder

BROW: browser

NOTE: notepad

|FIND|BROW|NOTE|    |BACK|

Three options will appear for the user to choose.  Depending on the

situation, one method maybe convenient compared to the other. Use

which ever method you find comfortable.  The three options are [FIND]

[BROW] and [NOTE].

PRESSING [FIND]:

If you press FIND, then you enter key character string search.

ENTER Name or Star

|    |    |    |    |    |

Note: Pressing [ON] will return back to the [FiX] main screen.

For example, say we enter the key character string "Spi".  Notice that

the search is case sensitive!  Also note that all names in the database

start with a capital letter.

The search algorithm searches sequentially through the names in the

database list until a match occurs.  If there is no match, the [FiX]

main screen will reappear and you will either have to pick [FIND] again

or one of the other menu options. If a match occurs, the resulting

screen will appear.

Coordinates Returned

From FIND

Spica [alpha]_Vir

RA: 13.25116

DEC: -11.09406

POINT Hit a key to Fix

|REDO|REDO|REDO|REDO|REDO|

If you make a mistake by picking the wrong star, you can press any of the

[REDO] menu keys or [ON] key to return back to the main [FiX] screen.

Now you are ready to perform the next step in the FIXing procedure.

Before the next procedure is explained,  the other two options

will be explained.

PRESSING [BROW]:

If we choose BROW instead, then we can browse through the catalog of guide

stars.

The following list will appear:

|Hamal [alpha]_Ari|

Altair [alpha]_Aql

Tarazed [gamma]_Aql

Cappella [alpha]_Aur

Menkalinan [Beta]_Aur

Arcturus [alpha]_Boo

Seginus [gamma]_Boo

|Pup|Pdwn|RTN|   |   |BACK|

Note: Pressing [ON] will return back to the [FiX] main screen.

Use the arrow keys to move the highlight bar through the list or use the

[Pup] page-up or [Pdwn] page-down keys to  browse through the catalog of

bright stars.  Notice that the catalogue is ordered alphabetically by

the constellation in which the star resides. Press [RTN] when you find

the appropriate star. You may be asking why [ENTER] is not used.

Good question :)

Lets use Spica as an example again.  So browse through the list until

you find Spica, then press [RTN]

Coordinates Returned

From BROW

Spica [alpha]_Vir

RA: 13.25116

DEC: -11.09406

POINT Hit a key to Fix

|REDO|REDO|REDO|REDO|REDO|

If you make a mistake by picking the wrong star, you can press any of the

[REDO] menu keys or [ON] key to return back to the main [FiX] screen.

As we can see, to pick out the guide star either method will work.

Now you are ready to perform the next step in the fixing procedure.

Before the next procedure is explained,  the last option will be

explained.

PRESSING [NOTE]

The last option that you can use to pick a guide involves using the

built-in notepad.  In order to use the notepad, you must have values

stored initially into the notepad.  There are two ways to get coordinates

into the notepad.  When you first started DSC48, you noticed there was a

[PAD] menu.  Here you can enter your own custom coordinates for your guide

star.  Also, using [BROW] from the main menu with [->PAD] menu allows

stars to be stored into the notepad.

Lets say you do have coordinates in the notepad.  So when you press

[NOTE],  your screen will show something like this:

:[alpha]Seginus: 14.320468 :...

:[alpha]M1:5.343 :[delta]M1:22.00...

:[alpha]hello:1.01212 :[delta]hel...

:[alpha]Spica:13.25116 :[delta]Sp

|Pup|Pdwn|RTN|    |    |BACK|

Use the arrow keys to move the highlight bar through the list or use the

[Pup] page-up or [Pdwn] page-down menu keys to browse through the notepad

of objects.  Lets pick Spica again like in the previous methods.  Use

[RTN] to pick the guide star.

The Following screen will show up:

Coordinates Returned

From BROW

Spica [alpha]_Vir

RA: 13.25116

DEC: -11.09406

POINT Hit a key to Fix

|REDO|REDO|REDO|REDO|REDO|

If you make a mistake by picking the wrong star, you can press any of the

[REDO] menu keys or [ON] key to return back to the main [FiX] screen.

So that ends the discussion on the methods you can you use to pick your

guide stars.  Like I mentioned before, use whatever method that seems

appropriate for the situation.  So, now we proceed to the next step.

*****************************************************************************

Next step:

By now, you must be wondering what is meant by "POINT Hit a key to Fix",

which appears at the bottom of the screen when you choose a guide star.

Well, its time to get the telescope out and plant it firmly, so it

doesn't move and it doesn't matter if the telescope is level.  Lets

pretend Spica is visible and we chose it as our 1st guide star.  Now POINT

your telescope at Spica and center the star within the field of a low

power eyepiece.  Yes, center the star within the field of the eyepiece and

not the viewfinder.  Of course, you would use the viewfinder to roughly

locate the star, then look through the eyepiece to center it.  Once you

are sure that the star is centered, press any key to fix the star.  Thus

"POINT Hit a key to Fix" implied two steps: point at star, then press a

key.

The next screen appears after you hit a key:

Enter DIR and ELEV

From Telescope

Hour.SubHour     [degree]

|   |   |   |   |   |   |

If you didn't feel like you centered the star properly, press [ON] to get

back to the previous screen, then re-center and hit a key to fix.  Or

you can press [ON] again and this will return you back to [FiX] main

screen.

If you feel confident that the star was centered properly, then you can

enter the setting circles coordinates.  Now this maybe the most confusing

step in the fixing procedure.  The two pieces of information that you

need to enter are readings from the right ascension and declination

setting circles.  At this stage, I assume the reader is familiar with

the operation of equatorial type setting circles.

Now read off the RA circle for the Hour.SubHour and DON'T move the

telescope's tube assembly and definitely DO NOT move the tripod.  Lets

say the reading is exactly between 15 and 16. The Hour would be 15. Now,

you ask what is a SubHour?  There are 12 divisions between any hour (on

my telescope).   Therefore, a SubHour is one of these divisions.  Since,

the reading is exactly between 15 and 16, then there are 6 SubHours

(12/2). So, enter 15.06 for DIR.  If there were 11 SubHours then you would

enter it as 15.11.  The ELEV is in the usual number of degrees of

declination.  Lets say, the reading was 23.5 degrees on the declination

circle.

This is how you should enter everything:

Enter DIR and ELEV

From Telescope

Hour.SubHour     [degree]

15.06 23.5

|   |   |   |   |   |   |

Congratulations, you just Fixed the first Guide star.  In the next step

you will be fixing guide star #2.

This will be the next screen after you fix star #1:

Choose a Method For

Guide Star # 2

FIND: keyword finder

BROW: browser

NOTE: notepad

|FIND|BROW|NOTE|   |   |BACK|

If you felt like you made a mistake, you can press [BACK] or [ON] and

this will return you back to the main DSC48 screen.

So repeat the exact same procedure as you preformed for guide star #1 but

with a use a totally different star that the one you used for guide

star #1.  Once you have completed FIXing guide star #2, the telescope is

now ready to locate objects as long as you DO NOT move the tripod on

which your telescope stands.  You maybe thinking that all these steps

involved are very complicated but in actual fact it takes very little

practice to get familiar with the procedures.

NOTE:  if you observe outside the two guide star boundary (i am having

trouble defining that myself) or if you feel that your observing

large distances away from where you initially setup your guide

stars,  the accuracy of pin-pointing objects will be less.  So

you may have to re-FIX the telescope for the new region where

you are observing.

A REALLY BRIEF SUMMARY:

1. Visually locate 2 guide stars.

2. Point and fix them.

3. Ready to Observe.

*****************************************************************************

*****************************************************************************

*****************************************************************************

*****************************************************************************

EXPLANATION OF REMAINING MENUS KEYS

*****************************************************************************

If you press [1]           1. RA,DEC -> TELESCOPE

Here you can enter the coordinates of the object you want to observe.

The objects coordinates must be right ascension and declination.

Remember to enter the coordinates together on the same entry line and

then press [ENTER].

So, you would enter something like this:

COORDINATES OF OBJECT

[alpha] (h.mmss)  [delta] (d.mmss)

10.4412 5.6788

|PAD->|   |    |    |    |    |

You may also notice the [PAD->] menu key.  You can access the notepad

through here.  So, press [PAD->] and you will see PAD-> on the entry

line.  Press [ENTER] and you will enter the notepad.  In the notepad

select the star you want to use and press [RTN] and the coordinates will

be returned to the entry line, where finally you will press [ENTER].

After entering a pair of coordinates the following screen will appear:

SET THE RA CIRCLE  TO:

20.06

SET THE DEC CIRCLE TO:

46.232

?

LAG: 60s

|    |    |    |    |   |BACK|

Set the RA circle to the 20th hour and 6th SubHour.  Then set DEC circle

to 46.2 degrees.  You will also notice that there is a timer running

near the bottom the screen.  This indicates the amount of time you

have in order to set up the circles to their proper coordinates.  It

doesn't matter if you are 10s late or early in setting the circles.  It

will just mean the object will be somewhat off center in the field of

view of a low power eyepiece.  The lag time can be adjusted. See [LAG]

for more details.

If you press [2]           2. TELESCOPE -> RA,DEC

Lets assume you find an object that you cannot recognize(a comet or

super nova?). This function will allow you to convert the telescopes

coordinate system to RA,DEC.  The accuracy of this function is

questionable but will give a rough estimate.  First center the object

in the field of the eyepiece, then select 2 by pressing key [2].

The following screen will appear:

Enter DIR and ELEV

From Telescope

Tick.SubTick        [degree]

|   |    |    |    |    |    |

Next read the Tick.SubTick and degrees declination from the setting

circles.  See fixing procedure for explanation of Tick.SubTick.

If you press [3]           3. TICK.SUBTICK -> RA

The telescope does not have to be fixed in order to use this function.

This converts the tick.subtick format to right ascension.

If you press [4]           4. RA -> TICK.SUBTICK

The telescope does not have to be fixed in order to use this function.

This converts right ascension to tick.subtick format.

If you press [5]

Select the current object catalogue. (bright star, messier, variable, or

double)

If you press [LAG]

Set the LAG time for positioning the setting circles during observing.

If you press [BROW]

Browse the current object catalogue.

If you press [PAD]

Store, edit, browse and much more using the internal notepad.

If you press [picture of key]

You can lock the [FIX] and [LAG] key so you don't accidentally change

the values.

If you press [QUIT]

Quit DSC48.

The instructions for theses keys are rather brief.  So, play around with

the keys to get familiar with their functions.  You don't want to be

fumbling around in the night trying to read the instructions :)

END

***************************************************************************

NOTES:

- not responsible for any damage to your calculator

- use at your own risk

- this program is freeware

- partially written sysRPL

Author: Arnold Moy (amoy@geog.ubc.ca)

***************************************************************************