NOTE: THIS IS NOT A SONALYSTS PRODUCT.  IT WAS DEVELOPED

INDEPENDENTLY BY THE AUTHOR.  IT IS MADE AVAILABLE ON INTERNET

ONLY AS A COURTESY.  THE AUTHOR MAY OFFER TECHNICAL

SUPPORT BECAUSE HE'S A NICE GUY IF YOU CATCH HIM ON ONE OF HIS

"GOOD DAYS".  OTHER THAN THAT, YOU'RE ON YOUR OWN!

User Notes for the N4QN Pocket Satellite Tracker, HP-48S Version

Albert P. Gerheim, Ph.D., N4QN

1 Woodline Dr.

Westerly, RI 02891-4931

email: gerheim@sonalysts.com

To install this program, just copy file SAT into your HP-48S

or HP-48SX.  The constants I supplied are set up for my own

station and operating habits, but they can be changed to suit

your own needs.  Refer to the HP-48 documentation for stuff like

file transfers, and for loading arrays and lists.

The program described here is similar to the N4QN Pocket

Satellite Tracker for the HP-42S, and all the mathematics for it

are described in detail in the user notes for that version.  It

is an abbreviated version of the Project Spacetrack SGP model,

described in the Aerospace Defense Command (USAF) Project Space

Track Report Number 3.  Both programs have two modes of

operation.  Here, they are realized in two different programs:

RT1S is a Real Time tracker for 1 Satellite, and FTMS is a Future

Time tracker for Multiple Satellites.  Both modes were realized

with the same code on the HP-42S.  The two HP-48 programs call

subroutines which do the actual tracking, displaying, and

printing, as indicated below.

Main Program:       RT1S

Subroutines Used:   TRAK (computes satellite position and puts it

in the array Q)

STN  (computes station location, computes

the vector to the satellite in local

coordinates, places the observation

in the list X: {Range, Azimuth,

Elevation}

SEE  (generates graphic display)

RTIM (computes time codes and strings from

the clock)

Constants Used:     UPP  (a graphics object containing the

background of the display)

BIRD, QTH, SATS, QTHS

Main Program:       FTMS

Subroutines Used:   TRAK, STN, HC (generates hard copy output)

SC   (generates a soft copy output in the

variable 'PASS')

AL   (sets alarms for RS10 and RS12)

Constants Used:     SKY, QTH

The constant UPP is supplied with the program.  It contains

the background of the "planetarium view" graphic.  The menus SATS

and QTHS, and the list SKY are set by the user.  They all contain

names of arrays.  For example, if we wish to track RS10, and

RS12, then SATS will contain the file names RS10 and RS12, along

with control statements.  (You really have to see the files QTHS

and SATS to see how they function as menus.)  The file RS10 will,

in turn, contain the Keplerian elements for RS10/11, as described

later.  Similarly, the file QTHS will contain names of files of

QTH's, along with control statements.  In turn, the arrays named

in QTHS will contain each station's latitude in degrees,

longitude in degrees, and altitude in kilometers.  The constant

SKY is a list of lists used by FTMS.  Specifically, it is a list

of all the BIRD variables to be scanned for availability.  For

example, if you only work the Russian satellites, SKY might be

{{RS10} {RS12} {MIR}}.

The Keplerian elements for the satellites are in the same

format as the HP-42S version.  There is a program for converting

NASA 2-line data to HP format.  It is called AUTO, and its

EXEcutable and C source files are included in the floppy.  The

file format is that of a matrix:

Element Number      Contents

(1)       Year since 1900  (example: 92)

(2)       Epoch  (day of the year, with fractional

part)

(3)       Inclination [deg]

(4)       Right Ascension of the Ascending Node [deg]

(5)       Eccentricity [deg]

(6)       Argument of Perigee [deg]

(7)       Mean Anomaly at Epoch [deg]

(8)       Mean Motion [rev./day]

(9)       First Derivative of Mean Motion [rev./day2]

The elements are in the same units and in the same order as

they appear in the AMSAT Journal.  Some elements printed in the

Journal are not used.  The files can be edited using the EDIT key

in the custom menu.  The program AUTO searches for satellite ID's

listed in the file SEARCH.LIS, and then sees if that satellite is

listed in the kep files which the user specifies.

Be sure to set the calculator's clock to UTC to operate the

real time tracker.  I also use DD.MMYYYY format for dates, and 24

hour format for times.

Once you load QTHS, SATS, and the arrays they name, all you

have to do to get a real time display of the bird is press the

custom menu key opposite "RT1S".  The program will allow you to

select the bird and station via menu.  After a short wait, the

left side of the display gives the time string, the slant range

in kilometers to the satellite, its azimuth and elevation, along

with the satellite's identifier (contents of BIRD) and the

station ID (your call sign, the contents of QTH).  The right side

of the display is taken up with a planetarium display.  This

display represents the sky as a circle with the horizon at the

perimeter, and "up" at the center.  When the satellite comes over

the horizon, a 4-dot icon appears at the appropriate position.

Notice that East is at the left side.  This because the view is

up, and it is to opposite-hand from the more common map

orientation.

To operate the FTMS program, you must enter the constants

QTH, the array it names, SKY, and the arrays it names.  These

aren't changed very often, so I'll let you load them by hand.

After you hit the "FTMS" key in the custom menu, a temporary menu

appears.  It allows you to toggle hard copy ('HC') and soft copy

('SC') outputs.  The output(s) with the box next to them are

active.  Hit the menu key marked "GO!" when the outputs are

right.  The default is soft copy only.

The soft copy mode writes a listing in the string variable

"PASS".  This variable can be dumped to the IR printer with the

"DUMP" command in the custom menu, or cleared with the "CLR"

command in the custom menu.  The hard copy mode writes the output

directly to the IR printer while executing.

After you hit the "GO!" key, 5 arguments are displayed for

your review.  These argments are the starting time, the starting

date, the ending time, the ending date, and the time increment.

The inputs are all in HH.MMSS and DD.MMYYYY format.  Notice that

the time increment is off the screen, and you have to cursor down

there to see it.  When the arguments are right, position the

calculator to use the IR printer, and hit "ENTER".  This copies

the arguments into the file "TIMES", for future use.  During

operation, the display shows the future time string.  When the

hard copy output is enabled, and a satellite comes over the

horizon, the IR printer shows the time string, the azimuth,

elevation and range of the satellite, the satellite ID, and the

station ID.  By the way, if you want a more exciting display in

this mode, it would be simple to delete the display commands (in

FTMS), include the SEE subroutine in FTMS after the call to STN,

and maybe even include a Print LCD command.

One of the motiviations for this project was that the

calculators have complete electromagnetic compatibility with a

typical ham station.  I use a fairly sensitive receiver, and a

"lot of RF" on mode K, and the calculator and station equipment

are oblivious of the other's operation.  Another motivation for

the program is that I work satellite packet, and that pretty much

takes over the PC monitor.  The HP-48 allows me to keep track of

the satellite without using a complex system of mirrors and

telescopes to see the antenna.

As I mentioned earlier, I included a custom menu.  It

includes commands for the two versions of the program, the "STD"

command (to reset the display mode), plus "CLR" and "DUMP"

commands for clearing and dumping the soft copy output to the IR

printer.  The "CLR" key puts a zero length string into the

variable 'PASS', and the "DUMP" command prints the variable

'PASS' on the IR printer.  The GC command puts you into the great

circle subdirectory.  It does great circle calculations, ALL IN

THE MENU.  Two more custom keys are available by pressing the

"NXT" key.  These keys are for EDITing keplerian element files

and QTH files, and for going into SERVER mode so you can receive

new keplerian element files over the serial interface.

See you on the air!!!

: >   AL

Here are the files on the disc:

SAT            -    This is the directory you load into the

HP-48.  You must edit files QTHS and SATS to

indicate QTH's and birds of interest to you.

These two files are used as temporary menus

during execution.  SAT is a complete

directory, and must be load while the home

directory is the source directory.

PPS      ASC

PPS      WP    -    These are the ascii and Word Perfect files

describing the mathematics of the modified

SGP tracker.

AUTO     C

AUTO     EXE  -     These are the source and executable for a

program which generates kep files which are

used by the HP-48.  Files MIR, RS10, and RS12

are examples.  This program prompts for a

search list (see SEARCH.LIS), and an input

file of NASA 2-line keps.

SEARCH   LIS  -     This file contains 5-digit satellite ID

numbers and file names of satellites of

interest to the user.  It is used by AUTO

MIR

RS10

RS12          -     These are examples of the output of AUTO.

SAFE     C

SAFE     EXE  -     These are the source and executable for a

program which removes checksum errors from

NASA 2-line kep files.

README   DOC  -     This file.