**Beam-v1.1 users manual**

**CONTENTS:**

1 **INTRODUCTION**

2 **'SECTIONS' parent directory**

3 **'Beam-v1.1' sub-directory**

3.1__INPUT__

3.1.1 __inputting new data__

3.1.2 __editing data__

3.2 __CALCULATION__

3.3 OUTPUT

4 **APPENDICES**

5 **DISCLAIMER**

1 **INTRODUCTION**

This application calculates the deflections and stresses in a homogeneous linear

beam using the finite element method. In fact, it has two functions. Firstly, it

has a parent directory called 'SECTION', with several useful programs for

calculating properties of area for common sections as well as for composite

sections using the parallel axis theorem. Secondly, there is a sub-directory

called 'Beam-v1.1', which calculates the deflection and stress in homogeneous

linear beams transversly loaded with point forces and moments as well as

uniformly distributed forces. This uses the finite element method which enables

statically indeterminate problems to be solved. The application is very user

friendly and no previous knowledge of the finite element method is necessary.

When you first use it, be careful to read everything that comes up on the

calculator screen, so you get used to the application and soon you should get

used to how it works. Where it might help in this manual, I have given the basic

principles of what is going on with the analysis.

Within the 'Beam-v1.1' sub-directory, you can enter data on the beam and edit

that data. The calculated results are displayed both as a graph showing the

effect of the loading on the beam, and the maximum absolute value with its

position on the beam. The finite element method divides the problem, the beam in

this case, into a series of elements, creates a simultaneous equation for each

element related to the beam and solves these equations which gives the results

for the whole beam. Because the elements are of a finite size, the finite

element method will always give you an approximation, but the accuracy can be

very acceptable. Improving the accuracy is quite easy by increasing the number

of elements. Usually I find the error is less than 4%, but it can really depend

on the particular problem you have and the number of elements. If necessary, the

accuracy can be improved in two ways; either by increasing the number of

elements, particularly in the vicinity you are interested in, and doing the

calculation again, or by turning the calculator into graphics mode and moving

the cursor manually to make a better guess at the extreme position.

It has been written for HP 48G series calculators in user-RPL, so you won't

crash your pride and joy. It could be run on HP 48S series calculators, but some

sub-routines would need to be added to perform some of the list processing and

matrix row and column manipulation commands. These sub-routines are in existence

and should be no problem to incorporate. There is a list of the commands that

Beam-v1.1 uses that are unique to the HP 48G series in the appendices at the end

of this manual. If you are using an HP48S, be careful of how much memory is

available for the application's storage and calculations.

The application itself takes up around 10 KB of memory and requires a further 5

KB minimum to run. It depends on the complexity of the problem, but a total of

20 KB for both the programs and working space should be sufficient. The time

taken to calculate results depends on the number of elements you choose. Usually

I find I'm waiting for 3 to 15 minutes.

2 **'SECTIONS' parent directory**

'SECTION' is the parent directory and has two functions. Firstly, it contains

several useful programs for calculating properties of area for common sections

and for composite sections using the parallel axis theorem. Secondly, it also

contains the starting program for launching into the sub-directory for the

finite element analysis.

Throughout this section, I use conventional symbols for properties of area.

The main menu gives you the following programs. The titles of these programs can

help you should you forget the order the variables should be imputed.

'IRectangle->b?d?'

Calculates the second moment of area of a rectangular section, bd. You need to

place b and d on levels 2 and 1 respectively. For a square section, press ENTER

twice. I have not gone further to calculate the section modulus (divide by c),

because very often I'm dealing with hollow sections, so have to subtract the I-

value for the void before calculating the section modulus. The answer is tagged

'IRect:'.

2: b input

1: d input

1: IRect: Ianswer

'IcAngle->a?t?'

Calculates the second moment of area and distance from neutral axis to extreme

fiber, c, for an equal-sided angle iron orientated in either a 'A' or 'V'

fashion. The formulas used are taken from "Roark's formulas for Stress &

Strain", pub by McGraw-Hill. You input the length of the side, a, on level 2 and

the thickness, t, on level 1. The results are placed on level 2 and 1 and are

tagged 'IAngle:' and 'c^Angle:' respectively. The c-value is the maximum, so

simply dividing the two values then gives the minimum section modulus.

2: a input

1: t input

2: IAngle: I answer

1: c^Angle: c max answer

'ICircle->d?'

Calculates the second moment of area of a circular section, diameter d. I have

not gone further to calculate the section modulus (divide by r), because often

I'm dealing with tubes or pipes, so have to subtract the I-value for the void

before calculating the section modulus. The answer is tagged 'ICircle:'.

1: d input

1: ICircle: I answer

'ACircle->d?'

Calculates the area of a circular section, diameter d. The answer is tagged

'ACircle:'.

1: d input

1: ACircle: A answer

'cAI->CompositeData'

Is for inputting data when calculating the I and c values for a composite

section beam. It is used in conjunction with 'IcComposite' which calculates the

I and c. You must first decide on a datum line from which to measure the values

of c for each of the sections comprising the composite section. For each of the

sections, you input c onto level 3, A onto level 2 and I onto level 1, then

press 'cAI->CompositeData'. Note the title of this program alerts you to the

input requirements, with the to remind you that voids have negative values for A

and I. Then repeat for the remaining sections comprising the composite section,

until all data for the composite section has been imputed.

3: c input

2: A input

1: I input

'IcComposite'

calculates the I and c for the data imputed into 'caicompositedata' using the

parallel-axis theorem. The results are placed on level 2 and 1 and are tagged

'IComp:' and 'cComp:' respectively. A note at the top of the screen reminds you

that the value of c calculated may not be the maximum for calculating the

minimum section modulus.

2: IComp: I answer

1: cComp: c answer

'Beam'

Starts the finite element analysis and enters the 'Beam-v1.1' sub-directory. It

also saves the current flag settings, so exit 'Beam-v1.1' using 'Quit' from the

menu keys to retain your original settings. A complete description is given

below.

3 **'Beam-v1.1' sub-directory**

'Beam-v1.1' is the sub-directory which contains the programs for calculating the

deflection and stress in linear beams transversly loaded with point forces,

uniformly distributed forces and moments. Always enter the 'Beam-v1.1' sub-

directory using 'Beam' from the 'SECTIONS' parent directory. This ensures that

your initial flag settings are maintained, providing you exit with 'Quit'. To

enter 'Beam-v1.1' from the 'SECTIONS' parent directory main menu, press NXT,

'Beam'

First, you are asked what you want to do about data, and offered to input 'New'

data, 'Edit' existing data, or 'Quit' on the menu keys.

3.1 __INPUT__

There's only one principle you have to understand when dealing with the finite

element method in this application. The finite element method works by dividing

the beam up into a series of elements connected by 'node' points. As the name

suggests, these elements are of a finite length. The method then develops a

series of simultaneous equations which it solves to produce the deflection and

stress results. You divide your beam up into a series of 'node' points, that

connect the beam elements between them.

Your beam problem will probably also have to be split into a series of

discontinuities. A discontinuity will be at a node. A discontinuity I have

defined as a change in the condition of the beam, which would result from:

A force or moment support or load applied. These loads are applied at a node;

An uniformly distributed load beginning, ending, or changing value. This

loading condition is automatically divided into point forces acting on the nodes

between consecutive discontinuities;

A change in section properties. This analysis only requires the second moment

of area and the distance from the neutral axis to the extreme fiber, but not the

section area. These properties apply to each element between consecutive

discontinuities;

A change in the concentration of elements. You would use this if you want

greater result accuracy in a particular region.

You need to decide how many elements you want between discontinuities. (I'm

usually using between 5 and 8.)

When starting the program, you are given 3 options:

Input new data

Edit existing data

Quit

3.1.1 __inputting new data__

'New'

This program is to input new data. If you cause an error during the input

sequence, the program will automatically begin the inputting program 'New' again

from the beginning.

For inputting data, you start at one end of the beam, taken as node 1, then

progressively enter data until you reach the node at the other end. To stop,

enter 0 when asked for the length to the next discontinuity.

It is useful to understand how the application categorizes the data. After

pressing 'New', the program asks you for the following data:

Youngs Modulus for the beam.

First the program asks for the value for Youngs Modulus for the entire beam.

(Recall the beam must be homogeneous.)

Support Data (force, moment);

Then the program will ask you about the supports at the node.

Point Load Data (force, moment);

Then the program will ask you about the point loads applied to the nodes (first

force and then moment). If there is already a support of the same type at the

node, the program doesn't bother to ask you if there is the same type of point

load, and automatically sets the value to 0.

Discontinuity Data (length to the next discontinuity, number of elements,

uniformly distributed force per unit length);

You are asked what the length is to the next discontinuity, then for the number

of elements you want to split the length up into.The number of elements must be

greater than one, or the program generates an error. The more elements there

are, the more accurate the solution will be. It is important to understand that

finite element always gives an approximation, but the error can be negligible if

an adequate number of elements are chosen. For a rough idea of what's happening

to the beam, I would suggest a minimum of 4 to 6 elements between

discontinuities. A better accuracy will be achieved using 8 or more elements.

Then you are asked if there is any uniformly distributed load acting over the

portion to the next discontinuity. You have the option to give the uniformly

distributed force as a force per unit length (which is the default), total force

acting to the next discontinuity, or the previous value of uniformly distributed

force. The data is stored as uniformly distributed force per unit length.

Section Property Data (second moment of area, distance from neutral axis to

extreme fiber);

Here you are asked for the second moment of area, I, and the distance from the

neutral axis to the extreme fiber, c to the next discontinuity. You are given

the choice of entering the previous values, or using the relevant 'SECTIONS'

menu items for properties of section. If only one number is on the stack when

you press 'CONT', the program automatically assumes that is I and asks you again

for c.

Then the program asks you for the data for the node at the end of this

discontinuity: both for force and moments for the supports and then loads.

The program now re-iterates the input loop, and asks you for the length to the

next discontinuity. If you enter 0, you exit the input routine and go to the

calculate routine, described below in section 3.2 CALCULATION.

Program interaction (prompts in quotes, followed by menu options):

"E = xxx ...OK?"

'Yes'

'No'

"Node n FORCE support ?"

'Yes'

'No'

"Node n MOMENT support ?"

'Yes'

'No'

"LEN to next discontinuity = (press '0' to stop)"

'cont' - continue with value on stack

'No' - puts 0 on stack.

"NUMBER of elements = (must be greater than 1)"

'cont '- continue with value on stack

'No' - puts 0 on stack

"Uniformly distributed FORCE ="

'cont '- continue with value on stack

'No' - puts 0 on stack

'F/L' - value of force per unit length

'F tot' - value of total force acting on length

'Pre F' - previous value (of force per unit length)

"I = , max c = "

'cont'- continue with values on stack (if only 1 value on stack, assumes it's I

and asks for c)

'pre Ic' - previous values

'SECT' - goes to 'SECTION' directory to calculate I & c values

Then continues with:

"Node n FORCE support ?"...

3.1.2 __editing data__

'Edit'

This program is for editing or viewing the previous data. You can select which

data you want to edit with a CHOOSE menu, using the up- or down-arrow keys to

select what you want, then press 'OK' to edit, or 'CANCEL' to exit. To edit

data, you next have to press the VIEW keyboard key which which puts you into the

calculators matrix editor. Having finished with the data, the data matrix is

returned to the stack and you have to press 'CONT' to carry on. After editing,

the user is returned to the menu for calculating the results, described under

section 5: CALCULATION, below.

Data is collected and stored under five categories, which are listed in

abbreviated form in 'Help':

Support Data '[force, moment];

Point Load Data [force, moment];

Discontinuity Data [length between discontinuities, number of elements,

uniformly distributed force per unit length];

Section Property Data [second moment of area, distance from neutral axis to

extreme fiber];

Youngs Modulus for the beam.

'old'

If you want to view the previous results for deflection and stress.

3.2 __CALCULATION__

The process the finite element method uses is to construct a stiffness matrix of

the entire beam, using the data on the supports, properties of section, element

length and Youngs modulus. Then a decomposition is done on this matrix. This is

done in the program 'ProsS'. Then the program 'AnsR' takes the decomposed

stiffness matrix along with the loading on the beam and determines the nodal

displacements. The result is an array of angular and linear deflections for each

node. The stress is calculated from the angular displacements. The values for

the beam deflections are calculated for each node along the beam. The values for

stress are calculated for each element mid-point along the beam.

The calculation is started either from 'New', or 'Edit'. From 'New', you have to

use the 'ProsS' program. If you have just been through 'Edit', the program will

give you either 'ProsS', or 'AnsR', depending on what you have changed in the

data. If the changes are only in the point forces, moments or uniformly

distributed force, then there is no need to go through 'ProsS' and re-calculate

the decomposed stiffness matrix again, and the menu will display 'AnsR' instead.

3.3 __OUTPUT__

The output gives data on both beam deflection and stress along the beam.

Deflection is given by pressing the 'y' menu key and stress given by pressing

the '"small-sigma-symbol"' menu key. Data is displayed graphically, with points

showing the values along the beam and the maximum absolute value circled. Zero-

value is marked by a straight horizontal line. At the top of the screen, the

maximum absolute value is given along with its position along the beam.

Remember that the finite element method will always give you an approximation.

If you have chosen sufficient elements to divide the beam up into, this will

give a good enough approximation for you to make a judgment on. However, if you

want more accuracy, there are two things you can do. Firstly, you can 'Edit' the

data and increase the number of elements between discontinuities in strategic

places along the beam, for example around the value you are interested in

(probably the max/min) and also in any portion that is showing a rapid change in

value. You may want to add a discontinuity just to increase the concentration of

elements in the vicinity of the region you are interested in. Secondly, you can

get an intermediate value between two calculated points by pressing 'Plot',

which places the last plot on the screen and puts the calculator in graphics

mode. You can move the cursor to the position that looks to be in the position

you are particularly interested in. The coordinates can be displayed by using

the '(X,Y)' command. The current (x,y) coordinate values can be returned to the

stack by pressing 'ENTER'.

'y'

For displaying the displacement. Position of maximum value is displayed at top

of the display, with a circle around the corresponding point on the graph.

'"small-sigma-symbol"'

For displaying the stress. Position of maximun value is displayed at top of the

display, with a circle around the corresponding point on the graph.

'Plot'

For entering the graphic environment. x and y values of the cursor position are

displayed ot the bottom of the display.

'New'

For starting again with new data.

'Edit'

For editing existing data.

'Quit'

For returning to the 'SECTIONS' parent directory and restoring the original flag

settings.

4 **APPENDICES**

Splines

There is a program that you may find useful if you want more accurate results

between plotted points. The program is called 'Spline' by Ruben Rosamian and is

available at:

cbs.cis.com:/pub/hp48g/graphics

"spline.zip - Interpolation v2.0 by Ruben Rostamian. SPLINE generates a

piecewise cubic and twice continuously differentiable interpolation y(x) of a

set of points (x_i,y_i), i=1,2,...,n. It is assumed throughout that x_1 < x_2 <

... < x_n)"

HP 48S series commands

The following commands are used in the application that are not available on the

HP48S series:

CHOOSE COL+ COL- COL-> HEAD sigmaLIST deltaLIST REVLIST ROW-

Beam-v1.1 technical manual, with full program listing, commentary and formulas

involved. Get in touch with me if you're interested.

5 **DISCLAIMER**

This is freeware. This software is distributed free, as is and used at users

risk--usual disclaimer. This software can be freely distributed provided this

READ ME is distributed with it. This software may not me sold.

Mark Butcher

<markfb@iname.com>