Selev 1.0

I.    FILES IN THE SELEV.ZIP FILE

SELEV       (binary directory for hp) contains:

CST         Custom variable for Custom Menu

BEG         Beggining station of Curve (1st left)

FLAT        Flat station (2nd left)

REV         Reverse station (3rd left)

PC          PC station (4th left)

FULL        Full station (5th left, last)

FULL2       Full station (right)

PT          PT station (right)

REV2        Reverse station (right)

FLAT2       Flat station (right)

END2        End station of Curve (right)

SHLD        Shoulder station (left)

SHLD2       Shoulder station (right)

KKK         1X7 matrix of last input values\

K           K constant

TR          Tangent Runout

SELEV       Main program (Run from custom menu)

SELEV.ASC   Ascii file (readable) of binary SELEV

SELEV.DOC   This file

SUEL.PCX    PC Paintbrush format file view of ODOT SUEL standard

ROOFTOP.PCX PC Paintbrush format file view of Std. road section

II.    EXAMPLE

1.  in SELEV Directory

CST    (press menu key, not CST variable)

SELEV  (Only key in CST menu)

A.  :PC Sta: 80803.73  (press down arrow key)

:PT Sta: 82811.54  (down twice, right twice)

:Snormal in/ft

1/4in=.25: .25     (.24 means 1/4" per foot slope, lane.

down, right)

:Max Super: .03    (Max. Super from chart for speed and degree

of Curve.  Press down arrow.)

:Length: 150       (Length - Comes from chart with K and L2 as

well as Minimum Length in feet. Down, right)

:Lt Curve=0: 1     (0 for left curve and any other for right.

down twice.)

:Sshld top in/ft   (Super of shoulder in inches/ft, 0.5=1/2" per

1/2in=.5:           foot.  Press Enter Key.)

B.  Press VAR key to see display of variables computed.  Think of

the stations in the order as listed in part I.  Kind of an N is

the way the variables are ordered.

BEG    80587.0633333  Station 805+87.06 (Left leg going up on

on the N shape.)

FLAT   80691.23       (Flat does not necessarily mean the slope

is flat.  Depends if you are considering

left, right, outside, or inside lane.)

REV    80795.3966667

PC     80803.73

FULL   80841.23       (Press NXT key)

FULL2  80841.23       (Going down the N shape now.)

PT     828811.54

REV2   82819.8733333

FLAT2  82924.04

END2   83028.2066667

SHLD   80832.8966667  (See Note below SHLD2, press NXT key.)

SHLD2  82782.3733333

Note:  The two shoulder stations above represent that station

that the shoulder reaches the maximum breakover (0.07).

I need to do some more work on those two.  It is

dependent on the shoulders top and bottom slopes.

III.   Explaining some points

A.  L=L2*C where, L=runoff length in feet

L2=from chart (ODOT SUEL standard) is intersection

of design speed in MPH and degree of curve.

C is found in same chart to the right.  It pertains

to the number of lanes.

L Minimum from chart is > than L2*C then use L minimum.

B.  A chart similar to the ODOT SUEL standard should be in the MUTCD

(Manual for Uniform Traffic Control Devices).

C.  K=S/L   where, K is the 'K' Rate of transition the driving lanes

rotate ft/ft in the area of super.

S is the super rate ft/ft for the curve.

D.  TR=Snormal/K   where, Snormal is the normal slope of the outside

driving lanes.

TR is the tangent runout length in ft.

(Take absoulute value for TR).

NOTES:  I hope you enjoy this program.

Several programs that I have written are listed in civil1-2.zip

in the readme.1st file.

BIO: - Kenneth Hobson wrote this document for you.

Ken graduated in Spr.'93 and is and Intern I Civil Engineer for the

Oklahoma Department of Transportation.  405-521-2677

Modify anything you like.  Please send me a copy if you do it and I

will document it and upgrade this version of SELEV.

khobson@telepath.com

Regards,  Kenneth Hobson          kenneth.hobson@torii.nova.com

kenneth.hobson@s-box.misc.uoknor.edu

work  internet address: kenneth.hobson@dsibm.okladot.state.ok.us