VCM is the binary directory to install on your HP48SX.  VCM.ASC is the ascii

encoded file that can be converted to binary or print out on the pc.  VCM.DOC

is what you are reading now.  VCM.INF is just an information file for Chris

Spell at ftp site of wuarchive.wustl.edu (hp files in systems dir.).  Look

there for some of my other files and many other contributors.

In the VCM directory I like this order:

{ SLMC SLVE MC A STCRV STMCRV HPLP SUBLSMC MCLIST T DATA STFM LSTC ELEV }

LEFT SHIFT VAR ORDER    This is a convenient order. Use STMCRV to store your

curve data, far right of display, and SLMC on the far left.

These programs are for muliply/single vertical curve(s) finish grade elevation

determinations used in SURVEY and other courses.

Note:  Several programs that I use and have posted are similar to commercial

cogo type routines.  These are free for your collection.  Try AREA (on

goodies disk 9, KAREA, COINTERCEPT (in dos cointerc), HCURVE, FRACDC, VC, VCRV,

TRIANGLS (/\S or #_S on the pc) or triangle (/\s it not dispalyable on

the pc), OCF (open channel flow, hydraulics), and VCM.

VCM is the latest version of a solver for multiply vertical curves.

For a short single vertical curve I recommend my VC program.  Second,

I would recommend VCRV (Dewayne Cushman wrote this one as an enhancement

for VC (it gives extra data).  Lastly, and the longest is the VCM.  It

works just like the VC and VCRV but it can do any number of curves, even

single (my modification of his modification).

Find most of the above at ftp site of wuarchive.wustl.edu in systems/hp/hp48

and down.  Please email if you fine them there.  It has been a while since I

posted them so they are buried in the backup directories......

Note:  Set mode to STD for these examples.

*******The instructions below are valid for the VCRV program and VCM******

*******Try the instructions at the end (one line of *'s) for VCM    ******

*******alternate instructions.  The beauty of this is that you can  ******

*******do it both ways.  I personally like the last method the best. *****

When extracted the dir VCRV contains 7 programs:  (included in VCM)

1.  STCRV - Requires 5 inputs on the stack.  e.g.

Station -  130+00   use 13000     (At point of intersection (PI))

Elevation - 926.21 feet   926.21    (At PI)

Grade 1 - -1%              -1       (Grade left of PI)

Grade 2 - 2.03%             2.03    (Grade right of PI)

VC Len. - 200 feet        200       (Vertical Curve Lenth)

or 13000,826.21.-1,2.03,200 STCRV (Store curve data of course) gives

P.I. Sta.:  130+00.00

P.I. Elv :  926.21

Grade 1:  -1%

Grade 2:  2.03%

Length:  200

R. of C.:  1.515 ft/ft    (Rate of curvature)

Anykey except ON

Low Point:  129+66.007

Elev:  926.88

Anykey except ON

2.  SLVE - Input the station say 130+50 as           13050 SLVE

and you get:

POINT ON CURVE

Sta:  130+50

Elev:  927.414

Any key (except ON).

for 128+99 (12899 SLVE) you get:  POINT ON TANGENT 1

Sta.  128+99

Elev:  927.22

Anykey

Note:  POINT ON TANGENT 1 is not in the vertical curve.  It is on the

tangent line as if it continued which it may or may not.

I did this because of error in FG (finish grade elevation) for real work.

You can give the g1 and g2 as the same and you get fg on a continuous

grade and give it a large VC length and this is on the slope.  It is really

handy for some things (i.e. analytical geometry, surveying....).

3.  DATA - keeps the 2 screens of data that part 1 gave (See part 1.).

4.  HPLP - high point or low point on the vertical curve to tangent.  Depends

on crest or sag of vertical curve. e.g.

HPLP gives:     Low Point:  129+66.007

Elev:  926.88

Anykey

5.  STFM - Used by other programs for formatting output.  12300 STFM gives:

"123+00.00"   delete (backspace) for next step.

6.  LSTC - Last info that gives a list of the curve data input plus beginning

of vertical curve (B.O.C.) station and elevation and E.O.C. and

rate of curvature. e.g.

LSTC gives:  { 13000 926.21 -1

2.03 200 12900

927.21 13100 928.24

1.515 }

It is a stored list that is handy for revising vc data entry.  Hit

(from above) PRG OBJ OBJ-> DEL (6times).  Now you have the original

vc data input on the stack.

******************************************************************************

Four vertical curves are done below.  Start from the lowest or highest station

number and progress sequentially.

When extracted the dir VCM contains 14 programs/variables:

(I will just go thru some examples)

1.  STMCRV (like 1. above) - Requires 5 inputs (min.) on the stack.

Since we have 4 vc's we need 20 inputs on the stack. e.g.

Station -  103+00   use 10300     (At point of intersection (PI))

Elevation - 944.86 feet   944.86    (At PI)

Grade 1 - -0.19%            -.19   (Grade left of PI)

Grade 2 - 1.65%             1.65    (Grade right of PI)

VC Len. - 120 feet        120       (Vertical Curve Lenth)

or I put it all on one line:

1:

10300,944.86,-.19,1.65,120,10430,947,1.65,0,140,10700,947,0,-1.07,

100,10850,945.4,-1.07,1,140 STMCRV

Note:    The data is stored in the MC variable and press MC PRG OBJ OBJ->

and backspace key to put all the data back on the stack.

2.  SLMC  (like SLV 2. above) - Requires the station that you want to find the

F.G> elevation for.  e.g.

1:

10350 SLMC      (station 103+50)

The display is:

P.I. Sta.:  103+00.00      (This is the curve data it uses to solve for

P.I. Elv :  944.86          F.G. elevation)

Grade 1:  -.19%

Grade 2:  1.65%

Length:  120

R. of C.:  1.5333 ft/ft    (Rate of curvature)

Anykey except ON

Low Point:  102+52.391

Elev:  944.962

Anykey except ON

POINT ON CURVE             (This is important to know)

Sta:  130+50                 (The station just in case we forgot)

Elev:  945.693      **TA DA** (The item we wanted F.G. elevation)

Any key (except ON).

***What happens if we are outside of the curve?  our vc data may not be ***

***applicable or it may.  Let's see what happens.                       ***

for 128+99 (12899 SLMC) you get:

P.I. Sta.:  108+50      (This is the curve data it uses to solve for

P.I. Elv :  945.4        F.G. elevation, the last vc.)

Grade 1:  -1.07%

Grade 2:  1%

Length:  140

R. of C.:  1.4786 ft/ft    (Rate of curvature)

Anykey except ON

Low Point:  108+52.367

Elev:  945.762

Anykey except ON

POINT ON TANGENT 2         (This is improtant to know.  The point is

to the right of the last curve.)

Sta:  128+99                 (The station just in case we forgot)

Elev:  965.89       **TA DA** (The item we wanted F.G. elevation)

Any key (except ON).

Note:  POINT ON TANGENT 2 is not in/on the vertical curve.  It is on the

tangent line as if it continued which it may or may not.

I did this because of error in FG (finish grade elevation) for real work.

You can give the g1 and g2 as the same and you get fg on a continuous

grade and give it a large VC length and this is on the slope.  It is really

handy for some things (i.e. analytical geometry, surveying....).

3.  DATA - keeps the 2 screens of data that part 1 gave (See part 1.).  Press

it, what have you got to lose?  Noda  (Just being funny).

3a. MC - See note part 1. above.

4.  HPLP - high point or low point on the vertical curve to tangent.  Depends

on crest or sag of vertical curve. e.g.

HPLP gives:     Low Point:  108+52.367

Elev:  945.762

Anykey

5.  STFM - Used by other programs for formating output.  12300 STFM gives:

"123+00.00"   delete (backspace) for next step.

6.  LSTC - Last info that gives a list of the curve data input plus begining

of vertical curve (B.O.C.) station and elevation and E.O.C. and

rate of curvature. e.g.

7.  A - last station entered to solve F.G. elevation for.

BIO - Kenneth Hobson wrote this document for you.  These programs were done by

Dewayne Cushman and Kenneth Hobson.  Dewayne was a CE student at OU. Ken

graduated in Spr.'93 and is employed by the Oklahoma Department of

Transportation as in engineering Intern I.

These programs are useful for survey and bridge work.  They would have

been nice to have had when I took SURVEY.

Modify anything you like, as Dewayne and I wrote these for ourselves and you can

use them if you like.

Regards,  Kenneth Hobson  internet at   umtr121@dsibm.okladot.state.ok.us

kenneth.hobson@torii.starship.com

kenneth.hobson@s-box.misc.uoknor.edu