Doubly Reinforced Concrete Beam Analysis

Program Description:

DRCBA is used for a doubly reinforced concrete beam (a beam with compression steel).  Like TBMA, if the compression steel should be neglected, the user is notified and RCBA is called.

Program:  DRCBA

%%HP: T(3)A(D)F(.);

\<< 2 SF CBM 2 CF

87000 fy - INV

87000 * fc * dp *

fy / d / .85 * \Gb1 *

As Asp - b / d /

IF >

THEN

"Neglect comp. steel"

CLLCD 1 DISP 1

FREEZE 4 SF RCBA

ELSE 87000 fy +

87000 / dp * d /

NEG 1 + 87000 *

'fs' STO

IF 'fs>fy'

THEN fy 'fs'

STO

END Asp b / d /

'\Grp' STO 87000 DUP

fy + / .85 * \Gb1 *

fc * fy / \Grp fs *

fy / + '\Grb' STO As

b / d / '\Gr' STO

IF '\Gr>.75*\Grb'

THEN

"Error: \Gr max!" 1

CF DOERR

END As Asp - fy

* .85 / fc / b /

'a' STO As Asp - fy

* a 2 / NEG d + * d

dp - Asp * fy * +

.9 * 12000 / 'Mu'

STO Mu "Mu(k-ft)"

\->TAG 1 CF

END

\>>

# 61E3h

786 bytes