Neutral    Axis

a

b

d

0.85*f

c

’*b

A

s

*f

y

b

d

h

f

b

w

A

s

Concrete Beam Analysis

Program Description:

These are the programs that actually calculate the moment capacity of a beam section.  See DRCBA for a beam with compression reinforcement, CBM and BETA for accessories to these programs, and VCAP for shear capacity of a section

RCBA is used for a rectangular concrete beam with tension reinforcement only.  If Flag 4 is set, it can also be called by other programs.

TBMA analyzes a t-beam but if the t-beam behaves as a rectangular beam, the user is notified and RCBA analyzes the section.

All three programs check for an over-reinforced beam.  RCBA checks for minimum reinforcement using the 200/fy criteria instead of moment of inertia and cracking moment criteria.

Program:  RCBA

%%HP: T(3)A(D)F(.);

\<<

IF 4 FS?

THEN

ELSE CBM

END 4 CF fy 87000

+ INV 87000 * .85 *

\Gb1 * fc * fy / '\Grb'

STO As b / d / '\Gr'

STO

IF '\Gr>.75*\Grb'

THEN

"Error:  \Gr max!"

DOERR

END

IF '200/fy>\Gr'

THEN

"Check cracking moment!"

CLLCD 1 DISP 1

FREEZE

END As fy * .85 /

fc / b / 'a' STO a

2 / NEG d + fy * As

* .9 * 12000 / 'Mu'

STO Mu "Mu(k-ft)"

\->TAG 1 CF

\>>

# 3D10h

466 bytes

Program:  TBMA

%%HP: T(3)A(D)F(.);

\<< 3 SF CBM 3 CF b

bw - hf * fc * .85

* fy / 'Asf' STO As

Asf - fy * .85 / fc

/ bw / 'a' STO

IF 'a<hf'

THEN

"Does not behave as

T-beam. RCBA was used."

CLLCD 1 DISP 1

FREEZE 4 SF RCBA

ELSE Asf bw / d /

'\Grf' STO 87000 DUP

fy + / .85 * \Gb1 *

fc * fy / \Grf + bw *

b / '\Grb' STO As b /

d / '\Gr' STO

IF '\Gr>.75*\Grb'

THEN

"Error: \Gr max!"

DOERR

END As Asf - fy

* d a 2 / - * hf .5

* NEG d + Asf * fy

* + .9 * 12000 /

'Mu' STO Mu

"Mu(k-ft)" \->TAG

END

\>>

# 8B8Ah

646.5 bytes