x

y

OLD

(X1,Y1)

OLD (X2,Y2)

NEW (X1,Y1)

NEW (X2,Y2)

Coordinate Transformation

Program Description:

Given coordinates of two points in two different systems of Cartesian coordinates, the program can convert the coordinates of any point in the first system to it coordinates in the second system.  Currently, the scale in both systems must be the same.  The program will check to make sure scales are consistent (the distance between points 1 and 2 in both systems must not vary by more than 0.001) and notify the user if this is not the case.

Run COORD and input data when prompted then using the CONT button (or key command) to resume the program. Make sure you maintain the contents of the stack during these halts.  CONVT takes x and y values on the stack and converts them to the new system. RP and PR are used for rectangular to polar system conversions.  Enter x,y then RP to get q,r.  PR reverses it.

Example:

Old system points:  	(7,17) 	(12,15)

Points in new system:  	(18,10)	 (16,5)

Old point (6,6) will be new point (7,11)

Program:  COORD

%%HP: T(3)A(D)F(.);

\<< { { "CONT"

\<< CONT

\>> } } TMENU

"Old X1,Y1?" PROMPT

'Y1' STO 'X1' STO

"Old X2,Y2?" PROMPT

Y1 - SWAP X1 - SWAP

R\->P SWAP NEG '\Ga'

STO "New X1,Y1?"

PROMPT 'Y3' STO

'X3' STO

"New X2,Y2?" PROMPT

Y3 - SWAP X3 - SWAP

R\->P SWAP '\Ga' STO+ -

ABS .001 > 0 MENU

IF

THEN

"Scale is not the same"

DOERR

END

\>>

# 3F60h

326 bytes

Program:  PR

%%HP: T(3)A(D)F(.);

\<< \-> \Gh r

\<< \Gh COS r * \Gh SIN

r *

\>>

\>>

# B410h

62 bytes

Program:  CONVT

%%HP: T(3)A(D)F(.);

\<< Y1 - SWAP X1 -

SWAP R\->P SWAP \Ga +

SWAP P\->R Y3 + SWAP

X3 + SWAP

\>>

# 74B6h

85.5 bytes

Program:  RP

%%HP: T(3)A(D)F(.);

\<< \-> XO YO

\<< XO SQ YO SQ + \v/

\-> L

\<< XO L / ACOS

YO L / ASIN SIGN

DUP 0 ==

IF

THEN 1 +

END * L

\>>

\>>

\>>

# D855h

133.5 bytes