EC2.lib (1.0 rel)

Lib number:	 1500

Size 		7.735 bytes

Checksum 	#6443d

Language: 	system RPL

Author: 		Riccardo Perlo ( ricperlo@tin.it )

In the library you will find enclosed a set of check and design programs according to EC2 and CEB “bulletin d’information”.

Noncommercial distribution allowed, provided that this copyright message is preserved,

and any modified version are clearly marked as such.

THIS SOFTWARE IS PROVIDED "AS IS" AND WITHOUT ANY EXPRESSED

OR IMPLIED WARRANTIES, INCLUDING, BUT NOT LIMITED TO, THE

IMPLIED	WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A

PARTICULAR PURPOSE.

The author shall not be liable for any incidental or consequential damages arising from

its use.

If you like and use, please send-me an e-mail.

Suggestion will be appreciated very much.

# VULS.gx (design routine)

RECTANGULAR SECTION subjected to shear force

You’ll find an “INFORM” input screen ad you’re required to fill in with

Bw (base),

D (total height – bottom recovering),

Asl (reinforcement area for bending moment),

fyd (design value for steel yield),

fck (characteristic value for concrete),

Gc (c safety factor for concrete resistance),

Msd (design value for bending moment),

Vsd (design value for shear force)

Alfa (inclination of the stirrups from orizontal, suggested value 90°)

Teta (inclination of concrete compressed trusses according with Ritter-Morsh theory)

Asw/s (total sectioned stirrups area/distance between stitrrups)

If Vrd1 > Vsd no shear area is required (more than minimum area that you must provide anyway) and Asw/s =0

If Vrd1 < Vsd the solving routine will check or solve the required area.

If Asw/s is left =0 the solving routine will give you the Asw/s value for Vrd3=Vsd and it will show Vrd3=0

If Asw/s is filled the solving routive will check if the area given is enought for Vrd3>Vsd

The “inform” input screen will loop until you press “cancel”. In this way you’re allowed to try different values without loosing and retyping other data. If some stack operations are required you can use “CALC” (in the second menu screen).

# MSLS.gx (check routine)

RECTANGULAR SECTION subjected to bending moment and axial force (but mainly bended)

The program gives you the stress in the materials.

You’ll find an “INFORM” input screen ad you’re required to fill in with

H (total height),

B (base),

C (top and bottom recovering),

M (design bending moment),

N (design axial force -compression),

Ac (top reinforcement area)

At (bottom reinforcement area)

The routine will show with MSGBOX screen the values of

x (distance from top of the section and neutral axis)

c (stress in concrete at top of the section)

ft (stress in bottom reinforcement)

fc (stress in top reinforcement)

The “inform” input screen will loop until you press “cancel”. In this way you’re allowed to try different values without loosing and retyping other data. If some stack operations are required you can use “CALC” (in the second menu screen).

# MULS.gx (design routine)

RECTANGULAR SECTION subjected to bending moment and axial force (but mainly bended)

You’ll find an “INFORM” input screen ad you’re required to fill in with

B (base),

D (total height – bottom recovering),

C (top and bottom recovering),

fyd (design value for steel yield),

fcd (design value for concrete),

Msd (design bending moment),

Nsd (design axial force -compression),

The routine will show the values of

mu (reduced value of bending moment (Msd/bd^2fcd))

Area2 (top reinforcement area if required)

Area1 (bottom reinforcement area)

The “inform” input screen will loop until you press “cancel”. In this way you’re allowed to try different values without loosing and retyping other data. If some stack operations are required you can use “CALC” (in the second menu screen).

# NULS.gx (design routine)

RECTANGULAR SECTION subjected to bending moment and axial force (but mainly compressed)

You’ll find an “INFORM” input screen ad you’re required to fill in with

B (base),

H (total height),

C (top and bottom recovering),

fyd (design value for steel yield),

fcd (design value for concrete),

Msd (design bending moment),

Nsd (design axial force -compression),

The routine will show the values of

mu (reduced value of bending moment (Msd/bd^2fcd))

vu (reduced value of axial force (Nsd/bdfcd))

Area2 (top reinforcement area)

Area1 (bottom reinforcement area)

The “inform” input screen will loop until you press “cancel”. In this way you’re allowed to try different values without loosing and retyping other data. If some stack operations are required you can use “CALC” (in the second menu screen).

# FD3.gx

Design values for resistence of materials

You’ll find an “INFORM” input screen ad you’re required to fill in with

fyk (design value for steel yield),

fcf (design value for concrete),

Gac (c safety factor for concrete resistance standard value 1.6),

Gas (s safety factor for steel resistance standard value 1.15),

The routine will show the values of

fct0.05

fcd

Ecm

fyd

The “inform” input screen will loop until you press “cancel”. In this way you’re allowed to try different values without loosing and retyping other data. If some stack operations are required you can use “CALC” (in the second menu screen).

# BARS.gx

Calculates the values of total area and total weight of a given number of reinforcement bars

You’ll find an “INFORM” input screen ad you’re required to fill in with

diameter

number of bars

The routine will show the values of

Total area

Total weight

The “inform” input screen will loop until you press “cancel”. In this way you’re allowed to try different values without loosing and retyping other data. If some stack operations are required you can use “CALC” (in the second menu screen).