HP 48 - Uniform Flow Solution for a Circular Channel Using Chezy’s Equation.

This example shows a solution using the International System of units (SI).

Problem: A discharge of 0.25 m3/s of water flows in a 3-m diameter circular channel of wall roughness, e = 0.0002 m.   Find the depth of flow in this channel if So = 0.0001.  Use a viscosity of 0.000001 m2/s.

______________________________________________________________________

HP variable    Variable		Description                                                              .

D		D		Diameter

				Center angle of water surface

Y		Y		flow depth (m)

C		C		Chezy’s coefficient

S		So		bed slope

Q		Q	 	discharge (m3/s)

A		A		area (m2)

P		P		wetted perimeter (m)

R		R		hydraulic radius (m)

V		V		velocity (m/s)

REY		Re		Reynolds’ number

VISC				kinematic viscosity

E		e		roughness size (m)                                                      .

Use the SOLVR feature in the HP48 calculator to solve for this problem.  Set your angle units to

radians and enter the following in your calculator:

{‘D^5*( - SIN()*COS())^3/(64*) =C^2*S0/Q^2’	< ---- list of equations

‘ Y = D/2*(1-COS())’ ‘A= D^2/4*( - SIN()*COS())’

‘P= *D’  ‘R = A/P’   ‘V = Q/A’    ‘REY =  4*R*V/VISC’

‘C = -   (32*G)*LOG(E/(R*12)+.844*C/(REY*    G))’ }

[ENTER]

‘CRCH’ [STO]                        				< ---- Circular Chezy

[VAR] [CRCH]						< ---- recalls contents of TRCH

‘ [] [E] [] [Q] [STO]					< ---- stores TRCH in EQ variable

[] [SOLVE] [ROOT] [SOLVR]				< ---- starts SOLVR feature

---next, we enter data for problem 34.1 in the textbook, assuming C = 100 for a first guess -----

3 [D]  0.25 [Q]  100 [C]   0.0001 [S]

[] []                                                   	:.724		< ---- solves for angle 

[NXT] [NXEQ][] [Y]			Y:.377		< ---- solves for depth, Y

[NXEQ][] [A]				A: .5141..	<---- solves for the area, A

[NXEQ][ ] [P]  			P:2.1745	< ---- solves for wetted perimeter, P

[NXEQ][ ][R] 				R: .2364...	< ---- solves for hydraulic radius, R

[NXEQ][ ][V]    			V: .4862..	< ---- solves for velocity, V

[NXEQ] 1 [EEX] 6 [+/-] [VISC]				< ---- enter kinematic viscosity, 

[][REY] 			           REY: 459868.55	< ---- solves for Reynold’s number

[NXEQ]9.81[G]0.0002 [E]				< ---- enter g, and roughness size, e

[][C] 					C:70.03		< ---- calculate new value of C

----second iteration-------

[NEXT][NEXQ] [] []              		:.799		< ---- solves for angle 

[NEXT][NEXQ][ ][Y] 			Y:.454		< ---- new value of y

[NXT][NXEQ][ ] [A]			A:.6735		<-- solves for the area, A

[NXEQ][ ] [P] 				P:2.397		<---- solves for wetted perimeter, P

[NXEQ][ ][R]				R:0.2809	<---- solves for hydraulic radius, R

[NXEQ][ ][V]   			V:.3711		<---- solves for velocity, V

[NXEQ][ ][REY]			REY:417109.47	<---solves for Reynold’s number

[NXEQ][ ][C] 				C:70.50		<--- calculate new value of C

--- next iteration --[continue until C converges to a solution, in this case C = 70.49, approximately ] -----