HP 48 - Uniform Flow Solution for a Trapezoidal Channel Using Chezy’s Equation.

This example shows a solution using the International System of units (SI).

Problem: A discharge of 2.83 m3/s of water flows in a 3.05 m wide rectangular channel of roughly made concrete walls.  Find the depth of flow in this channel if So = 0.0004.  T = 20oC.

______________________________________________________________________

HP variable    Variable		Description                                                              .

B		b		bottom width (m) (B = 0 for triangular channel)

Y		Y		flow depth (m)

M		m	 	side slope (M = 0 for rectangular channel)

C		C		Chezy’s coefficient

S		S		bed slope

Q		Q	 	discharge (m3/s)

A		A		area (m2)

P		P		wetted perimeter (m)

R		R		hydraulic radius (m)

V		V		velocity (m/s)

REY		Re		Reynolds’ number

VISC				kinematic viscosity

E		e		roughness size (m)                                                      .

Use the SOLVR feature in the HP48 calculator to solve for this problem.  Enter the following in your calculator:

{‘(B+2*Y*   (1+M^2))/((B+M*Y)*Y)^3=C^2*S/Q^2’	< ---- list of equations

‘A= (B+M*Y)*Y’    ‘P= B + 2*Y*    (1+M^2)’

‘R = A/P’   ‘V = Q/A’    ‘REY =  4*R*V/VISC’

‘C = -   (32*G)*LOG(E/(R*12)+.844*C/(REY*    G))’ }

[ENTER]

‘TRCH’  [STO]                       				< ---- Trapezoidal Chezy

[VAR] [TRCH]						< ---- recalls contents of TRCH

‘ [] [E] [] [Q] [STO]					< ---- stores TRCH in EQ variable

[] [SOLVE] [ROOT] [SOLVR]				< ---- starts SOLVR feature

---next, we enter data for problem 34.1 in the textbook, assuming C = 100 for a first guess -----

3.05 [B]  0.0 [M]  100 [C]   0.0004 [S]   2.83 [Q]

[NXT]                                                                                      < ---- to check if more variables need

[NXT]						                       to be entered.No more variables exist.

[] [Y]                                                   	Y:.677		< ---- solves for depth, Y

[NXT][NXEQ][] [A]			A: 2.066		<---- solves for the area, A

[NXEQ][ ] [P]  			P:4.404..	< ---- solves for wetted perimeter, P

[NXEQ][ ][R] 				R: 0.4690	< ---- solves for hydraulic radius, R

[NXEQ][ ][V]    			V: 1.369...	< ---- solves for velocity, V

[NXEQ] 1.003 [EEX] 6 [+/-] [VISC]			< ---- enter kinematic viscosity, 

[][REY] 			           REY: 2562234.32	< ---- solves for Reynold’s number

[NXEQ]9.81[G]0.043 [E]					< ---- enter g, and roughness size, e

[][C] 					C:36.65		< ---- calculate new value of C

----second iteration-------

[NEXQ][ ][Y] 				Y:1.464		< ---- new value of y

[NXT][NXEQ][ ] [A]			A:4.466		<-- solves for the area, A

[NXEQ][ ] [P] 				P:5.9785	<---- solves for wetted perimeter, P

[NXEQ][ ][R]				R:0.7470	<---- solves for hydraulic radius, R

[NXEQ][ ][V]   			V:0.633		<---- solves for velocity, V

[NXEQ][ ][REY]			REY:1887758.29	<---solves for Reynold’s number

[NXEQ][ ][C] 				C:40.23		<--- calculate new value of C

--- next iteration --[continue until C converges to a solution, in this case C = 39.96, approximately ] -----