SECTIONS

Program to Calculate the Geometric Properties of Sections through Coordinates

SECTIONS is a program created for the HP 48GX that can be used in the HP 49 series calculators, including the HP 50G.

I wrote the program when I was a student in EMI, La Paz, Bolivia, reason why the program is in Spanish.

The best way to understand a program is through an example; the following is a walk through of such an example:

- Once you download SECTIONS o your calculator and open the folder in your device you will have the following soft-key menu.

The DAToS file contains instructions on how to use the program, the translation of these instructions are as follows:

A. First Message Box; “Save the coordinate points in the ‘CooRD’ matrix as x and y columns (x,y)”

B. Second Message Box; “Input the coordinates clockwise for solid sections and counter-clockwise for hollow sections (holes)”.

C. Third Message Box; “The last point of the coordinates shall repeat the first point to close the section”, (xn,yn) = (x1,y1).

D. Fourth Message Box; “Renumber overlapping points, e.g. Point 2 & 14.

EJEM, ELE1, ELE2, ELE3, are examples of section coordinates.

- Make sure the calculator is set for RPN and Degrees. Open the ‘CooRD’ matrix to view the section coordinates we will work with, we should have a total of 27 points that will be used to calculate the properties of the section.

- Close ‘CooRD’ without changing the matrix, and press the soft-key at the right end of the list that says ‘SEC’ with a small downward triangle next to it, this will initiate the program.

- The title “ Geometric Sections / Coordinates -> CooRD” will appear and the program will request the number of points, insert 27 and [ENTER].

- The title “Graphic – Domain” will appear and the program will request the maximum and minimum graphic range of the section for plotting. It is recommended to provide a coordinate range slightly larger (or smaller for negative coordinates) than the limit coordinates of the section. In our case insert 24 [SPC] -24 [Down ARROW] 12 [SPC] -12 [ENTER]. The calculator will now initiate the calculations “Calculating …”

- Once the calculations are complete the calculator will show you the primary “Geometric Properties” of the section: the cross sectional area, centroidal x & y coordinates and centroidal inertias. Press [ENTER] once more to see the cross section graphically.

- Once the graphic appears, press the following soft-keys to see the graph in full [EDIT], [NEXT] (botton) and [MENU]. Press the [ON] button to continue. The function of the graph is to verify that your input coordinates is correct. I this case the section has three holes one next to each other in the canter, note how some points are repeated to represent the holes in the section.

- A Prompt requesting the “Number of angles of rotation for x,y” appears, this is asking you how many different coordinate rotation alternatives would you like to evaluate for the centroidal inertia analysis. A minimum of one shall be entered, in our case we will ask for one alternative: press 1 and [ENTER].

- Now you are prompted to insert the angle, press 22.4 and [ENTER], the angle is in degrees.

- A message box shall appear providing the centroidal Ix, Iy and Ixy for the rotated coordinate system, press [ENTER].

- The calculator stack should appear populated with all the geometric properties of the section. Please scroll up the stack to see all the output.

- The following is the nomenclature used:

A = Cross sectional area

x = Centroid in x

y = Centroid in y

Ix = Moment of inertia in x (non-centroidal)

Iy = Moment of inertia in y (non-centroidal)

Ixy = Moment of inertia in xy (non-centroidal)

Ixc = Centroidal moment of inertia in x

Iyc = Centroidal moment of inertia in y

Ixyc = Centroidal moment of inertia in xy

Theta prin = Angle in degrees of the principal inertia coordinate system

Ix.prin = Centroidal principal moment of inertia in x

Iy.prin = Centroidal principal moment of inertia in y

Ixy.prin = Centroidal principal moment of inertia in xy

rx = Radius of gyration in x

ry = Radius of gyration in y

rpx = Principal radius of gyration in x

rpy = Principal radius of gyration in y

Ipol-c = Centroidal polar inertia or tensional moment of inertia

The source code is open in case you would like to employ the algorithms in your own program or translate the labels.

I hope you find the program useful.

Best regards,

Bryan R. Osborne, PE

Black50g@gmail.com